3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216438

Electronic Meeting, 10 – 19 Oct, 2022

Title:	R18 Discussion on FR2 beam correspondence requirements in initial access

Source:	OPPO

Agenda item:	6.7.3.1

Document for:	Approval

- Introduction

In Rel-18, beam correspondence at initial access stage and RRC inactive is one of the objectives for FR2 enhancement as below table [1].

Beam correspondence requirements for RRC_INACTIVE and initial access - Specify UE beam correspondence requirements for initial access and RRC_INACTIVE state, for SSB-based beam correspondence without UL beam sweeping - For RRC_INACTIVE specify at least requirements for Random Access SDT and Configured Grant SDT - Requirements for other transmission within RRC_INACTIVE state are not precluded. - For initial access, specify requirements and verification of beam correspondence requirements based on msg1 spherical coverage (at least) - Study the potential impact on testability aspects (i.e., test time).

And in last meeting the WF [2] is approved with many open issues to be further discussed. This paper will discuss these aspects.

- Discussion

  - Beam correspondence requirement

Similar as connected mode, peak EIRP and spherical coverage can be considered as starting point. Meanwhile the difference also needs to be carefully evaluated. One point is the beam differences, as discussed above UE can apply rough or fine beam in the initial access. Then without differentiating these two kinds of implementation, the initial access beam correspondence requirements need to cover both and this means the worse scenario (rough beam) will be used as reference implementation to define requirements. If there is interest to differentiate the rough beam or fine beam implementation in initial access, then it needs to be carefully thinking how this can be achieved since there is no capability reporting at the initial access stage.

Observation 1:    Without differentiating rough beam and fine beam implementation in initial access, the beam correspondence requirements need to cover both and use the worse scenario (rough beam) to define requirements.

Proposal 1:         In order to cover both rough beam and fine beam implementation in initial access, propose to use the worse scenario (rough beam) as reference to define beam correspondence requirements.

Once rough beam is used, then the gain differences should be considered, e.g. the 7dB peak gain difference defined in RRM. One straight forward approach is to reuse the minimum peak EIRP and spherical coverage requirement minus 7dB as the requirement for initial access.

Proposal 2:         By considering the gain difference between rough beam in initial access and fine beam in connected, propose to consider define 7dB lower beam correspondence requirement (peak EIRP and spherical) comparing to connected mode as starting point.

However, one potential issue might be if UE use a good but not best beam to transmit RACH for the reason of speed up initial access procedure then 7dB might not be enough for this UE. This situation happens when DL signal quality measured via this good beam is good enough for UE and there is no need to further find the best beam. Then how to handle this situation might need more study.

Observation 2:    UE may use a good beam instead of best beam to transmit RACH when it’s measured DL signal quality is good enough for RACH. This will cause the UE performance be underestimated if test the peak and spherical requirements.

Proposal 3:         Further consider the situation that UE may use a good but not the best beam to transmit RACH to speed up initial access.

  - RRC inactive

Although different objectives are included in the WID for initial access and INACTIVE state, there is no difference in UE transforming to connected mode, i.e. both are based on preamble transmission in PRACH and RAR from NW. Considering the beam correspondence is to test UE radiating performance and there is no difference in the content carried in physical layer. From this perspective, the beam correspondence requirement should be same.

Observation 3:    There is no difference in Beam correspondence requirement for initial access and RRC Inactive from UE Tx power perspective.

Proposal 4:         Same beam correspondence requirements are applied for initial access and RRC Inactive.

- Conclusion

In this paper, we discussed the beam correspondence requirement for initial access, and got following observations and proposals.

Observation 1:    Without differentiating rough beam and fine beam implementation in initial access, the beam correspondence requirements need to cover both and use the worse scenario (rough beam) to define requirements.

Proposal 1:         In order to cover both rough beam and fine beam implementation in initial access, propose to use the worse scenario (rough beam) as reference to define beam correspondence requirements.

Proposal 2:         By considering the gain difference between rough beam in initial access and fine beam in connected, propose to consider define 7dB lower beam correspondence requirement (peak EIRP and spherical) comparing to connected mode as starting point.

Observation 2:    UE may use a good beam instead of best beam to transmit RACH when it’s measured DL signal quality is good enough for RACH. This will cause the UE performance be underestimated if test the peak and spherical requirements.

Proposal 3:         Further consider the situation that UE may use a good but not the best beam to transmit RACH to speed up initial access.

Observation 3:    There is no difference in Beam correspondence requirement for initial access and RRC Inactive from UE Tx power perspective.

Proposal 4:         Same beam correspondence requirements are applied for initial access and RRC Inactive.

- References

[1] RP-221862, WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3

[2] R4-2214454, WF on BC in RRC_INACTIVE and initial access, Nokia