3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216441

Electronic Meeting, 10 – 19 Oct, 2022

Title:	R18 Discussion on power domain enhancement

Source:	OPPO

Agenda item:	6.23.2

Document for:	Approval

- Introduction

In Rel-18 coverage enhancement WI, the power domain enhancement is one of the study points, as can be seen in below table. This paper will discuss this aspect.

- Study and if necessary specify following power domain enhancements - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations

- Discussion

In Rel-17, increasing UE power high limit for CA and DC has been introduced. And CR [1] was agreed to allow PC2 UE with PC3 in band A (TDD or FDD) and PC2 in band B (TDD) can transmit total power higher than 26dBm as can be seen in figure 1. Meanwhile UE capability higherPowerLimit-r17 is introduced per band combination to indicate whether UE Tx power can be increased or not.

Figure 1 Increasing UE power high limit for CA and DC definition in 38.101-1 spec.

Observation 1:   In Rel-17, increasing UE power high limit feature is only for PC2 total power band combinations with PC3 TDD/FDD + PC2 TDD.

During the discussion it is clear the with the increase of UE Tx power the emissions will also be higher, but it relies on UE implementation to solve this issue while keep emission requirements unchanged. If UE cannot meet the emission requirements, then it should not report the higherPowerLimit-r17 capability. With this in mind then the increased UE Tx power approach can be extended to more cases, like PC3, intra-band UL CA case, UL MIMO, or even 3Tx in the future.

Observation 2:   The Rel-17 increasing UE power high limit feature only increase UE Tx power but keep the emission requirements unchanged. If UE cannot meet the emission requirements with higher Tx power then it shouldn’t indicate the higherPowerLimit-r17 capability.

Proposal 1:         Consider extending higherPowerLimit-r17 feature to more cases:

        - Case 1: PC3 band combinations with PC5 band1 + PC3 band2

        - Case 2: PC2 intra-band UL CA band combinations with PC3 CC1 + PC2 CC2

        - Case 3: PC2 single band UL MIMO with PC3 PA + PC2 PA

        - Case 4: 3Tx scenario in the future

- References

[1] R4-2210767, CR: Increasing the maximum power limit for inter-band UL CA, QC