3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216442

Electronic Meeting, 10 – 19 Oct, 2022

Title:	R16 Discuss and reply LS on ModifiedMPR-Behaviour

Source:	OPPO

Agenda item:	8.3.3

Document for:	Approval

- Introduction

LS [1] from RAN5 asking the modifiedMPR-Behaviour definition in 101-2 spec, and in RAN4#104e reply LS [2] was sent to RAN5 with answers for some of the issues. This paper will discuss the left issues shown in below table and try to give feedback.

Question c) For Rel-16 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 mandatory or optional? In case it is mandatory then is the Rel-16 UE expected to signal modifiedMPR-Behaviour bit 0=true? Question d) For Rel-16 PC3 UE, which version of specification is taken as default MPR requirement, 38.101-2 v16.2.0 or latest version (v16.11.0 released in Apr 2022)? What are the Rel-16 MPR requirements if the UE signals respectively modifiedMPR-Behaviour bit 0=false and modifiedMPR-Behaviour bit 0=true?

- Discussion

2.1 Background of modifiedMPR-Behaviour in FR2

The Rel-15 FR2 MPR definition was reviewed in Rel-16 FR2 enhancement, and it was agreed to improve UE performance by extending the region 1 scope and narrow down the edge region with Rel-16 CR [3] agreed in RAN4#93. Then in RAN4#95, the modifiedMPR-Behaviour table was introduced for Rel-16 and Rel-15 FR2 which refers to 101-2 v16.2.0 MPR [4][5]. And the reason to ask Rel-15 UE to optionally support Rel-16 MPR was to give Rel-15 UE chance to improve its performance. After that, the modifiedMPR-Behaviour table was in the spec and no changes to that. Using modifiedMPR-Behaviour to indicate the MPR that UE complies is common in 38101 series.

Observation 1:    Improved MPR was introduced in Rel-16 38.101-2 v16.2.0 which makes two different MPRs exist in Rel-16 specs, and modifiedMPR-Behaviour IE was used to indicate which MPR UE supports.

2.2 modifiedMPR-Behaviour in different releases

Regarding the questions from RAN5 about the modifiedMPR-Behaviour indication in different releases, below will discuss in each question.

Question c): For Rel-16 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 mandatory or optional? In case it is mandatory then is the Rel-16 UE expected to signal modifiedMPR-Behaviour bit 0=true?

The improved MPR was introduced from 38.101-2 v16.2.0, it makes different MPRs exist in Rel-16, thus modifiedMPR-Behaviour bit 0 was defined to indicate which MPR apply.

UEs comply with 38.101-2 v16.2.0 and onwards shall support the improved MPR and signal modifiedMPR-Behaviour bit 0=true.

For UEs comply with 38.101-2 Rel-15 or Rel-16 v16.0.0 and v16.1.0 can consider this improved MPR as optional enhancement, thus can signal modifiedMPR-Behaviour bit 0=false.

Therefore, the MPR defined in 38.101-2 v16.2.0 is not mandatory for Rel-16 PC3 UE. This is also aligned with current spec definition in Table H.1-1 of 38.101-2.

Observation 2:    Rel-16 UE can comply with either the MPR defined before v16.2.0 or after that.

There was some comment in last meeting about the concept of “open release”, however, the meaning is not quite clear in RAN4. Usually whether a release is open or freeze is mainly about the ASN.1 in RAN2, and after it is freeze then the signalling will not be changed in principle and UEs can implement with that. Any signalling changes after that have to carefully consider whether it will cause NBC issue. However, in RAN4, there is no clear view on how to define the meaning of “open release” and “freeze release” as long as it doesn’t have the signalling impact. So usually people see that RAN4 WIs can close before or after the ASN.1 frozen. And even after ASN.1 frozen and RAN4 WI is closed the maintenance work continues with new requirements introduced or changes made. So the claim that when the requirements are defined in the “open release” UE shall always comply with the latest and after release frozen then the requirements are mandatory needs to clarify what is the meaning of “open release” and “freeze release” in RAN4 concept and get the common understanding first.

Observation 3:    It is unclear the meaning of “open release” and “freeze release” in RAN4, usually only ASN.1 matter and RAN4 WI can close before or after the ASN.1 frozen as long as no signaling impact.

Proposal 1:         For Rel-16 PC3 UE, the MPR defined in 38.101-2 v16.2.0 is optional.

Proposal 2:         Encourage proponents to clarify the meaning of “open release” and “freeze release” in RAN4, and align the understanding of requirement applications for different spec versions.

Question d): For Rel-16 PC3 UE, which version of specification is taken as default MPR requirement, 38.101-2 v16.2.0 or latest version (v16.11.0 released in Apr 2022)? What are the Rel-16 MPR requirements if the UE signals respectively modifiedMPR-Behaviour bit 0=false and modifiedMPR-Behaviour bit 0=true?

The improved MPR was introduced from Rel-16 v16.2.0, and after that modifications/updates could happen. For Rel-16 PC3 UE the latest version of specification can be taken as default MPR requirement, otherwise, the changes made after the v16.2.0 will become meaningless since they will never be referred in this case.

If Rel-16 UE signals modifiedMPR-Behaviour bit 0=false, the original MPR requirements apply and can refer to the latest of version of 38.101-2 with original MPR requirements which is v16.1.0.

If Rel-16 UE signals modifiedMPR-Behaviour bit 0=true, the Rel-16 latest version MPR requirements apply.

Observation 4:     If the modifiedMPR-Behaviour refer to the fixed version v16.2.0, then the changes after that will no longer be used, and there is no way for RAN4 to update the MPR section after v16.2.0. This is not reasonable.

And in RAN4#104e meeting, it was agreed in [6] to remove the spec version information from modifiedMPR-Behaviour tables as shown below, which means the latest version of specification version will be referred. If this is doable for FR1 then it should also be doable to FR2, and both should be aligned in the handling.

Observation 5:     It was agreed in FR1 that latest spec version will be referred in the modifiedMPR-Behavior definition, and remove the fixed version from the table.

Proposal 3:         Align the spec version handling between FR1 and FR2, and in FR1 the specific version has been removed, similar can be done in FR2.

Proposal 4:         For Rel-16 PC3 UE, the latest version of specification is used when modifiedMPR-Behaviour bit 0=true. And the 38.101-2 v16.1.0 is used when UE signals modifiedMPR-Behaviour bit 0=false.

2.3 Update of modifiedMPR-Behaviour to clarify different releases

To resolve the ambiguity in the spec cause by above modifiedMPR-Behaviour among different releases, it is proposed to update the modifiedMPR-Behaviour definition as below (use n257 as example).

- For Rel-15 and Rel-16

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n257 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 from v16.2.0 | - This bit may be set to 1 by a UE supporting n257 |

- For Rel-17

For Rel-17, there is only one MPR requirement applied in the spec from v17.0.0, and there is no point to keep the modifiedMPRbehavirour which refers to a Rel-16 specification even the MPR is same between Rel-16 and Rel-17.

If keep it, then the changes probably could be to remove the specification version as below changes and apply the Rel-17 spec, but what is the additional information this table can provide? Rel-17 UE shall comply with Rel-17 specification. This is the basic principle. Comparingly, remove the table is more clean solution.

And it should be noticed that this is different from the case that two requirements are defined in the same spec with one of them is optional. In that case, it is indeed the modifiedMPR-Behaviour will be applied in the later spec to say that the optional one will become mandatory in this later release. For the FR2 improved MPR discussion here, we only have one MPR defined in the spec since v16.2.0, and in Rel-17 the whole spec only contains one MPR, there is no chance for UE to comply with another MPR. That why in our view there is no point to saying Rel-17 UE shall meet Rel-17 MPR by setting the modifiedMPR-Behaviour IE to 1. This is redundant information.

Table H.1-1: Definitions of the bits in the field modifiedMPRbehavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n257 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 | - This bit shall be set to 1 by a UE supporting n257 |

Observation 6:    In Rel-17, only one MPR defined, there is no point for Rel-17 UE to comply with Rel-16 MPR specification.

Even remove the Rel-16 version information from Rel-17 spec, still there is no more information this modifiedMPR-Behavior can provide since it is basic principle that Rel-17 UE will comply with Rel-17 specification.

Proposal 5:         For Rel-15 and Rel-16, the modifiedMPR-Behaviour pointing to the latest version of specification by using “from v16.2.0”.

For Rel-17, remove the modifiedMPR-Behaviour table which pointing to the improved MPR requirements defined since v16.2.0.

- Conclusion

In this paper we discussed the modifiedMPR-Behavior handling for different releases and also the changes to be made for Rel-15/16 and 17.

Observation 1:   Improved MPR was introduced in Rel-16 38.101-2 v16.2.0 which makes two different MPRs exist in Rel-16 specs, and modifiedMPR-Behaviour IE was used to indicate which MPR UE supports.

Observation 2:   Rel-16 UE can comply with either the MPR defined before v16.2.0 or after that.

Observation 3:   It is unclear the meaning of “open release” and “freeze release” in RAN4, usually only ASN.1 matter and RAN4 WI can close before or after the ASN.1 frozen as long as no signaling impact.

Proposal 1:         For Rel-16 PC3 UE, the MPR defined in 38.101-2 v16.2.0 is optional.

Proposal 2:         Encourage proponents to clarify the meaning of “open release” and “freeze release” in RAN4, and align the understanding of requirement applications for different spec versions.

Observation 4:   If the modifiedMPR-Behaviour refer to the fixed version v16.2.0, then the changes after that will no longer be used, and there is no way for RAN4 to update the MPR section after v16.2.0. This is not reasonable.

Observation 5:   It was agreed in FR1 that latest spec version will be referred in the modifiedMPR-Behavior definition, and remove the fixed version from the table.

Proposal 3:         Align the spec version handling between FR1 and FR2, and in FR1 the specific version has been removed, similar can be done in FR2.

Proposal 4:         For Rel-16 PC3 UE, the latest version of specification is used when modifiedMPR-Behaviour bit 0=true. And the 38.101-2 v16.1.0 is used when UE signals modifiedMPR-Behaviour bit 0=false.

Observation 6:   In Rel-17, only one MPR defined, there is no point for Rel-17 UE to comply with Rel-16 MPR specification.

Even remove the Rel-16 version information from Rel-17 spec, still there is no more information this modifiedMPR-Behavior can provide since it is basic principle that Rel-17 UE will comply with Rel-17 specification.

Proposal 5:         For Rel-15 and Rel-16, the modifiedMPR-Behaviour pointing to the latest version of specification by using “from v16.2.0”.

For Rel-17, remove the modifiedMPR-Behaviour table which pointing to the improved MPR requirements defined since v16.2.0.

- References

[1] R4-2211526, LS on ModifiedMPR-Behaviour clarification for different power classes, R5->R4

[2] R4-2215091, draft Reply LS on ModifiedMPR-Behaviour clarification for different power classes, R4->R5

[3] R4-1915755, CR to 38.101-2-g10 Corrections to maximum output power reduction for power class 3, Skyworks

[4] R4-2009169, Indication of modified MPR behaviour for FR2 MPR enhancement (R16), Nokia

[5] R4-2008891, Indication of modified MPR behaviour for FR2 MPR enhancement (R15), Nokia

[6] R4-2214649, CR for 38.101-1: Correction of PC2 MPR and PC1.5 fallback to PC2 MPR, T-Mobile USA