3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216444

Electronic Meeting, 10 – 19 Oct, 2022

Title:	Considerations on the AoA pairs for the RF requirement of multi-Rx chain

Source:	OPPO

Agenda item:	6.8.2.2

Document for:	Approval

- Introduction

The technical discussion on Work Item of FR2 multi-Rx chain DL reception was started from last RAN4 meeting, i.e. RAN4#104-e. As the spherical coverage is the mainly considered RF requirement for multi-Rx chain DL reception, the AoA pairs are proposed and discussed. The agreement in the WF [1] proposed four candidate options as below, and other solutions are not precluded at current stage.

This contribution presents our views and considerations on the AoA pair for the RF requirement of FR2 multi-Rx chain DL reception.

- Discussion

The four candidate AoA pairs have their reasonable intention on standing for the UE RF requirement. However, an appropriate RF requirement needs to be considered taking several aspects into account. The consideration of the AoA pairs will be explicated, including basic comparison of the candidates, test time effort of the candidates.

- Basic comparison of the candidates

The Candidate 1/2/4 in the above agreement can be treated and discussed as a group.

The Candidate 1 (One Fixed AoA1 (e.g. Peak) + Full set AoA2) is the basic extension from the spherical coverage of one AoA, where one AoA sweeps the whole spherical surface with the other AoA fixed at beam peak direction.

The Candidate 2 (Multiple AoA1 + Full set AoA2) is the extension of the Candidate 1, where not only the Peak beam but also some of other available directions as the fixed/connected AoA are included into the UE RF requirement consideration.

The Candidate 4 (Full set AoA1 + Full set AoA2) is the further extension of the Candidate 1 and 2 that all the test points/directions on the measurement grid will be used as the fixed/connected AoA one by one, when the sweeping AoA measures the spherical coverage.

For Candidate 2 and Candidate 4, the following two cases are considered, i.e. where the multi AoA1s direct to. Supposing that two RF panels are implemented in the UE for different portions of spherical coverage. Case 1 illustrates that two (or multi) AoA1s direct to the same RF panel in the UE, while Case 2 illustrates that two AoA1s direct to different UE RF panels.

Fundamentally, the spherical coverage performance of AoA2 in Case 1 would be similar or even equivalent for AoA1 and AoA1’. For Case 2, the spherical coverage of AoA2 with AoA1 fixed will be quite different from that with AoA1’ fixed.

Observation 1: The spherical coverage performance of AoA2 in Case 1 would be similar or even equivalent for AoA1 and AoA1’.

Observation 2: For Case 2, the spherical coverage of AoA2 with AoA1 fixed will be quite different from that with AoA1’ fixed.

With the above observations, it is proposed that when appointing several AoA1s, relatively large separation between AoA1s should be adopted, e.g. the separation more than 120 degree.

Proposal 1: It is proposed that when appointing several AoA1s for Candidate 2, relatively large separation between AoA1s should be adopted, e.g. the separation more than 120 degree.

On the other hand, the Candidate 3 (Fixed offset between the two AoAs, both probes swept simultaneously) is the totally different solution from Candidate 1/2/4. It is a variant of the spherical coverage of one AoA, where the two combined AoAs (with fixed offset between the two AoAs) can be treated as a whole (an array with two probes). The RF requirement of the spherical coverage of one AoA can be reused on that of two AoAs in maximum extent. The key issue for Candidate 3 is how to decide the fixed separation between the two AoAs.

Observation 3: Based on the Candidate 3, the RF requirement concept of the spherical coverage of one AoA can be reused in maximum extent.

Observation 4: The key issue for Candidate 3 is how to decide the fixed separation between the two AoAs.

- Test time effort of the candidates

The test time effort spent on the UE RF requirement is an important aspect to be considered when specifying the requirement. The test time of the four candidates depends on how many times the spherical coverage need to be tested.

For the Candidate 1, the test time is equal to the measurement of the spherical coverage of one single AoA, i.e. sweeping the whole spherical surface one time.

For the Candidate 2, the test time is multiplied M times compared with the Candidate 1, where M is the number of AoA1s.

For the Candidate 4, the test time is multiplied N times compared with the Candidate 1, where N is the number of spherical test points of AoA1 and N>M.

For the Candidate 3, the test time depends on the specified approach of spherical surface sweeping. In detail, when we have the information of the direction of AoA1 and the fixed offset between the two AoAs, we actually may have multiple AoA2 directions on the spherical surface, as illustrated in the below figure. The number of AoA2 required to measure for every AoA1 decides the total test time of the Candidate 3.

The test time effort of the four candidate AoA pairs are summarized in the following table.

| Candidate Options | Test time effort |
| --- | --- |
| Candidate 1 | The same to 1 AoA spherical coverage test time, T |
| Candidate 2 | M*T, M is the number of AoA1s |
| Candidate 3 | P*T, P is the number of AoA2 required to measure for every AoA1 and P≥1. |
| Candidate 4 | N*T, T is the number of spherical test points of AoA1 and N>M. |

Considering that 4-layer DL MIMO may not always happen with one of AoA on the direction of UE beam peak, Candidate 1 is not the popular scenario for 4-layer DL reception. Therefore, combined with the test time effort information, it is proposed to down-select to Candidate 2 and Candidate 3 for further discussion.

Proposal 2: It is proposed to down-select to Candidate 2 and Candidate 3 for further discussion.

- Conclusions

This paper discussed about the candidate AoA pairs for the RF requirement of FR2 multi-Rx chain DL reception. And got the following conclusions.

Proposal 1: It is proposed that when appointing multi AoA1s for Candidate 2, relatively large separation between AoA1s should be adopted, e.g. the separation more than 120 degree.

Proposal 2: It is proposed to down-select to Candidate 2 and Candidate 3 for further discussion.

- References

[1] R4-2214457, WF on FR2 multi-Rx chain DL reception, Sony