3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216449

Electronic Meeting, 10 – 19 Oct, 2022

Title:	FR1 MIMO OTA in browsing mode

Source:	OPPO

Agenda item:	6.15.3

Document for:	Approval

- Introduction

The new work item on NR MIMO OTA enhancement was approved in the last RAN Plenary meeting, i.e. RAN #97-e. FR1 MIMO OTA test methodology enhancement was one objective of Core part WI, where to study the test methodology for smartphone in browsing mode using hand phantom was the first time set as the objectives for FR1 MIMO OTA [1].

This contribution presents our views and considerations on the enhancement of the FR1 MIMO OTA test methodology in browsing mode using hand phantom.

- Discussion

Nowadays, phantoms, including head phantom, hand phantoms and various customized phantoms, are widely used for TRP and TRS measurement, to evaluate the DUT antenna performance under common usage scenarios. The measurement results of TRP and TRS on many commercial devices show that the UE antenna pattern and total efficiency will substantially change if the usage scenarios are different. Therefore, the multi antennas in the UE will have different antenna correlation under different usage scenarios, and further produce different MIMO performance behavior. From this perspective, MIMO OTA test in browsing mode using hand phantom is not only a helpful improvement for free space testing, but also a necessary method for UE to evaluate and optimize the MIMO performance under the most commonly usage scenario, i.e. browsing mode. It is consequently proposed that it is necessary to specify enhanced FR1 MIMO OTA test methodology in browsing mode using hand phantom.

Proposal 1: It is necessary to specify enhanced FR1 MIMO OTA test methodology in browsing mode using hand phantom.

According to FR1 MIMO OTA performance requirement specified in TR 38.827 [2], the TRMS is the averaging value of three data mode in free space, i.e. Free space data mode screen up (FS DMSU), Free space data mode portrait (FS DMP) and Free space data mode landscape (FS DML). The positioning of three modes defined in Annex A of TR38.827 are shown as below. As pointed out, the three modes of FS DMSU, FS DMP and FS DML are the most representative free-space usage scenarios for the UE.

For the usage scenario of browsing mode, the representative cases should be reconsidered. Firstly, the hand phantoms currently described in 3GPP specifications, whether PDA grip hand or Wide grip hand, are designed for gripping the UE vertically with portrait manner. Secondly, when holding the UE in hand for browsing, the screen of the UE is normally facing toward the eyes of the user and naturally “screen-up” is not the common case for daily usage. Therefore, it is proposed to focus on portrait mode with hand phantom for FR1 MIMO OTA browsing usage scenario, shown as below.

Proposal 2: it is proposed to focus on portrait mode with hand phantom for FR1 MIMO OTA browsing usage scenario.

- Conclusions

This paper discussed about the enhancement of the FR1 MIMO OTA test methodology in browsing mode using hand phantom.. And got the following conclusions.

Proposal 1: It is necessary to specify enhanced FR1 MIMO OTA test methodology in browsing mode using hand phantom.

Proposal 2: it is proposed to focus on portrait mode with hand phantom for FR1 MIMO OTA browsing usage scenario.

- References

[1] R4-222668, R18 New WID: Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) test methodology and requirements for NR UEs, CAICT, OPPO, Keysight Technologies

[2] 3GPP TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)