3GPP TSG-RAN WG4 Meeting #104-e	R4-2216451

Electronic Meeting, 15 – 26 Aug, 2022

Title:	TP for TR38.892

Source:	OPPO

Agenda item:	5.27.2

Document for:	Approval

Introduction

In previous RAN 4 #104e meeting, a new TR for APT NR 600 MHz with its skeleton was approved. In accordance with the workplan, this contribution provides the text proposals to TR 38.892.

Discussion

Referring to RP-212540, the reply LS from AWG, the band could be called as “APT 600 MHz band”, respectively. This band name should be consistent within TR 38.892.

- Text proposal to TR 38.892: section 5

# 5	Frequency band arrangement

## 5.1	Operating band, channel bandwidth and channel arrangement

The APT 600 MHz band was arranged to operating in the bands illustrated in Table 5.1-1 with the assigned operating band number n105. The reverse-duplex was utilized for the benefit of aligning the uplink band adjacent to 3GPP band 28 thereby minimizing interference conditions at the 703 MHz boundary, which has taken the consideration of APT 700 MHz band plan

Table 5.1-1: NR operating band for APT 600MHz band

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

The channel bandwidth with its SCS parameters was specified in Table 5.1-2 below.

Table 5.1-2: Channel bandwidth and SCS parameters

| SCS (kHz) | channel bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 |
| 30 |  | 10 | 15 | 20 | 25 | 30 | 35 |

In view of compatibility with B71/n71, the 100 kHz channel raster was reused in Table 5.1-3.

Table 5.1-3: Applicable NR-ARFCN for APT600 band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| N105 | 100 | 132600 – <20> – 140600 | 122400 – <20> – 130400 |

The applicable SS raster for APT 600 MHz band is defined in Table 5.1-4.

Table 5.1-4: Applicable SS raster entries for APT600 band

| NR operating band | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n105 | 15 kHz | Case A | 1535 – <1> – 1624 |

## 5.2	Duplex spacing

The Rx-Tx separation is specified as -51 MHz in Table 5.2-1.

Table 5.2-1: Applicable SS raster entries for APT600 band

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| N105 | -51 MHz |

Given the overlapping frequency bands offers an occasion to reuse existing devices operating in B71/n71 band on transition basis, it enables a more flexible duplex spacing for APT 600 MHz band in different use case. Therefore, the variable frequency separation could be utilized.

- Text proposal to TR 38.892: section 6

# 6 Compatibility with B71/n71

APT 600 MHz band compatible over 30 MHz that overlaps with n71 as shown in Figure 6-1.  The devices supporting both APT 600 MHz band and B71/n71 band is desirable.

Figure 6-1  APT 600 MHz and B71/n71

A UE implementation that supports variable Rx-Tx separation for n71 and the network utilising blocks 2-8 (or a subset) could support the same frequency range as band n71. UL block 8 and DL block 1 are outside of n71 filter range and cannot be used by legacy device.

The difficulty is that Band n71 utilizes a different Tx-Rx separation than Option B1, even if the frequency ranges overlap.  This can be partially addressed as described below.

Observations for 40 MHz B2F full duplexer is listed.

- 	UL 2-7 can be paired with DL 2-7 if the UE implementation supports variable Rx-Tx separation

-	Operators that own blocks 1 and 8 cannot take advantage of this

-	Band n71 UE may not have been tested with other than default n71 Rx-Tx separation, but reading the SIB is mandatory for UE’s

-	With this approach, it is possible to enable a UE implementation for n71 (over 30 MHz limited frequency range) and new option B1 UE’s

- Text proposal to TR 38.892: section 7

# 7	UE and BS requirements for APT 600 MHz

## 7.1	UE requirements

### 7.1.1 	UE transmitter characteristics

#### 7.1.1.1	UE TX maximum output power

The maximum output power of the UE operating in APT 600 MHz band is defined in Table 7.1.1.1-1

Table 7.1.1.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n105 |  |  |  |  |  |  | 23 | +2/-2.5 |

#### 7.1.1.2 	NS value and Spectrum Emission Mask

### 7.1.2	UE receiver characteristics

#### 7.1.2.1	UE REFSENS

#### 7.1.2.2	In-band blocking

#### 7.1.2.3 	Out-of-band blocking