3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216455

Electronic Meeting, Oct 10- Oct 19, 2022

Agenda Item:	4.6.3.1.2

Source: 	Ericsson

Title:	Discussions on RedCap mobility requirements

Document for:	Discussion

# Introduction

RAN4 has discussed the impact on RRM Handover requirements due to NCD-SSB in last meeting and the remaining issues are as follow [1].

Issue 2-1-2: Requirements for HO to a BWP which has different SSB with the one used for measurement Company positions after 1st round: - Option 1 (QC, vivo, MTK): The scenario when handover is performed to a BWP which has different SSB than the one used during measurement should be considered as handover to an unknown cell. - Option 2 (Apple, HW, Nokia, CMCC): NCD-SSB and CD-SSB of the target cell carry same information. Option 1 not needed. - Option 3 (E///): Additional Trs delay for AGC when handover is performed to a BWP which has different SSB than the one used during measurement

In this contribution we further provide our view on the remaining open issues.

# Discussion

## NCD-SSB related Handover

Mismatch between the measured SSB type and HO SSB type

In last meeting, RAN4 further discussed whether additional delay extension is needed when NW configures the HO with a SSB type which is different as the SSB type used for measurement. Some companies believe additional delay is needed for fine AGC retuning and frequency/time tracking. However, after further check with RAN2 spec, we understand that UE can know the PCI of the SSB which is used for HO. If the PCI is the same as the SSB used for measurement, no additional delay is needed since the CD-SSB and NCD-SSB are QCLed. UE can get the AGC and timing info. from the measured SSB either CD-SSB or NCD-SSB.

Proposal 1: No additional delay is needed when UE handover to a BWP which has different SSB type with the one used for measurement.

# Summary

In this contribution we have discussed the mobility requirements for RedCap, and made following observations and proposals:

Proposal 1: No additional delay is needed when UE handover to a BWP which has different SSB type with the one used for measurement.

# References

- R4-2214485, “WF on RedCap RRM requirements”, Ericsson