3GPP TSG-RAN WG4 Meeting #104bis-e	R4-221xxxx

Electronic Meeting, Oct 10- Oct 19, 2022

Agenda Item:	4.6.3.1.5

Source: 	Ericsson

Title:	Discussions on RedCap measurement requirements

Document for:	Discussion

# Introduction

The NCD-SSB offset values agreed in last meeting as 5, 10, 15, 20, 40, 80ms. The remaining issue is captured as follow [1].

Issue 3-1-2: NCD-SSB time offset impact - Proposals - Option 1: When the SSB for intra-frequency measurement is fully-partially overlapping with the MG due to SSB offset, UE is required to perform intra-frequency measurement and drop the configured MG. (Ericsson) - Option 2: Up to NW configuration to address this issue (Apple vivo Huawei MTK) - Option 3: FFS (Huawei xiaomi)

In this contribution we further provide our view on the remaining open issues.

# Discussion

When NW configures different time offset for CD-SSB and NCD-SSB(s), RAN4 needs to further consider the impact to RAN4 measurement requirement. For example, if CD-SSB configures in the 1st half-frame, and NCD-SSB configures in the 2nd half-fame for the same frame(NCD-SSB offset = 5ms), when measurement gap is configured to measure one type of SSB, the MG may also partially overlap with the other type of SSB shown in the figure below.

Observation 1: The MG may collide with both CD-SSB and NCD-SSB due to time offset=5ms.

In legacy NR, NW can coordinate to align the SMTC positions among different frequency layers to guarantee the to be measured SMTC is within MG. Thus, RAN4 doesn’t discuss the scenario when SMTC and MG are partial overlapping. However, in RedCap, different type of SSBs will be configured with a offset since NCD-SSB is introduced. NW cannot coordinate to guarantee both SSBs are within the MG. In last meeting, some companies think NW can handle it by configuring short SSB duration. In Rel-17 NTN, a proximity condition between SMTC and MG is defined. From our understanding, the proximity condition is still met even if NW configures a shorter SSB duration.

Thus, RAN4 needs to define clear UE behaviours when the MG and the SMTC meets the proximity condition with the time distance = 4ms.

Proposal 1: In RedCap, RAN4 to define UE behaviour when the MG and the SMTC meets the proximity condition with the time distance = 4ms.

In Rel-17 concurrent measurement gaps discussion, a gap dropping rule is introduced. When teo MGs are overlapping, one of the MGs will be dropped. From our understanding, in RedCap, when the type of SSB for intra-frequency layer is fully-partially overlapping with the MG due to SSB offset, UE should perform intra-frequency measurement and drop the configured MG.

Proposal 2: When the SMTC for intra-frequency layer is fully-partially overlapping with the MG due to NCD-SSB offset, UE is required to perform intra-frequency measurement and drop the configured MG.

Figure 1. CD-SSB and NCD-SSB configured with time offset

# Summary

In this contribution we have discussed the mobility requirements for RedCap, and made following observations and proposals:

Proposal 1: In RedCap, RAN4 to define UE behaviour when the MG and the SMTC meets the proximity condition with the time distance = 4ms.

Proposal 2: When the SMTC for intra-frequency layer is fully-partially overlapping with the MG due to NCD-SSB offset, UE is required to perform intra-frequency measurement and drop the configured MG.

# References

- R4-2214486, “WF on RedCap RRM requirements”, vivo