3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216459

Electronic Meeting, Oct 10- Oct 19, 2022

Agenda Item:	6.21.2

Source: 	Ericsson

Title:	Discussions on MUSIM gaps

Document for:	Discussion

# Introduction

In last meeting, RAN4 had an initial discussion on Multi-SIM requirements for MUSIM gap patterns with some progress [1].

Sub-topic 2-1 General aspects Issue 2-1-1: On MUSIM gap patterns - Proposals - Option 1: All specification work listed in the 2nd item of WI “Define RRM requirements for Rel-17 MUSIM gaps” are based on existing Rel-17 MUSIM gap patterns defined in Table 9.1.10-1 of TS38.133 (Apple Ericsson MTK CMCC Huawei Xiaomi Charter Qualcomm Oppo vivo Nokia) - Option 2: Keep it open Tentative agreements: option 1 is agreed Issue 2-1-2: On MUSIM gap pattern purpose - Proposals - Option 1: All MUSIM gaps cannot be used by any measurements configured by network A and all network A measurements are carried out outside MUSIM gaps. (Apple MTK CMCC Huawei xiaomi Charter Qualcomm oppo Ericsson vivo Nokia) - Option 1a: MUSIM gaps do not fulfil any measurement objectives on network A (Qualcomm) - Option 2: it is necessary to discuss whether MUSIM gap patterns can be used for RRM measurement or only used for MUSIM (CMCC) - Option 3: Open to option 2 in case that measurements configured by NW-A is fully overlapped with MUSIM gap (oppo) Tentative agreements: Option 1 is agreed

In this contribution, we will continue to discuss the RRM requirements for Rel-17 MUSIM gaps.

# General principles

The main issues for MUSIM gaps are how to handle the collision scenarios as follow:

- Collisions between MUSIM gap and legacy measurement gap

- Collisions between MUSIM gap and SMTC and other L3/L1 measurement resources

- Collisions between different MUSIM gaps

Before going to the detail for each scenario, RAN4 should agree the general principles. From our understanding, the first issue is how to minimize the impact on both NW-A and NW-B.

## Paging monitoring in NW-B

In last meeting, some companies proose the paging issue: how to guarantee the UE to monitor paging for NW-B.

Issue 2-3-1-1: On network A priority assignment scheme - Proposals: - Option 1: RAN4 to study the issue when the priority is all assigned by NW A, under the current signalling framework, which might lead to missing significant activities in NW B due to MUSIM gap collision handling (e.g., reading the paging in NW B, which are unknown to NW A) (MTK Apple Ericsson Huawei QC oppo vivo) - Option 2: Clarification is needed for option 1 (Nokia) Tentative agreements: Further study on the issue in option 1.

On the one hand, in Idle mode, the most important procedure for UE is to monitor paging to avoid any missing of the call from network. On the other hand, the configured measurement gap for L3 measuerment is a periodic procedure for NW-A. Furthermore, the paging periodicity in Idle mode is sparser than MGRP for NW-A, such as the typical paging periodicity is 1.28s in the network. Thus, paging should be kept when the paging occasion for NW-B is colliding with measurement gap in NW-A. In conclusion, when RAN4 discusses the gaps collision between MUSIM gaps and legacy MG, it should differentiate the different usages of the MUSIM gaps.

Observation 1: To support MUSIM, paging monitoring is one of the key procedures in NW-B IDLE mode.

Observation 2: Paging occasions in NW-A’s IDLE mode is sparser than MGRP in NW-B’s CONNECTED mode.

Proposal 1: When MUSIM gaps collide with legacy MG, RAN4 to differentiate different usages of the MUSIM gaps, such as L3 measurement for cell reselection, pagining monitoring etc.

Proposal 2: General principle 1: The paging for NW-B cannot be dropped when the paging occasion is colliding with MG in NW-A.

Furthermore, in NR paging monitoring, an additional AGC retuning is also needed before UE waking up for PO monitoring after a long DRX cycle. Thus, to guarantee UE successfully monitoring paging, the SSB before paging for AGC should also be kept. Normally, the SSB for AGC retuning shall be ahead of paging but less than 160ms.

Proposal 3: General principle 1: The SSB for paging AGC retuning in NW-B cannot be dropped when the SSB occasion is colliding with MG in NW-A if the time distance between the SSB and paging occasion is less than 160ms.

## RRM mobility procedures in NW-A

In last meeting, another important issue is how to handle the collision between MUSIM gaps and signals for NW-A’s mobility procedures.

Issue 2-3-3-2: Priority of MUSIM against SMTC, and other L3/ L1 measurement resources - Proposals: - Option 1: MUSIM gaps should have high priority against SMTC and L1 measurement resources (Apple xiaomi oppo Qualcomm Huawei MTK vivo) - Option 2: NW-A’s RRM procedure, including DL SMTC should have higher priority than MUSIM gaps. The MUSIM periodic gaps should be dropped once the gap proximity rule is met. (Ericsson) - Option 3: As baseline solution, UE can only perform gap-less L3 measurement and L1 operation outside MUSIM gap. Other solutions are not precluded to handle collision between MUSIM gap and SMTC/RS for L1 operation. (Apple oppo xiaomi) - Option 4: FFS (Ericsson Nokia CMCC vivo) Tentative agreements: No Issue 2-3-3-3: Priority of MUSIM against uplink signals, such as PRACH, CSI-RS reporting etc. - Proposals: - Option 1: NW-A’s RRM procedure, including UL CSI-RS, PRACH, should have higher priority than MUSIM gaps. The MUSIM periodic gaps should be dropped once the gap proximity rule is met. (Ericsson) - Option 2: PRACH procedure can be higher priority than MUSIM gaps (MTK) - Option 3: FFS (Huawei Ericsson Apple Nokia CMCC Qualcomm Xiaomi MTK vivo) Tentative agreements: No

In the MUSIM WID, it is clearly captured the justification as follow.

## 3 Justification … In NR Rel-17 specification, gap patterns particularly for MUSIM purpose were introduced. However, corresponding RRM requirements are not specified due to lack of RAN4 TUs for Rel-17 MUSIM WI. Without corresponding RRM requirements, implementing Rel-17 MUSIM feature in practical deployment may not guarantee minimized impact on network A and there could be interoperability issues. In order to guarantee network performance, particular for network A, it is desirable to define RRM requirements for MUSIM WI in Rel-18 standards for both the core requirements and corresponding performance parts.

As mentioned in the WID, the main intention to define the MUSIM gaps requirement is to guarantee minimized impact on NW-A’s performance. We noticed that the requested MUSIM gaps are basically periodical gaps for measurement, paging monitoring. However, some mobility procedures in NW-A are one-shot procedures, such as Handover, Re-establishment, RRC redirection, SCell activation. These procedures are very important from NW-A. If the proceudre’s delay is extended, it will have severe impact to NW-A. On the other hand, the L1/L3 measurement procedures are periodic procedures.

Furthermore, dropping some important procedures for network B may be also not permitted or have severe impact on MUSIM KPI, for example, the PRACH for on-demand SI or paging monitoring for NW-B. In MUSIM gaps, a new type of aperiodic gap is also introduced. To avoid missing the important procedure for NW-B, UE can request an aperiodic MUSIM gap with a higher priority. Considering aperiodic gap is a one-shot gap, the aperiodic MUSIM gap can be prioritized once colliding with other MGs.

Thus, when RAN4 discusses the MUSIM gaps colliding with DL RS or UL signals, it’s necessary to differentiate the usage of the DL RSs and UL signals.

Proposal 4: When MUSIM gaps collide with DL RS or UL signals, RAN4 to differentiate different usages of the DL RSs and UL signals in NW-A, such as SMTC for L3 measurement, SMTC for Hanover.

Proposal 5: General principle 2: RAN4 to define the priorities for each procedure in both NW-A and NW-B in desending order as follow. The gaps or resources for higher priority procedures should be kept once the collision happens.

- Priority 1: One-shot RRM mobility procedures in NW-A, such as Handover/ Re-establishment/RRC redirection/SCell activation/SI update;

- Priority 2: Periodic paging monitoring or one-shot procedure in NW-B Idle mode, such as On-demand SI reading, SIB reading;

- Priority 3: Measurements procedures for both NW-A and NW-B

In last meeting, some companies proposed to prioritize MUSIM gaps than L1/L3 measurement for NW-A. At the same time, companies proposed to further discuss whether apply the priority rule or introduce the sharing rule. After further thinking, we believe it’s too hurry to get the conclusion in such beginning stage of the WI. Thus, RAN4 should further discuss the following two possible solutions:

- RAN4 believes MUSIM gaps for measurement as ‘gap’ and apply the similar rule as legacy MG. MUSIM gaps should have high priority against SMTC and L1 measurement resources.

- RAN4 follows the NTN to define the proximity between SMTC/L1 measurement resources with MUSIM gaps.

  - Apply priority rule between MUSIM gaps and SMTC and L1 measurement resources for NW-A based on NW-A’s priority indication

  - Apply sharing rule between MUSIM gaps and SMTC and L1 measurement resources for NW-A

Proposal 6:  General principle 3: RAN4 to discuss the following possible solutions to handle the collision between MUSIM gaps for NW-B Idle mode measurements and the RS resources for NW-A’s L1/L3 measurement.

- Option 1: RAN4 believes MUSIM gaps for measurement as ‘gap’ and apply the similar rule as legacy MG. MUSIM gaps should have high priority against SMTC and L1 measurement resources.

- Option 2: RAN4 follows NTN to define the proximity between SMTC/L1 measurement resources with MUSIM gaps.

  - Apply priority rule between MUSIM gaps and SMTC/L1 measurement resources for NW-A based on NW-A’s priority indication, or

  - Apply sharing rule between MUSIM gaps and SMTC/L1 measurement resources for NW-A

# MUSIM gaps requirement for network A

MUSIM gap group

In Rel-17, RAN4 introduced a new gap capability, concurrent gaps(Con-MGs) which is a good framework to incorporate the MUSIM gaps.

Issue 2-3-1: General principles on gap collision handling - Proposals: - Option 1: For priority based solution, priorities can be allocated to each existing gap patterns and when two or more gap collide, only the highest priority gap is kept and all other gaps are dropped (Apple Huawei Xiaomi vivo) - Option 2: Apply gap-group priority to handle collisions between different gaps groups (i.e., MUSIM gaps group and legacy MGs group). Then, within each gap group, apply different priorities to handle the collision between the gaps within the same group (Ericsson Charter) - Option 3: Agree at high-level that applying priority rule to handle collisions, but the way how to apply it can be FFS (oppo MTK CMCC vivo) - Option 4: priority-based scheme for (a) Collisions between a MUSIM gap and measurement gaps and (b) Collisions between MUSIM gaps, but the definition of collisions may be different for cases a and b. (Qualcomm) - Option 5: FFS (Nokia)

As we discussed in last meeting, MUSIM gaps can be treated as a gap group which can have the following benefits. In Rel-17, at most 2 gaps are supported in concurrent gaps and it seems most of companies suggest to keep this number in R18 MG enh WI. If we believe MUSIM gaps as ‘one gap’, then we don’t need to further discuss this issue on the max number og gaps but following legacy concurrent gaps agreement. Both NW and UE can believe MUSIMs gap as one gap. The gap overlapping rule can be used to indicate which group of gaps shall be prioritized once overlapping happens.

Figure 1: Gap overlapping between MG for L3 mobility and the new MGs for MU-SIM

Proposal 7: MUSIM gaps can be believed as a gap set with a single priority.

- The gap for paging monitoring can be an exception within the gap set.

Gap collision between MUSIM gaps with othter gaps

In last meeting, the possible proposals to handle the collision between MUSIM gap and legacy MG are shown as follow.

Issue 2-3-2-2: Collisions handling rules between MUSIM gap and legacy measurement gap - Proposals: - Option 1: Priority-based gap collision handling introduced in concurrent gaps design can be used as a base for collisions between MUSIM gap and legacy measurement gap (Charter Apple CMCC Xiaomi oppo Qualcomm vivo Huawei MTK Ericsson Nokia) - Option 1a: Request RAN2 to introduce optional signalling so that the UE can request the priority level of MUSIM gaps (relative to measurement gaps) via UAI (Qualcomm) - Option 2: Other enhanced gap collision solutions are open for study. (Charter Apple CMCC Xiaomi oppo Qualcomm vivo Huawei MTK Ericsson Nokia) Issue 2-3-2-3: Priority of MUSIM against other legacy gaps - Proposals: - Option 1: MUSIM gaps should have high priority in the event of a collision (Charter Qualcomm) - Option 2: MUSIM gaps can be defined as the lowest priority, and periodic MUSIM gaps will be dropped once the gap dropping rule defined in Con-MGs is met () - Option 3: Up to NW configuration (Apple CMCC Huawei Xiaomi Charter oppo vivo) - Option 3a: Up to UE request (MTK) - Option 4: FFS (Ericsson Nokia)

As we know, the key obstacle of this discussion is how to differentiate the paging gap and measurement gap within MUSIM gaps. However, if we apply the general pricinples defined in section 2 in this issue, the collision can be handled clearly.

- MUSIM paging and AGC occasions should have higher priority than NW-A MG if collision happens.

- RAN4 to further discuss whether priority rule or sharing rule will be applied when MUSIM other gaps collide with NW-A’s MG

Natually, both NW-A and UE should have the same understanding on which MUSIM gap is used for paging. Furthermore, from NW’s perspective, the MUSIM gap for paging should be an exclusive gap. In other words, monitoring paing in more than one MUSIM gaps isn’t allowed.

Proposal 8: Both NW-A and UE should have the same understanding on which MUSIM gap is used for paging monitoring. RAN4 to further discuss how to identify this paging gap within MUSIM gaps.

Proposal 9: UE should request an exclusive MUSIM gap for paging instead of monitoring paging in several MUSIM gaps.

Proposal 10: RAN4 to use the proximity condition to define the collision between MUSIM gaps with legacy MG.

Proposal 11: When MUSIM gaps collide with legacy MG,

- MUSIM paging and AGC occasions should have higher priority than NW-A MG

- Aperiodic gap for On-demand SI or other usage should have higher priority than NW-A MG

- The priority between other MUSIM gaps and legacy MG can be indicated by NW

  - Whether priority rule or sharing rule will be applied for other MUSIM gaps is FFS

Collision between MUSIM gaps with DL RS or UL signals in NW-A

As mentioned before, there is no good coordination between NW-A and NW-B. Due to the timing difference, the MUSIM gaps may collide with the SMTCs which may impact the NW-A’s performance. However, if we apply the general pricinples defined in section 2 in this issue, the collision can be handled clearly. The issue will be the condition to apply the collision rule. From our understanding, the condition of SMTC/SSB overlapping with legacy gap cannot be reused since both the SMTC/SSB and legacy MG are all configured by NW-A. Thus, the partially overlapping between SMTC and MG is a corner case. However, MUSIM gaps are requested by UE based on NW-B’s configuration. The timing drifting issue is similar as NTN. Thus, we propose to reuse NTN’s proximity between SMTC and MG to define the collision condition

Proposal 12: RAN4 to use the proximity condition to define the collision between MUSIM gaps with SMTC and L1 measurement resources.

Collision within MUSIM gaps

The time proximity between SSB and related PO is uncertain depending on different SSB and PO multiplexing pattern, default/non-default association between SSB and PO, and SSB index indication(ssb-position-in-burst). On the one hand, if the time proximity of SSB occasion and PO is larger than 4ms, two independent gaps with MGL=6ms are preferred. In this case, no MUSIM gaps’ collision happens.

On the other hand, if the time proximity of SSB occasion and PO is shorter than 4ms, these two MUSIM periodic gaps can be believed as collision. To avoid the further considering the collision among MUSIM periodic gaps, one single gap with long MGL is preferred. That’s the reason to introduce two long MGLs with 10ms and 20ms to handle the AGC retuning and PO monitoring within one gap. However, both MUSIM gaps should not be dropped once UE requests two periodic gaps which meets the collision proximity.

Observation 3: To avoid the collision within MUSIM gaps, UE can request a single periodic gap instead of two separate periodic gaps provided that the distance between these two gaps is shorter than 4ms.

Proposal 13: When the time duration between the two closest gap occasions within the two measurement gap patterns is shorter than [4]ms and the second gap occasion is for paging, UE should keep both gap occasions instead of dropping any of them.

Figure 2: Example of MG for PO monitoring in NW-B Idle mode

In other words, when UE request these MUSIM gaps, only UE knows how to use these gaps or the purpose to request such MUSIM gap patterns. On the one hand, NW-A cannot decide which MUSIM gaps should be dropped or kept based on limited information. On the other hand, UE definitely will schedule reasonable MUSIM gaps’ configuration with some dedicated purposes when UE requests the MUSIM gaps. It’s impossible to waste the gap occasions on purpose from UE side. Thus, from network’s perspective, all MUSIM gaps requested by UE should be kept even if the collision proximity is met. RAN4 can further identify the specific scenarios in which the collision rule shall be applied.

Observation 4: NW-A doesn’t know how to use the MUSIM gaps requested by UE.

Observation 5: UE won’t waste the gaps on purpose when UE request the MUSIM gaps.

Proposal 14: All MUSIM gaps requested by UE should be kept even if the collision proximity is met.

Proposal 15: RAN4 to further identify the specific scenarios in which any MUSIM gap shall be dropped case by case.

Aperiodic gap

The last remaining issue for NW-A is aperiodic gap.

Issue 2-3-5-1: On aperiodic gap priority - Proposals: - Option 1: UE can request aperiodic MUSIM gap with a higher priority. (Ericsson Charter) - Option 2: Option 1 is up to UE implementation (vivo) - Option 3: FFS (Apple CMCC Huawei Qualcomm Xiaomi MTK)

As we discussed before, aperiodic gap shall have higher priority than other periodic MUSIM gaps. There is no reason to request aperiodic gap as lower priority and then drop the aperiodic gap when colliding with other MUSIM gaps. The longer delay for measurement is expected due to gap cancellation. The remaining issue is how to handle the collision between aperiodic gap and paging gap. As we mentioned before, UE will request the MUSIM gaps with a reasonable purpose based on gap scheduling design. Thus, we think it’s impossible to request an aperiodic gap which is colliding with paging gap. Thus, the scenario won’t happen.

Proposal 16: Aperiodic gap should have higher priority than periodic gaps once collision happens within MUSIM gaps.

# MUSIM gaps requirement for network B

In current WI, there is an open issue on measurement requirement for NW-B’s Idle/Inactive mode. It is said up to RAN4 to decide whether to define requirements for NW-B. From network’s perspective, it’s important to define an Idle mode requirement for NW-B which is helpful for both NW-A and NW-B. NW-A can further understand UE’s beahviour under the MUSIM gaps and the gap usage ratio based on the pre-defined requirement once NW-B’s requirement is defined. Otherwise, the UE’s behaviour under the MUSIM gaps will be a black box to NW-A. At the same time, UE’s behaviour under the MUSIM gaps is also a black box to NW-B if no requirement for NW-B Idle mode is defined. When UE is in NW-B’s Idle mode, UE should continue the measurement to be aware of possible change of cell/channel, such as cell evaluation requirement. If UE doesn’t perform cell reselection immediately, UE may miss the paging in a poor serving cell. To guarantee UE’s performance in NW-B’s Idle mode, the requirement should be defined.

Proposal 17: RAN4 to define measurement requirement for NW-B Idle mode which is helpful for both NW-A and NW-B.

- Summary

In this contribution, we have discussed the MUSIM gaps requirements. Based on the discussions, we have made following proposals and observations:

Observation 1: To support MUSIM, paging monitoring is one of the key procedures in NW-B IDLE mode.

Observation 2: Paging occasions in NW-A’s IDLE mode is sparser than MGRP in NW-B’s CONNECTED mode.

Observation 3: To avoid the collision within MUSIM gaps, UE can request a single periodic gap instead of two separate periodic gaps provided that the distance between these two gaps is shorter than 4ms.

Observation 4: NW-A doesn’t know how to use the MUSIM gaps requested by UE.

Observation 5: UE won’t waste the gaps on purpose when UE request the MUSIM gaps.

Proposal 1: When MUSIM gaps collide with legacy MG, RAN4 to differentiate different usages of the MUSIM gaps, such as L3 measurement for cell reselection, pagining monitoring etc.

Proposal 2: General principle 1: The paging for NW-B cannot be dropped when the paging occasion is colliding with MG in NW-A.

Proposal 3: General principle 1: The SSB for paging AGC retuning in NW-B cannot be dropped when the SSB occasion is colliding with MG in NW-A if the time distance between the SSB and paging occasion is less than 160ms.

Proposal 4: When MUSIM gaps collide with DL RS or UL signals, RAN4 to differentiate different usages of the DL RSs and UL signals in NW-A, such as SMTC for L3 measurement, SMTC for Hanover.

Proposal 5: General principle 2: RAN4 to define the priorities for each procedure in both NW-A and NW-B in desending order as follow. The gaps or resources for higher priority procedures should be kept once the collision happens.

- Priority 1: One-shot RRM mobility procedures in NW-A, such as Handover/ Re-establishment/RRC redirection/SCell activation/SI update;

- Priority 2: Periodic paging monitoring or one-shot procedure in NW-B Idle mode, such as On-demand SI reading, SIB reading;

- Priority 3: Measurements procedures for both NW-A and NW-B

Proposal 6:  General principle 3: RAN4 to discuss the following possible solutions to handle the collision between MUSIM gaps for NW-B Idle mode measurements and the RS resources for NW-A’s L1/L3 measurement.

- Option 1: RAN4 believes MUSIM gaps for measurement as ‘gap’ and apply the similar rule as legacy MG. MUSIM gaps should have high priority against SMTC and L1 measurement resources.

- Option 2: RAN4 follows NTN to define the proximity between SMTC/L1 measurement resources with MUSIM gaps.

  - Apply priority rule between MUSIM gaps and SMTC/L1 measurement resources for NW-A based on NW-A’s priority indication, or

  - Apply sharing rule between MUSIM gaps and SMTC/L1 measurement resources for NW-A

Proposal 7: MUSIM gaps can be believed as a gap set with a single priority.

- The gap for paging monitoring can be an exception within the gap set.

Proposal 8: Both NW-A and UE should have the same understanding on which MUSIM gap is used for paging monitoring. RAN4 to further discuss how to identify this paging gap within MUSIM gaps.

Proposal 9: UE should request an exclusive MUSIM gap for paging instead of monitoring paging in several MUSIM gaps.

Proposal 10: RAN4 to use the proximity condition to define the collision between MUSIM gaps with legacy MG.

Proposal 11: When MUSIM gaps collide with legacy MG,

- MUSIM paging and AGC occasions should have higher priority than NW-A MG

- Aperiodic gap for On-demand SI or other usage should have higher priority than NW-A MG

- The priority between other MUSIM gaps and legacy MG can be indicated by NW

  - Whether priority rule or sharing rule will be applied for other MUSIM gaps is FFS

Proposal 12: RAN4 to use the proximity condition to define the collision between MUSIM gaps with SMTC and L1 measurement resources.

Proposal 13: When the time duration between the two closest gap occasions within the two measurement gap patterns is shorter than [4]ms and the second gap occasion is for paging, UE should keep both gap occasions instead of dropping any of them.

Proposal 14: All MUSIM gaps requested by UE should be kept even if the collision proximity is met.

Proposal 15: RAN4 to further identify the specific scenarios in which any MUSIM gap shall be dropped case by case.

Proposal 16: Aperiodic gap should have higher priority than periodic gaps once collision happens within MUSIM gaps.

Proposal 17: RAN4 to define measurement requirement for NW-B Idle mode which is helpful for both NW-A and NW-B.

- References

- R4-2214349, “WF on RRM requirements for Rel-17 MUSIM gaps”, Vivo