3GPP TSG-RAN WG4 Meeting #104-bis-e		         R4-2216496

Online, 10th Oct – 19th Oct, 2022

Source: 	Keysight Technologies

Title: 	FR2-2 BS conformance test consideration for Tx testing 
Agenda Item:	4.3.4.2

Document for:	Discussion

# Introduction

FR2-2 BS conformance test topic was discussed in previous meetings. This contribution explains our view and further analysis of Tx testing including some of the points raised and captured in WF [1].

# Discussion

- Regarding with applicability and suitability of OTA chamber,

- CATR is applicable for 71 GHz extension, with up to 142 GHz spurious measurement. There is test system setup changes are needed to switch from cables to wageguide, antenna, etc. at certain frequency below 142 GHz.  Also chamber and reflector etc. should be designed to be capable to work for this range of frequency.

  - Regarding with focal length and pathloss, with using equation listed in TR38.810 clause 5.2.3.2 (focal length = 3.5 * 2 * D ), with using largest assumption of Antenna size for 52.6 GHz (91.3mm x 91.3mm) ~ 71G (67.6mm x 67.6mm) (this is from array size 32x32 assumption)

    - This makes 904mm for focal length (for 52.6 GHz 32x32 case) which has 66 dB pathlos

    - With some margin, take 1.0 m focal length as assumption, which has 66.9 dB @ 52.6 GHz, 69.3 @ 70 GHz, 73.3 dB @ 110 GHz, and 75.5 dB @ 142 GHz as pathloss

    - In most case, LNA is needed. Use of LNA should be assumed.

- Following is link budget with some adjusted level/components assumptions

| Frequency | IFF (CATR) system F = 70 GHz | IFF (CATR) system F = 110 GHz | IFF (CATR) system F = 142 GHz |
| --- | --- | --- | --- |
| Equipment noise floor (dBm/MHz) | -73 | -55 | -62 |
| Cable loss 2.0m x 10dB/m = 20 dB | 20 dB | N/A | N/A |
| Waveguide 1.5m (3.0 dB/m) with 0.5m co-ex cable (13.4 dB/m) for 110 GHz | N/A | 11.2 dB | N/A |
| Waveguide 1m (4.6 dB/m) with 1m IF cable (2.0dB/m) for 142 GHz | N/A | N/A | 6.6 dB |
| Measurement antenna gain (dBi) | 13 | 24 | 24 |
| Pathloss @70 G, 110G, 142 GHz (dB) | 69.3 | 73.3 | 75.5 |
| Measurable signal (lower level) (dBm/MHz) before LNA | +3.3 | +5.5 | -3.9 |
| Assumed LNA gain for each frequency 70G, 110G, 142G (Gain-NF) | 30 | 28.5 | 28.5 |
| Measurable level (dBm/MHz) | -26.7 | -23.0 | -32.4 |

- Applicability of power sensor as power measurement equipment for Tx tests

There are multiple points to consider and well managed for use of power sensor/meter as power measurement equipment for BS conformance test use:

  - Power sensor/meter takes all of signal power within supported frequency range.  It’s important to make sure there is only signal which to measure power exist inside supported frequency range. With this regard, possibility to use power sensor for BS conformance testing is Channel power measurement only.

  - Thermal power sensor should be used for better accuracy, with input signal range inside -30 dBm ~ +10 dBm at sensor input connector. This limits applicability of power sensor very limited. Use of power sensor should be considered as optional, should not be used for MU calculation. Because when it’s used in MU calculatoin, it mandates to use power sensor.

  - When power sensor is used, it should be used with averaging mode with considering UL/DL ratio. Measured result should be adjusted (calculated) to be actual power when signal exist with considering UL/DL ratio. (Note, for conformance test, test model has fixed UL/DL ratio so that it’s possible to do this calculation however, for the case when UL/DL ratio changes not fixed and not predetermined, it’s not possible to do such calculation.)

- Measurement uncertainty and Calibration consideration

  - Total test system MU calculation principle is described in TR37.941[2] clause “5.2 Uncertainty budget calculation principles” and BS conformance test follows this principle for MU calculation. As it’s stated, this is based on the ISO Guide on Evalulation of measurement data document [3] (a.k.a GUM)

  - In GUM [3], for MU budget calculation, either Type A evaluation or Type B evaluation approach is possible for each MU term. (Note, Type A is terminology used in GUM, which is to do multiple measurement results for estimating uncertainty statistically, while Type B is terminology for use with manufacter’s specification and each individual equipment device calibration results for MU term) Approach used for conformance test is to use Type B evaluation with using manufacture (in this case Test Equipment vender for test equipment) numbers. Alternative approach is Type A evaluation which is to use measured results (observations), however, number of measured results (observations) should be large enough to provide reliable estimate so that it’s not practical. Please note that, in this case, many numbers of measurements done on each of various conditions. We understand that this is why conformance testing uses Type B evaluation approach.

  - Manufactorer’s MU number may be looked a bit larger but this is because covering and assuring various condition in measurement by smaller number of values to represents various condition includes variation by each individual test setup, environment (ex. temperature/humidity variation), signal type, frequency range and level, bandwidth, etc.

  - In the case of conformance testing, there is calibration procedure in test procedure which is to measure some test system characteristic such as loss etc. this is procedure to make sure test system behavior and ensure these are in range of MU terms. It’s not for replace MU term by other MU term.

- Measurement Uncertaity consideration

  - With frequency goes high, mis-match term increases due to higher return loss (larger SWR) of each connection point. With assuming use of following configuration

    - Feed Antenna – Wageguide adapter – cable – switch for paththroug or attenuator or LNA – cable – Test Equipment

    - Actual number varies depending on frequency and connectors and components. While there is no common system assembly assumption for BS conformance testing, rough estimate can be in range of 1.0 ~ 1.5 (as 1-sigma) up to 110G, and FFS for 142 GHz. This needs more study. As for information, assumed system components are different for UE system but UE system assumes 2.30 (as 1-sigma) for spurious system up to 80 GHz for mis-match term MU.

  - For Power measurement equipment MU, In FR2-1 BS spurious measurement, following values used as agreed value for frequency range from 40 GHz to 60 GHz with use of external mixer in assumption (TR37.941)

      - 0.60 (as 1-sigma value) for RF power measurement equipment (in this case, IF measurement)

      - 2.25 (as 1-sigma value) for mixer

      - Combined value of above two by RSS is 2.33 – this can be considered as power measurement equipment MU (1-sigma) value of mixer use for spurious emission test up to 60 GHz (in TR37.941)

  - For power measurement equipment MU up to 80 GHz for spurious measurement, following two are option and choose one of these as agreement,

    - With respect to previously agreed values, and as compromise, use 2.33 as 1-sigma value from 40 GHz to 60 GHz, use 2.0 as 1-sigma value from 60 GHz t0 80 GHz.

    - Use 2.33 as 1-sigma value to extend to from 40 GHz to 80 GHz for spurious measurement power measurement equipment number, this is to keep using the same value already used in TR37.941

  - For above 80 GHz, we have agreed to assume use of mixer for spurious measurement, power measurement equipment MU for spurious measurement would be larger than 2.33 (as 1-sigma value).

    - Mixer MU number increases as frequency coverage wider when frequency goes up.  Comparing WR-19 mixer (40 GHz ~ 60 GHz) and WR-8 mixer (90 GHz ~ 140 GHz), frequency range is 2.5 times wider coverage. Although this doesn’t mean uncertainty degrade 2.5 times, but some degradation expected for WR-8 mixer in comparison with WR-19 mixer. Actual value is FFS at this point.

  - For other testing, Channel power, ACLR, Tx Off power, MU numbers are different from Spurious emission test because of various test condition (ex. for ACLR, mesure noise level adjacent to channel power which requires enough dynamic range) and signal type such as modulated wider bandwidth.

  - For other measurement, we should agree 1st on use of existing principle in TR37.941 1st before discuss these numbers and spurious emission measurement number up to 80 GHz.

  - For network anayzer MU value, previously used value up to 60 GHz for TR 37.941 is actually too small and challenging. However, while it’s already used in calculation, propose following as compromise for BS test system setup.

    - Up to 60 GHz, use 0.30 (as 1-sigma value) then 60 ~ 80 GHz, use 0.85 (as 1-sigmal value)

# Proposal

Chamber applicability

Proposal-1

- CATR is applicable for 71 GHz extension test case up to 142 GHz spurious measurement. CATR system should be designed to cover device antenna size and pathloss for this much frequency.

- Define 142 GHz as max frequency of spurious measurement

Calibration and procedure consideration for possible reduction of total system MU

Propsal-2

- Total test system MU calculation principle described in TR37.941, which is based on the ISO Guide on Evaluation of measurement dat document, should be continuously used and existing MU budget table should be used for 71GHz extention MU calculation.

- Calibration procedure assure defined and calculated MU, which doesn’t improve MU. Swithcing to Type A evaluation is not practical. (note, Type A evaluation means, use measured result (calibration results) to estimate MU. In this case, calbration procedure in test procedure is to measure some test system characteristic such as loss etc. Use of such results for MU estimation requires enough and many amount of measured results under various conditions. Which is not practical). In short, calibration procedure/method arrangement doesn’t improve system MU.

Use of power meter/sensor for Tx testing

Proposal-3

- Use of power sensor for conformance testing should be considered as optional (shouldn’t be mandated) when it’s applicable because usable power level range is very limited. Only possibile use is channel power measurement because senser takes all of signal power comes into sensor. (can’t measure ACLR as example)

Test equipment measurement uncertainty

Proposal-4

- For power measurement equipment MU up to 80 GHz for spurious measurement, following two are option and agree with one of these,

    - With respect to previously agreed values, and as compromise, use 2.33 as 1-sigma value from 40 GHz to 60 GHz, use 2.0 as 1-sigma value from 60 GHz t0 80 GHz.

    - Use 2.33 as 1-sigma value to extend to from 40 GHz to 80 GHz for spurious measurement power measurement equipment number, this is to keep using the same value already used in TR37.941

Proposal-5

- For test equipment MU value for other measurement case (such as ACP, Tx Off power), we should agree 1st on use of existing principle in TR37.941 first (in proposal-2) and existing MU budget table before discuss these numbers.

Proposal-6

- For network anayzer MU value, previously used value up to 60 GHz for TR 37.941 is actually too small and challenging. However, while it’s already used in existing MU calculation, propose following as compromise for BS test system setup.

    - Up to 60 GHz, use 0.30 (as 1-sigma value), for 60 GHz ~ 80 GHz use 0.85 (as 1-sigmal value)

# References

[1] R4-2214373, WF on FR2-2 BS test environments

[2] TR 37.941 V17.0.0 (2022-03) RF conformance testing background for radiated BS requirements (Release 17)

[3] JCGM 100:2008 “Evaluation of measurement data – Guide to the expression of uncertainty in measurement”

[4] R4-2214374, WF on FR2-2 BS conformance testing

[5] R4-2213925, about FR2-2 BS conformance test system, Tx system link budget, Keysight