3GPP TSG-RAN4 Meeting #104bis-e	R4-2216509

# Electronic Meeting, 10 – 19 October, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2638 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on requirements for UE Rx-Tx measurement for PDC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Introduction of DRX for PRS and TRS Measurement Period - Introduction of Kgap PRS Measurement Period. - Simplification of requirements related to TPRS. CR is based on agreed CR in R4-2214524 CR on requirements for UE Rx-Tx measurement for PDC, agreed in RAN4#104. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduction of DRX for PRS and TRS Measurement Period - Introduction of Kgap PRS Measurement Period. - Simplification of requirements related to TPRS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDC measurement requirements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 9.12.4	Measurement period requirements

#### 9.12.4.1	PRS Measurement Period

When UE is configured with prs-Ref-r17 in MeasObjectRxTxDiff-r17 defined in TS 38.331 [2] and provided with PRS resource configuration, the UE shall be able to measure UE Rx-Tx time difference on PCell within the measurement period TUERx-Tx_PRS, where:

TUERxTx, PRS=NPRSslotN'Lavailable_PRSN*Nsample-1NPRSslotN'*Nsample*TeffectPRS*TDRX*Kgap_overlap+Tlast

Where:

Lavailable_PRSTPRS

NPRSslot is the maximum number of DL PRS resources configured in a slot,

{N,T}

N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to [maxNumOfDL-PRS-ResProcessedPerSlot] as specified in [TBD],

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample= 4,

- TlastTlastTTPRS

Teffect

Teffect=TPRS

Teffect= TTPRS*TPRS

T

TPRS is the PRS resource periodicity specific for RTT-based propagation delay compensation. TPRS is applicable only if TPRS≥DRX.

TDRX is the DRX cycle in use if DRX is configured (section 3.6.1). Otherwise, TDRX=1. TDRX is:

If TPRS<DRX: TDRX=DRX;

If TPRS=DRX: TDRX=1;

If TPRS>DRX: TDRX=1;

Kgap_overlap is the ratio accounting when PRS occasiosn are not available due to overlapping with a measurement gap during the measurement period and is defined as [FFS]. Kgap_overlap=	1 if measurement gaps are not configured or active.

TUERxTx, PRS

UE is only required to perform UE Rx-Tx time difference on PRS within the active DL BWP.

When UE is configured to perform UE Rx-Tx time difference measurement based on PRS, the requirements apply provided that the SCS of the PRS is same as that of the active BWP on PCell.

For FR2, the requirements apply if QCL information is provided for all the PRS resources in the resource set and for each resource one RS has QCL-TypeD.

Editor’s Note: FFS how to develop requirements when DRX is in use.

#### 9.12.4.2	TRS Measurement Period

When UE is configured with csi-RS-Ref-r17 in MeasObjectRxTxDiff-r17 defined in TS 38.331 [2] and provided with TRS resource configuration, the UE shall be able to measure UE Rx-Tx time difference on PCell within the measurement period TUERx-Tx_TRS, where:

TUERxTx,TRS=Nsample *TTRS *TDRX *Kgap_overlap

Where

-	Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample is [4],

TDRX is the DRX cycle in use if DRX is configured (section 3.6.1). Otherwise, TDRX=1. TDRX is:

If TTRS<DRX: TDRX=DRX;

If TTRS=DRX: TDRX=1;

If TTRS>DRX: TDRX=1;

Kgap_overlap is the ratio accounting when TRS occasiosn are not available due to overlapping with a measurement gap during the measurement period and is defined as [FFS]. Kgap_overlap=	1 if measurement gaps are not configured or active.

-	TTRS is the TRS resource periodicity specific for RTT-based propagation delay compensation.

UE is only required to perform UE Rx-Tx time difference on TRS within the active DL BWP.

When UE is configured to perform UE Rx-Tx time difference measurement based on TRS, the requirements apply provided that the SCS of the TRS is same as that of the active BWP on PCell.

For FR2, the requirements apply if QCL information is provided for all the PRS resources in the resource set and for each resource one RS has QCL-TypeD.

Editor’s Note: FFS how to develop requirements when DRX is in use.

<End of Change 1>