3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216510

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Measurement accuracy requirements for TUE-RX

Agenda item:	4.7.2

Document for:	Agreement

Introduction

RAN4 has been discussing measurement accuracy requirement related to NR_IIOT_URLLC_enh for a number of meetings. Based on the simulation assumption companies have provided their simulation results in RAN4#103 and RAN4#104. The results have been collected in [1] and [2].

In this paper we propose the measurement accuracy requirements for TUE-RX accuracy with TRS for FR1 and FR2 respectively.

Discussion

Table 1. Simulation results for TUE-RX accuracy with TRS, 4 samples, AWGN, TDD

From RAN4#103e and RAN4#104, companies have submitted simulation results for TUE-RX with 4 samples TRS in AWGN. The table 1 shows the collected result

|  |  | TUE-RX accuracy with TRS, 4 samples, AWGN, TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Accuracy (Tc) |  |  |  |  |  |  |  |  |  |
| SCS [KHz] | TRS bandwidth RB | Ericsson,R4-2212901, FR1 | Ericsson, R4-2212901, FR2 | Huawei, R4-2209237 | Huawei, R4-2209237 | Nokia, R4-2213775, FR1 | Nokia,R4-2213775, FR2 | vivo, R4-2208821, FR1 | vivo, R4-2208821, FR2 | Qualcomm, R4-2210226 | Qualcomm, R4-2212207 |
| 15 | 24 | 114.0 |  | 88.0 |  | 64.0 |  | 128.0 |  | 120.5 |  |
|  | 52 | 59.0 |  | 56.0 |  | 32.0 |  | 62.3 |  | 56.8 |  |
|  | 104 | 29.0 |  | 24.0 |  | 16.0 |  | 30.4 |  | 29.9 |  |
| 30 | 24 | 58.0 |  | 40.0 |  | 32.0 |  | 62.3 |  | 56.4 |  |
|  | 48 | 29.0 |  | 28.0 |  | 16.0 |  | 30.5 |  | 28.3 |  |
|  | 132 | 14.0 |  | 16.0 |  | 8.0 |  | 14.8 |  | 15.1 |  |
| 60 | 24 | 29.0 | 29.0 |  | 24.0 | 16.0 | 20.0 | 30.4 | 30.4 |  | 28.9 |
|  | 64 | 14.0 | 14.0 |  | 14.0 | 8.0 | 8.0 | 14.8 | 14.8 |  | 14.3 |
|  | 132 | 7.2 | 7.0 |  | 8.0 | 4.0 | 4.0 | 7.3 | 7.3 |  | 7.4 |
| 120 | 32 |  | 14.0 |  | 16.0 |  | 8.0 |  | 14.5 |  | 14.3 |
|  | 64 |  | 7.0 |  | 7.0 |  | 4.0 |  | 7.3 |  | 7.2 |
|  | 128 |  | 4.0 |  | 4.0 |  | 2.0 |  | 3.6 |  | 3.7 |

Based on table 1, we do average over companies’ results to compute the following table 2 and table 3

|  |  | TUE-RX accuracy with TRS, 4 samples, AWGN, TDD |
| --- | --- | --- |
|  |  | Accuracy (Tc) |
| SCS [KHz] | TRS bandwidth RB | Average |
| 15 | 24 | 103 |
|  | 52 | 53 |
|  | 104 | 26 |
| 30 | 24 | 50 |
|  | 48 | 26 |
|  | 132 | 14 |
| 60 | 24 | 25 |
|  | 64 | 12 |
|  | 132 | 6 |

Table 2 Average TUE-RX accuracy with TRS, 4 samples, AWGN, TDD, FR1

|  |  | TUE-RX accuracy with TRS, 4 samples, AWGN, TDD |
| --- | --- | --- |
|  |  | Accuracy (Tc) |
| SCS [KHz] | TRS bandwidth RB | Average |
| 60 | 24 | 26 |
|  | 64 | 13 |
|  | 132 | 7 |
| 120 | 32 | 13 |
|  | 64 | 7 |
|  | 128 | 3 |

Table 3 Average TUE-RX accuracy with TRS, 4 samples, AWGN, TDD, FR2

Apply averaging over companies’ TRS measurement accuracy simulation results.

Agreement:

- RAN4 will use same group delay calibration margin from PRS to accuracy for TRS based UE Rx-Tx

From RAN4#103e, it has the following agreement

Therefore, the accuracy results in Table 2&3 should be adjusted with current group delay defined in TS 38.133 – 10.1.25.2

Adopt the TRS measurement accuracy requirements in Table 2 and Table 3 addition with the group delay defined in TS 38.133 – 10.1.25.2.

Given by the above discussions, we have updated our results in out CR.

Conclusion

In this paper we propose the measurement accuracy requirements for TUE-RX accuracy with TRS for FR1 and FR2 respectively. We propose:

- Apply averaging over companies’ TRS measurement accuracy simulation results.

- Adopt the TRS measurement accuracy requirements in Table 2 and Table 3 addition with the group delay defined in TS 38.133 – 10.1.25.2.

We have provided CR capturing the proposal in [3]

- References

- R4-2210619, Simulation Results for NR_IIOT_URLLC_enh, Nokia, Nokia Shanghai Bell

- R4-2214339, Collection of simulation result for NR_IIOT_URLLC_enh, Nokia, Nokia Shanghai Bell

- R4-2216511, CR on UE Rx-Tx time difference measurement accuracy requirements for RTT-based PDC, Nokia, Nokia Shanghai Bell