3GPP TSG-RAN4 Meeting #104-bis-e	R4-2216516

# Electronic meeting, October 10-19, 2022

Source:			Ericsson

Title:			NTN enhancements – System parameters and bands definition

Agenda item:		6.22.1.1

Document for:		Approval

# Introduction

In the scope of Release 18, a new WI on NR NTN enhancements [1] has been approved by RAN. One of the objectives is to consider NR-NTN deployment in above 10 GHz bands.

This paper is early discussing the system parameters for NTN operation above 10GHz.

# Discussion

## Band definition

In our other contribution on regulatory aspects ([2]), we have made proposals on NTN band(s) definition for NTN deployment over 10 GHz based on the regulatory situation. Based on this contribution, considering other companies views and inputs, further discussion would be needed to finalize the exact definition of one or several exemplary NTN band(s) over 10GHz.

We just copy here after from [2] the proposals related to NTN band definition:

[2]-Proposal4: The spectrum allocated by ITU to MSS, i.e. 19.7-20.2 GHz (space-to-Earth) and 29.5-30.0 GHz (Earth-to-space), could be considered as a starting point for further discussion related to the definition of the NTN Ka-band.

[2]-Proposal5: Instead of specifying a unique NTN Ka-band band for all regions, RAN4 should specify regional/local band(s) depending on local regulation.

## Channel bandwidth

The targeted continuous spectrum for NTN operation above 10 GHz would most likely be larger than 200 MHz. We would then propose the following:

Proposal1: The specified channel bandwidths for NTN operation above 10 GHz should be at least 50, 100 and 200MHz. Other channel bandwidths are not precluded and would need further discussion.

## Sub-carrier spacing

For NTN operation above 10 GHz, sub-carrier spacing values specified for TN FR2 should be considered.

Proposal2: For NTN operation above 10 GHz, specify 60 and 120 kHz sub-carrier spacing.

## Spectrum utilization – transmission bandwidth configuration

Like it was agreed for NTN FR1, targeting an efficient use of the spectrum, the spectrum utilization for NTN operation above 10 GHz should be aligned with TN FR2 spectrum utilization.

Proposal3: For NTN operation above 10 GHz, specify at least the following transmission bandwidth configuration:

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz |
| --- | --- | --- | --- |
|  | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 |
| 120 | 32 | 66 | 132 |

## Channel raster and synchronization raster

The channel raster and synchronization raster intervals shall be finalized once the first band will be defined. This is so pending on the band discussion’s conclusion.

Nevertheless, RAN4 could already decide on the respective raster type and step size.

Proposal4: For NTN operation above 10 GHz, specify 60kHz and 120 kHz channel raster with step size of 1 and 2 respectively.

Proposal5: For NTN operation above 10 GHz, specify 120kHz (case D) and 240 kHz (case E) sync raster with step size of 1 and 2 respectively.

# Conclusion

In this contribution, we discussed the system parameters for NTN operation above 10 GHz. We made the following observation and proposals:

From [2]-Proposal4: The spectrum allocated by ITU to MSS, i.e. 19.7-20.2 GHz (space-to-Earth) and 29.5-30.0 GHz (Earth-to-space), could be considered as a starting point for further discussion related to the definition of the NTN Ka-band.

From [2]-Proposal5: Instead of specifying a unique NTN Ka-band band for all regions, RAN4 should specify regional/local band(s) depending on local regulation.

Other proposals:

Proposal1: The specified channel bandwidths for NTN operation above 10 GHz should be at least 50, 100 and 200MHz. Other channel bandwidths are not precluded and would need further discussion.

Proposal2: For NTN operation above 10 GHz, specify 60 and 120 kHz sub-carrier spacing.

Proposal3: For NTN operation above 10 GHz, specify at least the following transmission bandwidth configuration:

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz |
| --- | --- | --- | --- |
|  | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 |
| 120 | 32 | 66 | 132 |

Proposal4: For NTN operation above 10 GHz, specify 60kHz and 120 kHz channel raster with step size of 1 and 2 respectively.

Proposal5: For NTN operation above 10 GHz, specify 120kHz (case D) and 240 kHz (case E) sync raster with step size of 1 and 2 respectively.

# References

- RP-222654, NR NTN (Non-Terrestrial Networks) enhancements, Thales

- R4-2216515, NTN enhancements – Regulatory aspects, Ericsson