3GPP TSG-RAN4 Meeting #104-bis-e	R4-2216517

# Electronic meeting, October 10-19, 2022

Source:				Ericsson

Title:					NTN enhancements – Simulation assumptions for NTN FR2 co-existence study

Agenda item:		6.22.2

Document for:		Approval

# Introduction

This document analyses and proposes the simulation assumptions for the NTN coexistence study for the NTN enhancements frequency band in the scope of the NTN enhancements WI ([1]).

Remaining issues for further discussion are highlighted in yellow mark with square brackets.

# Discussion

## Deployment types

With high bands, 2 types of scenarios should be considered: urban macro and dense urban.

Usually, in RAN4, dense urban type of deployment consists in 2 adjacent operators deploying micro cells in clusters included in a typical macro cell, as shown in Figure 1.

Figure 1: Dense urban deployment

With NTN, such deployment scenario would not be applicable: NTN SAN beams would overlap TN cells. Further discussion would be needed on how to consider such deployment for coexistence simulations.

Proposal1: Further discuss the dense urban deployment scenario.

## NTN SAN type

According to the NTN enhancement WI ([1]), only GSO type of satellite should be considered in the scope of Rel-18:

It’s unclear to us (see [3]) which scenario should be considerd for the fixed VSAT type of NTN device. Still, fixed VSAT operation in the 19.7-20.2 GHz / 29.5-30.0 GHz frequency ranges which are allocated to MSS would make sense.

Proposal2: Consider only GSO type of satellite for RAN4 coexistence study with ESIM and consider GSO and NGSO types of satellite for fixed VSAT, pending on further clarification for fixed VSAT scenarios.

## NTN UE types

According to the NTN enhancement WI ([1]), several type of NTN UEs should be considered: fixed VSAT and mobile VSAT (or ESIM).

According to ITU, there are several type of ESIMs:

- Land ESIM or L-ESIM.

- Aeronautical ESIM or A-ESIM.

- Maritime ESIM or M-ESIM.

The Annex 2 of Resolution COM5/6 adopted in WRC-19 (XX) has specified:

- A minimum distance of 70 km for a M-ESIM to operate without the agreement of any administration.

- A maximum pfd below and above an altitude of 3 km for a A-ESIM to operate.

Observation1: ITU Resolution COM5/6 has specified a minimum distance (M-ESIM) and a maximum pfd (A-ESIM ) for those 2 types of ESIM to operate.

Based on this observation, we would propose to not consider M-ESIM in the RAN4 coexistence study, relying then on ITU Resolution COM5/6 and national regulation for those type of NTN UE. Also, we would propose to consider A-ESIM as second priority for the coexistence study.

Proposal3: RAN4 should only consider L-ESIM type of NTN device for its coexistence studies, A-ESIM might be considered as second priority.

## Scenario

NTN bands are FDD type while upper 5GHz TN bands are all TDD. This means that, in its coexistence studies,  RAN4 should investigate considered NTN UL vs both TN UL and DL and NTN DL vs both TN UL and DL.

It could be observe that, per today, there is no TN band specified around the 17.7-21.2 GHz frequency range. One might then suggest to not study NTN DL -TN UL/DL coexistence for that frequency range.

Nevertheless, we would like to remind that the RAN4 coexistence studies are the first input to specify BS/UE ACLR/ACS, OBUE and blocking requirements.

By ignoring the 17.7-21.2 GHz frequency range, RAN4 will not be able to specify the NTN SAN ACLR, the NTN UE ACS, the NTN SAN blocking requirement and the NTN OBUE/SEM requirements. As a consequence, the NTN Ka-band could only be specified as a FDD UL band only and not as FDD DL/UL band.

Also, by not considering the 17.7-21.2 GHz frequency range, the NTN Ka-band could not be considered as a reference band anymore: coexistence studies will most likley be needed again when specifying other NTN bands.

Proposal4: RAN4 shall consider both NTN UL and NTN DL cases when doing its coexistence studies.

## Reference frequencies

The NTN enhancements WI ([1]) has proposed to consider the Ka-band (from 17.7 to 21.2 GHz in space-to-Earth direction and 27.5 to 31.0 GHz in Earth-to-space direction) as reference for RAN4 specification.

Based on this information, we propose:

Proposal5: RAN4 should use 20 GHz as frequency reference for NTN DL and 30 GHz for NTN UL.

## Isolation area

For FR1 and coexistence simulations case 1 (TN DL agressor and NTN DL victim), an isolation area was assumed to minimize the impact on NTN UE ACS.

For the NTN Ka-band, we would propose to not consider any isolation area in a first step. From the initial simulation outcomes, after checking the potential impact on NTN UE ACS, such isolation area might then be reconsidered.

Proposal6: For coexistence simulations in the NTN Ka-band, RAN4 should not conisder any isolation area in a first step.

# Simulation assumptions

## Proposal

Based on the above observations and proposals, starting from the NN FR1 simulation assumptions, we would propose the following simulation assumption for the coexistence simulations in NTN Ka-band.

Proposal7: Adopt the assumptions in Annexes 1 and 2 for the coexistence simulations in NTN Ka-band.

Observation2: ESIM parameters would need further study.

# Conclusion

In this contribution, we analysed the NTN enhancements WI and the scenarios to consider for the coexistence study. We made the following observations and proposals:

Proposal1: Further discuss the dense urban deployment scenario.

Proposal2: Consider only GSO type of satellite for RAN4 coexistence study with ESIM and consider GSO and NGSO types of satellite for fixed VSAT, pending on further clarification for fixed VSAT scenarios.

Observation1: ITU Resolution COM5/6 has specified a minimum distance (M-ESIM) and a maximum pfd (A-ESIM ) for those 2 types of ESIM to operate.

Proposal3: RAN4 should only consider L-ESIM type of NTN device for its coexistence studies, A-ESIM might be considered as second priority.

Proposal4: RAN4 shall consider both NTN UL and NTN DL cases when doing its coexistence studies.

Proposal5: RAN4 should use 20 GHz as frequency reference for NTN DL and 30 GHz for NTN UL.

Proposal6: For coexistence simulations in the NTN Ka-band, RAN4 should not conisder any isolation area in a first step.

Proposal7: Adopt the assumptions in Annexes 1 and 2 for the coexistence simulations in NTN Ka-band.

Observation2: ESIM parameters would need further study.

# Reference

- RP-222564, NR NTN (Non-Terrestrial Networks) enhancements, Thales

- TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), Rel-16

- R4-2216515, NTN enhancements – Regulatory aspects, Ericsson

# Annex 1: Simulation assumptions

## Co-existence simulation scenarios

The proposed scenarios for coexistence study are in the following table 6.1-1.

Table 6.1-1 Scenarios for NTN-TN co-existence

|  |  |  |  | Satellite and UE types |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | GEO |  | LEO600 | LEO1200 |
| Freq. | Deployment | Aggressor | Victim | VSAT | ESIM-L | VSAT | VSAT |
| 20 GHz | Urban macro | NTN DL | TN UL | Yes | Yes | Yes | Yes |
|  |  | NTN DL | TN DL | Yes | Yes | Yes | Yes |
|  |  | TN UL | NTN DL | Yes | Yes | Yes | Yes |
|  |  | TN DL | NTN DL | Yes | Yes | Yes | Yes |
|  | Dense urban | NTN DL | TN UL | tbd | tbd | tbd | tbd |
|  |  | NTN DL | TN DL | tbd | tbd | tbd | tbd |
|  |  | TN UL | NTN DL | tbd | tbd | tbd | tbd |
|  |  | TN DL | NTN DL | tbd | tbd | tbd | tbd |
| 30 GHz | Urban macro | NTN UL | TN UL | Yes | Yes | Yes | Yes |
|  |  | NTN UL | TN DL | Yes | Yes | Yes | Yes |
|  |  | TN UL | NTN UL | Yes | Yes | Yes | Yes |
|  |  | TN DL | NTN UL | Yes | Yes | Yes | Yes |
|  | Dense urban | NTN UL | TN UL | tbd | tbd | tbd | tbd |
|  |  | NTN UL | TN DL | tbd | tbd | tbd | tbd |
|  |  | TN UL | NTN UL | tbd | tbd | tbd | tbd |
|  |  | TN DL | NTN UL | tbd | tbd | tbd | tbd |

The proposed frequency and bandwidth are listed as table 6.1-2.

Table 6.1-2.  Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- |
| TN Urban macro | 20/30 GHz | 100MHz | TDD | 1 |
| GEO | 20/30 GHz | 100 MHz | FDD | 1 |
| LEO | 20/30 GHz | 100 MHz | FDD | 1 |

## Network layout model

### Co-existence between NTN and TN

Cellular cell structure is considered for both NTN and TN network layout.

Coordination System

Referring to TR 38.811 Section 6.3 and Annex A, a 3D global coordinate system is considered (Earth-Centered Earth Fixed) for simulating NTN beams direction and location on the earth surface. It means the NTN beam location, TN randomly dropping location are generated with a set of three parameters (x,y,z).

Simulation Methodology

Following simulation steps can be used for NTN-TN co-existence study.

- Generate aggressor and victim networks.

- NTN central beam is at satellite nadir, surrounded with 6 co-frequency beams. NTN FRFs higher than 1 need to be considered. Assume one NTN aggressor as default.

- Deployment of TN network (19 cells with wraparound) refers to Annex 2.

- UE associations

- TN UE are generated randomly inside the TN network, make sure enough TN UEs are associated to each TN sectors based on coupling loss.

- Deployment of NTN UE refers to Annex 2.

- Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

- Throughput is computed in the victim systems without considering ACI as below:

- ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

For TN-NTN SINR calculation, the satellite receiver off angle should be considered in the satellite receiver gain calculation when calculating SINR. Note that such angle is not considered in TR 38.821 section 6.1.3 equations. Thus, those equations should be used for SINR calculation.

- Throughput is computed considering ACI as below:

-	ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

- RF parameters are determined based on the degradation cause by ACI as below:

-	LossACI=1-ThputACIThputSINGLE.

## Simulation parameters

### NTN parameters

Satellite parameters

Two sets of satellite parameters are shown in Table 2.3-2 and Table 2.3-3 according to TR 38.821.

The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 6.3.1-1 NRB configuration per Bandwidth size and SCS

| Configuration FR2 Ka-band | NRB (100MHz BW) |
| --- | --- |
| SCS 120 kHz | 66 |

Table 6.3.1-2 Set-1 satellite parameters for co-existence study

| Satellite orbit |  |  | GEO | LEO-1200 |  | LEO-600 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km | 1200 km |  | 600 km |  |
| Payload characteristics for DL transmissions |  |  |  |  |  |  |  |
| Satellite EIRP density |  | 20GHz | 40 dBW/MHz | 10 dBW/MHz |  | 4 dBW/MHz |  |
| Satellite max TX power in dBm | BW (MHz) |  | 100 |  | 100 |  | 100 |
|  | SCS 120kHz |  | tbd |  | tbd |  | tbd |
| Satellite Tx max Gain |  |  | 58.5 dBi | 38.5 dBi |  | 38.5 dBi |  |
| Channel bandwidth |  |  | 100MHz | 100MHz |  | 100MHz |  |
| 3dB beamwidth or HPBW (Half-Power Bandwidth) of main central beam |  |  | 0.1765 deg | 1.7647 deg |  | 1.7647 deg |  |
| ABS (Adjacent Beam Spacing) of adjacent beams from the central beam |  |  | 0.3474 deg | 3.8206 deg |  | 3.8206 deg |  |
| Satellite beam diameter |  |  | 110 km | 40 km |  | 20 m |  |
| Payload characteristics for UL transmissions |  |  |  |  |  |  |  |
| G/T |  | 30 GHz | 28 dB K-1 | 13 dB K-1 |  | 13 dB K-1 |  |
| Satellite Rx max Gain |  |  | 58.5 dBi | 38.5 dBi |  | 38.5 dBi |  |

Table 6.3.1-3 Other parameters for NTN

| Parameters | NTN | Remark |
| --- | --- | --- |
| Carrier frequency | 20/30 GHz |  |
| The number of active UE (UL) | 9 UEs and nRBs per UE for GEO and LEO1 |  |
| The number of active UE (DL) | 1 | Same with TN |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | See Session 2.6.2 |  |
| NTN satellite Noise figure in dB | See Table 2.3-3-1 |  |
| Handover margin | 3dB |  |
| Note 1: UEs are equally splitted inside the channel bandwidth into ACIR 3 regions. Scheduled PRB position for UE1 per satellite beam should be also fully aligned to simulate the worst case for co-channel interference and this is also aligned with full buffer case. |  |  |

Table 6.3.1-4 NTN satellite Noise figure in dB

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 28 | 13 | 13 |
| G_Rx (dBi) | 58.5 | 38.5 | 38.5 |
| NF (dB) | 5.9 | 0.9 | 0.9 |

UE parameters

UE parameters are shown in Table 3.4-5.

The parameters specified in TR 38.821 are still TBD. We captured in the “ESIM (ITU)” column the parameters that have been used for some ITU studies on L-ESIM.

Table 6.3.1-5 UE characteristics for system level simulations

| Characteristics | Fixed VSAT | ESIM (TR 38.821) | ESIM (ITU) |
| --- | --- | --- | --- |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) | Antenna pattern: S.580-6 |
| Polarisation | circular | Linear: +/-45°X-pol |  |
| Rx Antenna gain | 39.7 dBi | TBD dBi per element | 45 dBi |
| Antenna temperature | 150 K | TBD K |  |
| Noise figure | 1.2 dB | TBD dB |  |
| Tx transmit power | 2 W (33 dBm) | [TBD W (TBD dBm)] | Transmit power density（dBW/Hz）-46.3 |
| Tx antenna gain | 43.2 dBi | TBD dBi per element | 45 dBi |

### TN parameters

Table 6.3.2-1 Simulation assumptions of TN based on NR

|  | NR |
| --- | --- |
| Carrier frequency in GHz | 20 / 30 |
| Size of each nominal channel BW in MHz | 100 |
| Transmission bandwidth in MHz | N/A |
| Environment | Deployment scenario related, check Table 3.4.2-2. |
| Network layout | 19-sites 57 sectors with wrap-around |
| Inter-site distance in meter | Deployment scenario related, see Table 3.4.2-2 |
| System loading and activity | See Annex 2 |
| Network location | See Annex 2 |
| DL subcarrier spacing | 120kHz |
| UL | OFDMA |
| DL power control | No |
| UL power control | 36.942 |
| Frequency reuse | 1 |
| Number of scheduled UE per cell (DL) | 1 |
| Number of scheduled UE per cell (UL) | 3 |
| UE antenna height in meter | Same as 3D-UMi in TR 36.873 |
| UE TX power in dBm | -40 to 23 |
| UE antenna gain in dBi | 0 |
| Building penetration loss | In pathloss model, TR 38.901 |
| Cell selection margin in dB | 3 |
| BS-MS min distance in meters | 35 |
| BS noise figure in dB | 5 |
| UE noise figure in dB | 9 |
| BS-UE path-loss model | TR 38.901 |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901. |
| Shadowing correlation | Inter-cell 0.5 Intra-cell 1 |
| Link-level performance model | See Section 2.9 Throughput-SINR mapping |
| UE distribution | Uniform |
| Evaluation metrics | See Section 2.9 Throughput loss criteria |

Table 6.3.2-2 Deployment-related parameters of TN (20/30 GHz)

|  | Urban Macro | Remarks |
| --- | --- | --- |
| ISD in meters | 200 |  |
| BS Antenna height in meters | 25 |  |
| UE Outdoor/indoor | 100% Outdoor |  |
| UE height in meter | 1.5 |  |

## Antenna and beam forming pattern modelling

### Satellite and UE Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling of satellite could be referred to section 6.4.1 in TR 38.811.

Satellite antenna pattern

The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered

1

where J1(x) is the Bessel function of the first kind and first order with argument x, a is the radius of the antenna's circular aperture, k = 2f/c is the wave number, f is the frequency of operation, c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam. Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency.

The antenna patterns for LEO 600km, 1200km and GEO are show in Figure 2.4.1-1 and 2.4.1-2.

Figure 6.4.1-1:  Antenna pattern for LEO 600KM and 1200KM (4.4127 deg for 3dB beamwidth)

Figure 6.4.1-2 Antenna pattern for antenna aperture of GEO (0.4011 deg for 3dB beamwidth]

Satellite and UE beam forming pattern

The following table is agreed for the beam layout definition for a single satellite simulation in S-Band.

Table 6.4.1-1: Beam layout definition for single satellite simulation

| Beam layout definition | Baseline: Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. Only the 3dB beam width parameters should be used. The beam diameter and beam spacing values can be computed directly from the 3 dB beam width assumptions and should be considered as informative. |  |  |
| --- | --- | --- | --- |
| Number of beams | Baseline: 7-beam layout (i.e. 6 co-frequency beams surrounding the central beam) |  |  |
| UV plane illustration (extracted from [19]) |  |  |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after: The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |  |  |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern: ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2) with ABS [degree]=180/pi x ABS[rad] and with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern. |  |  |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam center is considered at nadir point Case 2: 45° for GEO and LEO Interested companies can bring analysis and results for other values. |  |  |
|  | Option 1: FRF=1 | Option 2: FRF=3 | Option 3: FRF=2 |
| Polarization re-use | Disable |  |  |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |  |  |
| UE distribution | The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers. |  |  |
| UE configuration | Ka-band: (VSAT) ESIM-L |  |  |
| UE orientation | VSAT and ESIM: Ideal Tracking serving beam; |  |  |
| UE attachment | RSRP |  |  |
| NOTE 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. NOTE 2: For the calibration purpose, the ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude). |  |  |  |

### TN BS and UE antenna and beam forming pattern modelling

BS antenna

AAS antenna parameters for 20-30 GHz are listed in table 6.4.2-1.

Table 6.4.2-1 AAS antenna parameters for 20-30GHz

|  | Macro urban |
| --- | --- |
| Base Station Antenna Characteristics |  |
| Antenna pattern | TR 38.921 |
| Element gain GE,max (dBi) | 5.5 |
| Horizontal 3dB /vertical 3dB 3 dB beam width of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio Am (dB) | 30 for both H/V |
| Side lobe suppression SLAv (dB) | 30 |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 elements |
| Horizontal/Vertical radiating element spacing | dh = 0.5 l dv = 0.5 l |
| Array Ohmic loss LE (dB) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 22 |
| Base station maximum coverage angle in the horizontal plane (degrees) jrange | -60 to 60 |
| Base station vertical coverage range (degrees) qrange | 90-105 |
| Mechanical downtilt (degrees) | 10 |

UE antenna

UE antennas parameters are listed in table 6.4.2-2

Table 6.4.2-2: UE antenna element pattern

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| (Mg, Ng, M, N, P) | (1, 1, 2, 2, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |
| NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. |  |

## ACIR model

The following ACIR model is agreed to be used to derive ACIR values for co-ex study between NTN and TN.

The number of RBs in Table 2.5-1 should be updated and aligned with the agreed number of UL UE in NTN and TN assumptions.

Figure 2.5-1. ACIR model

Table 2.5-1. Uplink ACIR value

| Frequency offset between aggressor (105 RBs) and victim (105 RBs) | ACIR value |
| --- | --- |
| 0-[34] RBs | 30 + X |
| [35-69] RBs | 43 + X |
| >[69] RBs | 43+ X |

## Propagation model

Editor’s note: RAN4 confirms using the agreed NTN propagation model from TR 38.811 as starting point for NTN coexistence work with respect to both calibration and simulation purpose. New proposals for NTN propagation model optimizations are potentially acceptable, but only if they would change the result to an extent where that work is justified.

### Propagation model between NTN and UE

Propagation model between NTN and UE could be referred to section 6.6 in TR 38.811.

On top of this, based on TR 38.821 (XX), the following additional losses should be considered.

#### Atmospheric absorption

Figure 6 of ITU-R P.676 (XX) shows the corresponding zenith attenuation  for frequencies between 1 and 350 GHz. For an elevation angle α, the corresponding attenuation  is given by:

#### Rain and cloud attenuation

For system-level simulations, the baseline is to consider clear sky conditions only. No rain and cloud attenuation should then be considered.

#### Scintillation

The ionospheric scintillation corresponds to rapid fluctuations of the received signal amplitude and phase. Ionosphere propagation shall only be considered for frequencies below 6 GHz. This is not applicable then for coexistence system simulations for the NTN Ka-band.

The tropospheric scintillation corresponds to rapid fluctuations of the received signal amplitude and phase. Tropospheric propagation shall only be considered for frequencies above 6 GHz.

Table 6.6.6.2.1-1 from TR 38.821 could be used as starting point.

### Propagation model between TN BS and UE

Propagation model between TN BS and UE could be referred to section 7.4 in TR 38.901.

### Propagation model between NTN BS and TN BS

Propagation model between NTN BS and TN BS should reference to TS 38.811 which is used for DL-UL cross link interference for S band.

## Transmission power control model

### TN UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 could be applied for TN with following parameters.

Where, Pmax = 23dBm, Rmin = TBD dB, CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y,

where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

### NTN UL TPC

For calibration purpose, reuse the same TN TPC for NTN with SNR target 15dB.

For the coexistence study, the same TPC model of TN for NTN UL scenarios is adopted but needs to revise CLx-ile to align with UE UL power control parameters used in TR38.821. The CLx-ile value should be adapted for rural scenario.

### DL TPC

For downlink scenario, no power control scheme is applied.

## Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

Where,

RX_PWR is the received power

TX_PWR is the transmitted power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

## Performance metric

For NR,

The average throughput loss and 5%-idle throughput loss should be less than 5%.

For NTN,

The average throughput loss and 5%-ile throughput loss should be less than 5%.

## Throughput ~ SNR mapping

Adopt Section 5.2.7 of TR 38.803 as the SINR-Throughput performance metrics, but a, SNIRMIN, and SNIRMAX need to be further studied and decided for NR NTN.

[Parameters below can be a starting point for α, SNIRMIN, and SNIRMAX, but other options are not precluded.

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | [0.6] | [0.4] | Represents implementation losses |
| SNIRMIN, dB | [-10] | [-10] | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | [30] | [22] | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

It’s infeasible to achieve 30dB/22dB DL/UL maximum SNIR for NR NTN, so the following parameters need to be further studied and RAN4 need to check them with RAN1.]

# Annex 2. Consideration of Network and UE deployment

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped at the edge of TN clusters | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | All active TN clusters which has the NTN UE(s) at its edge. |

| 2 | TN with NTN | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped at the edge of TN clusters | Consider an active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |

| 3 | TN with NTN | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: NTN UEs dropped outside or at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values) NTN UE: NTN UEs dropped outside or at the edge of TN clusters | TN clusters randomly placed in this NTN beam |  |

| 4 | TN with NTN | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs dropped at the edge of TN clusters | TN randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge. |
| --- | --- | --- | --- | --- | --- | --- |

| 5 | TN with NTN | NTN UL | TN DL | NTN cell: Nadir point NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge. |

| 6 | TN with NTN | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped outside or at the edge of TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |

| 7 | TN with NTN | NTN DL | TN UL | NTN cell: Nadir point NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge. |

| 8 | TN with NTN | TN UL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped outside or at the edge of TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |