3GPP TSG-RAN4 Meeting #104-bis-e	R4-2216526

# Electronic meeting, October 10-19, 2022

Source:			Ericsson

Title:			NTN FR1 open issues

Agenda item:		4.2.2.1

Document for:		Approval

# Introduction

The NTN WI ([1]) has been approved in RAN#88e meeting to specify requirements for the support of NTN. It has been revised in last RAN#94-e meeting [2].

In last RAN4#104-e meeting, few open issues were remaining from the discussion occurred on the need for ΔfOBUE parameter, without any conclusion.

This contribution is mainly further discussing this aspect.

# Discussion

## Background

In last RAN4#104-e meeting, a discussion on the definition and even the need of ΔfOBUE parameter was triggered. And the following options were listed in the agreed Way Forward ([5]):

- Sub-topic #1-2-5: ΔfOBUE for SAN

  - Option 1: Keep the current definition with ΔfOBUE for SAN in TS 38.108. Specify ΔfOBUE for SAN.

  - Option 2: Do not keep the current definition with ΔfOBUE for SAN in TS 38.108. Do not specify ΔfOBUE for SAN.

  - Option 3: Keep ΔfOBUE for SAN and correct it with the following values:

Table 6.6.1-1: Maximum offset of OBUE outside the downlink operating band

| SAN type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| SAN type 1-H | FDL,high – FDL,low < 100 MHz | 2*BWChannel 2.5×BWChannel |

Table 9.7.1-1: Maximum offset ΔfOBUE outside the downlink operating band

| SAN type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| SAN type 1-O | FDL,high – FDL,low < 100 MHz | 10 2.5×BWChannel |

  - Option 4 (combination of Option 3 with the values from Option 2): Keep ΔfOBUE for SAN and correct it with the following values:

Table 6.6.1-1: Maximum offset of OBUE outside the downlink operating band

| SAN type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| SAN type 1-H | FDL,high – FDL,low < 100 MHz | 2*BWChannel 2×BWChannel |

Table 9.7.1-1: Maximum offset ΔfOBUE outside the downlink operating band

| SAN type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| SAN type 1-O | FDL,high – FDL,low < 100 MHz | 10 2×BWChannel |

- Sub-topic #1-2-6 (New): Follow ITU-R definition of OoB (Out-of-Band) domain emissions

  - Proposal: Any emission outside the necessary bandwidth which occurs in the frequency range separated from the assigned frequency of the emission by less than 250% of the necessary bandwidth of the emission will generally be considered an emission in the OoB domain.

Note 1: In RAN4#103-e it has been already agreed to follow ITU recommendation SM.1541-6 (Unwanted emissions in the out-of-band domain) to define SAN OBUE requirements (please see R4-2210633), but it seems that the interpretations are different.

## Background

ΔfOBUE is defined as the maximum offset of the operating band unwanted emissions mask from the downlink operating band edge. And it’s used to clearly identify the boundary limit between OBUE and spurious domain.

The ΔfOBUE parameter was introduced with NR to deal with the large bands which have been allocated to 5G. The previous definition (10 MHz) of the spurious limit boundary had to be revised, considering such large bands, and keeping the existing definition for the legacy or smaller bands.

With NTN, instead of specifying new OBUE requirements, RAN4 agreed to reuse the mask specified in SM.1541-6. To be consistent, our proposal would then be to align with SM.1541-6 boundary of the spurious domain.

Proposal1: Align NTN SAN spurious domain boundary with SM.1541-6 definition.

This ITU Recommendation defines the boundary of the spurious domain according to the following:

For the purpose of this Recommendation all emissions, including intermodulation products, conversion products and parasitic emissions, which fall at frequencies separated from the centre frequency of the emission by 250% or more of the necessary bandwidth of the emission will generally be considered as emissions in the spurious domain.

The spurious domain starts then at 250% of the necessary bandwidth, from the centre frequency. This means that option 3 is not an acceptable proposal.

Observation2: Option 3 of the WF [5] is not aligned with SM.1541-6 spurious domain boundary definition.

Regarding the remaining options, option 4 is actually aligning OTA definition of ΔfOBUE with the conducted definition of ΔfOBUE in TS 38.108. The 10 MHz value for ΔfOBUE is most likely due to a copy/paste of TS 38.104 for n1/n65 band.

The options 1, 2, 4 and 6 are actually equivalent. The main difference should only be the definition or not of ΔfOBUE.

Considering ΔfOBUE parameter definition was mainly motivated to better support the NR large bands and in the context of the definition of OBUE requirement, we would propose the following:

Proposal2: Do not define ΔfOBUE and clarify the out of band / spurious domain boundary in TS 38.108.

We have written 2 companion CRs ([6] and [7]) according to this proposal.

# Conclusion

In this contribution, we discussed the remaining SAN RF open issue from last RAN4#104-e meeting made the following observations and proposals:

Proposal1: Align NTN SAN spurious domain boundary with SM.1541-6 definition.

Observation2: Option 3 of the WF [5] is not aligned with SM.1541-6 spurious domain boundary definition.

Proposal2: Do not define ΔfOBUE and clarify the out of band / spurious domain boundary in TS 38.108.

# References

- RP-201256, Solutions for NR to support non-terrestrial networks (NTN), Thales

- RP-213691, Solutions for NR to support non-terrestrial networks (NTN), Thales

- TS 38.108, NR, Satellite Access Node radio transmission and reception, v17.0.0

- ITU-R SM.1541-6 Unwanted emissions in the out-of-band domain

- R4-2214370, Way Forward on NTN Solutions SAN RF Maintenance, Thales

- R4-22xxxx, CR to TS 38.108 - Updates related to DfOBUE - conducted clauses, Ericsson

- R4-22xxxx, CR to TS 38.108 - Updates related to DfOBUE - OTA clauses, Ericsson