3GPP TSG-RAN WG4 Meeting # 104bis-e                	R4-2216546

Electronic, 10th - 19th Oct, 2022

Agenda item:	7.5.2

Source: 	ZTE Corporation

Title: 	Further discussion on system parameters

Document for:	Approval

Introduction

In RAN#95e meeting, the work item [RP-221556] on NB-IoT/eMTC core & performance requirements for NTN was approved as one of Rel-18 RAN4 package. In the last RAN4 meeting, we have reached good progress on the system parameters for IoT over NTN, however there are still some open issues left for further discussions. In this contribution, we want to share some further inputs on these issues.

Issue 1-3:    Channel spacing

- Agreement:

- Reuse TN specification 36.101/36.104 norminal channel spacing for Cat-M1 and NB-IoT

  - Further discuss the frequency gap for NB-IoT over NTN deployment

- Removal of in-band/guard band item.

Issue 1-4-2:    M_DL for standalone NB-IoT

Further discussion below options in next meeting:

- Option 1: To follow the same MDL values as TN FDD as baseline

- Option 2: Update the M_DL value to improve spectral efficiency (R4-2213243)

- Option 3: Others are not precluded.

Issue 1-4-3:    EARFCN:

Further discussion in next meeting:

- Start from the highest EARFCN  262143 value and descending manner for B256 and B255

Discussion on system parameters for IoT over NTN

## 2.1. Channel spacing

In the last RAN4 meeting, there were some initial discussions on frequency ranges between NB-IoT carriers and it’s still kept open for further discussions. Some companies argued that 200kHz freq offset is only applicable to the edge of RF bandwidth as shown in the following Figure 1, however other companies argued that 200kHz freq gap is not only applicable to the edge of RF bandwidth and but also applicable for adjacent NB-IoT carriers based on the current NB-IoT BS ACLR/UEM requirement which is somehow also aligned with the practical deployment.

Figure 1. the illustration of standalone NB-IoT carrier placement within the RF bandwidth

Indeed based on the coexistence study back to Rel-13 NB-IoT, 200kHz frequency gap is proposed for 100kHz guard band between IoT and GSM carriers from the coexistence perspective. When defining the UEM requirement for standalone NB-IoT BS, it was agreed that the existing MSR UEM requirement specified in clause 6.6.2.2 of TS 37.104 for BC2 operation is reused for all operating bands using the same Foffset, RAT value (200kHz)for NB-IoT as for GSM. In other words, frequency offset 200kHz should be applicable for the edge of RF bandwidth and this should be also applicable for channel arrangement of adjacent NB-IoT carriers from coexistence perspective.

Observation 1: for the legacy TN NB-IoT spec, 100kHz frequency gap between adjacent NB-IoT carriers within the RF bandwidth, it should be applicable.

For Rel-18 IoT over NTN, 100kHz guard band might be questionable as we mentioned in the last RAN4 meeting. Based on the initial simulation results between IoT over NTN and TN NR [xxx] as indicated in the last RAN4 meeting, SAN DL ACLR requirement of IoT over NTN should not be the bottleneck anymore, the current ACLR requirement of NR over NTN in Rel-17 should be sufficient. However there are lots of other coexistence scenarios not analyzed yet (e..g IoT over NTN coexisting with IoT over NTN, IoT over NTN coexisting with TN standalone NB-IoT/GSM etc), therefore we propose to postpone the discussion of guard band between IoT over NTN carriers until we get more concrete simulation results to validate its necessity.

Proposal 1: to postpone the discussion of guard band between IoT over NTN carriers until we get more concrete simulation results to validate its necessity.

## 2.2. M_DL for standalone NB-IoT non-anchor carriers

During the last RAN4 meeting, there were some candidate proposals to improve the spectral utilization between adjacent anchor carrier and non-anchor carriers by adjusting the M_DL value of non-anchor carriers to ensure its orthogonality between anchor carriers and non-anchor carriers. We could understand its motivation, however we don’t see its urgency especially considering this was not agreed for TN NB-IoT system yet. Since we seek to reuse the existing TN NB-IoT design as much as possible, at least we propose not to consider it in this Release and still follow the same M_DL value for TN FDD band.

Proposal 2: to follow the same M_DL value of TN FDD band in Rel-18.

## 2.3. EARFCN

During the last RAN4 meeting, we have reached the consensus for channel raster and principle to specify the EARFCN for IoT over NTN. Based on the agreement reached so far, we provided the exact EARFCN value as following for each operating band. In addition, as we agreed that only 1.4MHz is applicable for eMTC over NTN in this release, therefore the parameters related with other channel bandwidth should be removed.

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| 256 | 2170 | 243544 | 243544 –<1>-– 243843 | 1980 | 261844 | 261844 –<1>- 262143 |
| 255 | 1525 | 243204 | 243204 –<1>-– 243543 | 1626.5 | 261504 | 261504 –<1>- 261843 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7, 15, 25, 50, 75 and 100 channel numbers at the lower operating band edge and the last 6, 14, 24, 49, 74 and 99 channel numbers at the upper operating band edge shall not be used for channel bandwidths of 1.4, 3, 5, 10, 15 and 20 MHz respectively. |  |  |  |  |  |  |

Proposal 3: to agree the EARFCN value in the above table for Band 256 and B255.

Conclusions

In this contribution, we want to share some further consideration on system parameters for IoT over NTN and observations and proposals are made as following:

Observation 1: for the legacy TN NB-IoT spec, 100kHz frequency gap between adjacent NB-IoT carrier within the RF bandwidth, it should be still applicable.

Proposal 1: to postpone the discussion of guard band between IoT over NTN carriers until we get more concrete simulation results to validate its necessity.

Proposal 2: to follow the same M_DL value of TN FDD band in Rel-18.

Proposal 3: to agree the EARFCN value in the above table for Band 256 and B255.

References

- RP-221556, WID on NB-IoT/eMTC core & performance requirements for NTN.

- R4-2214380,WF on system parameter for IoT over NTN, Ericsson, approved.

- R4-160915, NB-IoT BS RF – UEM MSR mask, Ericsson, Noted

- R4-2213695, system parameters and coexistence verification for IoT over NTN, ZTE, Noted