3GPP TSG-RAN WG4 Meeting # 104bis-e                	R4-2216547

Electronic, 10th - 19th Oct, 2022

Agenda item:	7.5.3

Source: 	ZTE Corporation

Title: 	Further discussion on simulation assumptions and evaluation results for IoT over NTN

Document for:	Discussion

Introduction

In RAN#95e meeting, the work item [RP-221556] on NB-IoT/eMTC core & performance requirements for NTN was approved as one of Rel-18 RAN4 package.During the last RAN4 meeting, we have reached good progress for the simulation assumption for IoT over NTN coexistence study, however there are still some remaining issues left for further discussions. In this contribution, we want to share some further consideration on these issues.

- The number of active UE for IoT over NTN;

- ACLR/ACS model for IoT over NTN coex

- How to verify that we can reuse the existing requirements (etc. UTRA ACLR requirements);

Discussion on system parameters for IoT over NTN

## 2.1. The number of active UE for IoT over NTN

During the last RAN4 meeting, there were some initial discussions on the number of active UE in IoT over NTN study, however the conclusion are not reached yet. Indeed back to Rel-13 TN NB-IoT coexistence study, it was agreed to have the full buffer scheduling, therefore the number of active scheduled UE should be 3 for multiple tone, 12 for single tone 15kHz and 48 for single tone 3.75kHz. All these NB-IoT UEs are randomly dropped per sectors. For IoT over NTN, if we continue to reuse the full buffer scheduling assumption and drop all IoT UEs next to or at the edge of the TN cluster, this should be ideally worst case, however from our understanding, its occurring probability should be quite low considering the coverage per satellite beam is quite large compared with TN cluster. As the compromised assumption, we propose to emulate 3 users for multiple tone, 9 users for single tone 15kHz and 36 users for single tone 3.75kHz.

Proposal 1: to emulate 3 users for multiple tone, 9 users for single tone 15kHz and 36 users for single tone 3.75kHz.

## 2.2. ACLR/ACS model for IoT over NTN coex

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |

During the last RAN4 meeting, there were some discussions on ACLR/ACS modelling for IoT NTN coexistence study, e.g. reusing the effective ACLR/ACS modelling with scaling factor inherited from TR 36.802. However for Case 4 and Case 5, company might argue that flat ACLR modelling with scaling factor is too pessimistic assumption which will result in the unnecessary high ACLR requirement for IoT over NTN. The main reason to include the scaling factor for scenario 4 and scenario 5 is due to the ACLR definition for standalone NB-IoT at the beginning. The original assumption for standalone NB-IoT UE ACLR requirement is for 1st adjacent PRB, then if the bandwidth of victim system is wider than 1 PRB e.g. 5MHz or 10MHz, then scaling factor should be considered in the simulation study. However if we go directly to define the ACLR requirement for adjacent NR carrier/E-UTRA/UTRA carriers, then we don’t need to consider the scaling factor anymore since ACLR requirement for this scenario has been changed. Based on the above consideration and for the simplification of IoT over NTN coexistence study, for scenario 4 and scenario 5, we directly define the ACLR requirement over adjacent NR/E-UTRA carriers.

Proposal 2: for Case 4 and Case 5, to directly define the ACLR requirement over the adjacent NR/E-UTRA carriers.

Initial simulation results for IoT over NTN coexistence study

- The assumption for aggressor and victim combinations are listed in Table 6.1-2 within the appendix.

  - . Simulation results of Case 1

- In case 1, TN DL would cause the interference to IoT over NTN DL. The effective ACLR of TN BS and ACS of NTN UE is according to the section 6.3 of TR 36.802[3] in appendix of 6.2.4. Isolation distance of 1500m for urban macro scenario is introduced according to appendix 6.2.1. The simulation results are shown as the following table.

| Case 1 | ACS [dB] | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Urban_macro TN_AAS_DL | 5% SINR loss | 27.08 | 22.09 | 17.12 | 12.24 | 7.66 | 3.95 | 1.76 | 0.69 |
|  | 50%SINR loss | 17.80 | 12.97 | 8.44 | 4.69 | 2.27 | 1.06 | 0.50 | 0.23 |
|  | 95% SINR loss | 11.11 | 7.13 | 4.29 | 2.40 | 1.23 | 0.59 | 0.27 | 0.12 |
| GEO_Urban_macro TN_non_AAS_DL | 5% SINR loss | 27.09 | 22.10 | 17.18 | 12.27 | 7.80 | 4.14 | 1.92 | 0.75 |
|  | 50%SINR loss | 19.02 | 14.14 | 9.49 | 5.46 | 2.62 | 1.13 | 0.49 | 0.20 |
|  | 95% SINR loss | 14.22 | 9.67 | 5.90 | 3.23 | 1.61 | 0.71 | 0.31 | 0.14 |

  - . Simulation results of Case 2

- In case 2, TN UL would cause the interference to NTN UL when NTN SAN receives UL transmission from IoT over NTN UE. The effective ACLR of TN UE is according to the section 6.3 of TR 36.802[3]. The simulation results are to be provided later.

| Case 2: Multiple tone | ACS [dB] | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 55 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Urban_macro TN_AAS_UL | 5% SINR loss |  |  |  |  |  |  |  |  |
|  | 50%SINR loss |  |  |  |  |  |  |  |  |
|  | 95% SINR loss |  |  |  |  |  |  |  |  |
| GEO_Urban_macro TN_non_AAS_UL | 5% SINR loss |  |  |  |  |  |  |  |  |
|  | 50%SINR loss |  |  |  |  |  |  |  |  |
|  | 95% SINR loss |  |  |  |  |  |  |  |  |

| Case 2: Single tone 15kHz | ACS [dB] | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 55 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Urban_macro TN_AAS_UL | 5% SINR loss |  |  |  |  |  |  |  |  |
|  | 50%SINR loss |  |  |  |  |  |  |  |  |
|  | 95% SINR loss |  |  |  |  |  |  |  |  |
| GEO_Urban_macro TN_non_AAS_UL | 5% SINR loss |  |  |  |  |  |  |  |  |
|  | 50%SINR loss |  |  |  |  |  |  |  |  |
|  | 95% SINR loss |  |  |  |  |  |  |  |  |

| Case 2: Single tone 3.75kHz | ACS [dB] | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 55 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Urban_macro TN_AAS_UL | 5% SINR loss |  |  |  |  |  |  |  |  |
|  | 50%SINR loss |  |  |  |  |  |  |  |  |
|  | 95% SINR loss |  |  |  |  |  |  |  |  |
| GEO_Urban_macro TN_non_AAS_UL | 5% SINR loss |  |  |  |  |  |  |  |  |
|  | 50%SINR loss |  |  |  |  |  |  |  |  |
|  | 95% SINR loss |  |  |  |  |  |  |  |  |

  - .Simulation result of Case 3

- In case 3, NTN DL transmission may interfere TN UE when TN UE receives DL transmission from TN BS. The effective ACLR of NTN SAN should be provided according to the section 6.3 of TR 36.802[3]. The simulation result is shown as the following table.

| Case 3 | ACLR [dB] | 0 | 5 | 10 | 15 | 20 | 25 | 30 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Rural TN_AAS_DL | Average throughput loss | 22.88 | 10.10 | 3.80 | 1.25 | 0.41 | 0.13 | 0.05 | 0.01 |
|  | Cell edge through loss | 31.78 | 13.49 | 4.91 | 1.49 | 0.50 | 0.16 | 0.05 | 0.02 |
| GEO_Rural TN_non_AAS_DL | Average throughput loss | 31.09 | 13.88 | 5.25 | 1.80 | 0.60 | 0.15 | 0.06 | 0.01 |
|  | Cell edge through loss | 53.77 | 26.93 | 10.44 | 3.41 | 0.89 | 0.29 | 0.11 | 0.05 |
| LEO600KM_Rural TN_AAS_DL | Average throughput loss | 65.07 | 43.80 | 24.39 | 10.97 | 4.16 | 1.39 | 0.44 | 0.15 |
|  | Cell edge through loss | 84.35 | 62.85 | 34.10 | 13.73 | 4.78 | 1.44 | 0.52 | 0.14 |
| LEO600KM_Rural TN_non_AAS_DL | Average throughput loss | 81.39 | 59.52 | 33.93 | 15.03 | 5.71 | 1.83 | 0.50 | 0.17 |
|  | Cell edge through loss | 100.00 | 100.00 | 57.48 | 29.79 | 11.85 | 4.23 | 1.39 | 0.42 |

  - .Simulation results of Case 4

- In case 4, IoT over NTN UL may interfere TN BS when TN BS receives UL transmission from TN UE. The effective ACLR of IoT over NTN UE should be provided according to the section 6.3 of TR 36.802[3]. The simulation result is shown as the following table.

| - Case 4 : Multiple tone | ACLR [dB] | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Urban_macro TN_AAS_UL | Average throughput loss | 16.58 | 9.26 | 4.97 | 2.55 | 1.27 | 0.67 | 0.29 | 0.17 |
|  | Cell edge through loss | 61.39 | 38.78 | 21.82 | 9.97 | 6.77 | 3.06 | 1.03 | 1.01 |
| GEO_Urban_macro TN_non_AAS_UL | Average throughput loss | 12.02 | 6.37 | 3.40 | 1.97 | 1.41 | 0.59 | 0.20 | 0.09 |
|  | Cell edge through loss | 33.26 | 16.48 | 7.16 | 2.61 | 0.89 | 0.00 | 0.00 | 0.00 |

| Case 4: Single tone 15kHz | ACLR [dB] | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Urban_macro TN_AAS_UL | Average throughput loss | 18.37 | 9.94 | 5.27 | 2.74 | 1.53 | 0.77 | 0.43 | 0.26 |
|  | Cell edge through loss | 60.88 | 38.01 | 22.55 | 11.53 | 5.74 | 3.47 | 1.20 | 0.46 |
| GEO_Urban_macro TN_non_AAS_UL | Average throughput loss | 12.96 | 6.57 | 3.77 | 2.18 | 1.21 | 0.64 | 0.26 | 0.08 |
|  | Cell edge through loss | 43.64 | 20.68 | 8.49 | 2.98 | 0.91 | 0.07 | 0.01 | 0.00 |

| Case 4: Single tone 3.75kHz | ACLR [dB] | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Urban_macro TN_AAS_UL | Average throughput loss | 14.94 | 8.24 | 4.37 | 2.33 | 1.19 | 0.60 | 0.35 | 0.20 |
|  | Cell edge through loss | 50.64 | 30.43 | 17.46 | 7.83 | 4.03 | 2.36 | 1.32 | 0.47 |
| GEO_Urban_macro TN_non_AAS_UL | Average throughput loss | 14.38 | 8.14 | 4.63 | 2.44 | 1.07 | 0.40 | 0.12 | 0.06 |
|  | Cell edge through loss | 42.93 | 19.95 | 7.09 | 1.62 | 0.37 | 0.07 | 0.01 | 0.00 |

  - .Simulation results of Case 5

- IoT over NTN UL interfering TN DL, if IoT over NTN UE is close to TN UE e.g. 1m distance separation, then coupling loss is around 40dBc, the in-band blocking level of TN UE in adjacent channel would be -21dBm which is very stringent and demanding requirements than existing requirements ACS and in-band blocking requirements for FR1 UE, therefore from this perspective, IoT over NTN service cannot been provided together with TN service in the same geographical area especially within close frequency spectrum. If IoT over NTN UL could be done in OOBB range of TN service, then its kind of cross-link interference could be still acceptable or NTN service is provided with certain geographical area separation.

  - .Simulation results of Case 6

- In case 6, TN DL may interfere IoT over NTN SAN when IoT over NTN SAN receives UL transmission from IoT over NTN UE. The effective ACLR of TN BS should be provided according to the section 6.3 of TR 36.802[3].  The simulation results are to be provided later.

| Case 6 | ACS [dB] | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_UL_Rural TN_AAS_DL | Average throughput loss |  |  |  |  |  |  |  |  |
|  | Cell edge through loss |  |  |  |  |  |  |  |  |
| GEO_UL_Rural TN_non_AAS_DL | Average throughput loss |  |  |  |  |  |  |  |  |
|  | Cell edge through loss |  |  |  |  |  |  |  |  |
| GEO_Urban_macro TN_AAS_UL | Average throughput loss |  |  |  |  |  |  |  |  |
|  | Cell edge through loss |  |  |  |  |  |  |  |  |
| GEO_Urban_macro TN_non_AAS_UL | Average throughput loss |  |  |  |  |  |  |  |  |
|  | Cell edge through loss |  |  |  |  |  |  |  |  |
| LEO600KM_Rural TN_AAS_UL | Average throughput loss |  |  |  |  |  |  |  |  |
|  | Cell edge through loss |  |  |  |  |  |  |  |  |
| LEO600KM_Rural TN_non_AAS_UL | Average throughput loss |  |  |  |  |  |  |  |  |
|  | Cell edge through loss |  |  |  |  |  |  |  |  |
| LEO600KM Urban_macro TN_AAS_UL | Average throughput loss |  |  |  |  |  |  |  |  |
|  | Cell edge through loss |  |  |  |  |  |  |  |  |
| LEO600KM Urban_macro TN_non_AAS_UL | Average throughput loss |  |  |  |  |  |  |  |  |
|  | Cell edge through loss |  |  |  |  |  |  |  |  |
| LEO1200KM_Rural TN_AAS_UL | Average throughput loss |  |  |  |  |  |  |  |  |
|  | Cell edge through loss |  |  |  |  |  |  |  |  |
| LEO1200KM_Rural TN_non_AAS_UL | Average throughput loss |  |  |  |  |  |  |  |  |
|  | Cell edge through loss |  |  |  |  |  |  |  |  |
| LEO1200KM Urban_macro TN_AAS_UL | Average throughput loss |  |  |  |  |  |  |  |  |
|  | Cell edge through loss |  |  |  |  |  |  |  |  |
| LEO1200KM Urban_macro TN_non_AAS_UL | Average throughput loss |  |  |  |  |  |  |  |  |
|  | Cell edge through loss |  |  |  |  |  |  |  |  |

Conclusions

In this contribution, we want to share some further consideration on simulation results for IoT over NTN and TN with the effective ACLR for NTN and TN coexistence study. Proposals are made as following:

Proposal 1: to emulate 3 users for multiple tone, 9 users for single tone 15kHz and 36 users for single tone 3.75kHz.

Proposal 2: for Case 4 and Case 5, to directly define the ACLR requirement over the adjacent NR/E-UTRA carriers.

References

- RP-221556, WID on NB-IoT/eMTC core & performance requirements for NTN.

- R1-2200042, LS on DL synchronization enhancements for IoT NTN, RAN1 (Jan-22)

- TR 36.802,Technical Report for BS and UE radio transmission and reception.

- TR 38.863

- R4-2214381,WF on coexistence study for IoT over NTN, MTK, approved.

Appendix for simulation

- 6.1  Co-existence simulation scenario

The aggressor and victim combinations are listed in Table 6.1-2.

Table 6.1-2: Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |

- The proposed frequency and bandwidth for co-existence study are listed in following table.

Table 6.1-4: Proposed frequency and bandwidth for co-existence study for standalone NB-IoT over NTN

|  | Frequency | Bandwidth | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- |
| TN Rural | 2 GHz | 20MHz | FDD, TDD | 1 |
| TN Urban macro | 2 GHz | 20MHz | FDD, TDD | 1 |
| GEO | 2 GHz | 200kHz | FDD | 1, 31 |
| LEO | 2 GHz | 200kHz | FDD | 1, 31 |
| NOTE: Only FRF=1 has been used in co-existence studies for simplification. |  |  |  |  |

6.2	Co-existence simulation assumption

6.2.1	Network layout model

6.2.1.1	Co-existence between NTN satellite and TN

Cellular cell structure is considered for both NTN satellite and TN network layout.

Referring to TR 38.811[5] Section 6.3 and Annex A, a 3D global coordinate system is considered (Earth-Centred Earth Fixed) for simulating NTN satellite beams direction and location on the earth surface. It means the NTN satellite beam location, TN randomly dropping location are generated with a set of three parameters (x,y,z).

Deployment of NTN satellite and TN cells and UEs for co-existence study is listed in Table 6.2.1.1-1.

NOTE:	The NTN UE(s) shall be dropped at the edge of the “central 19 TN cells (cluster)”. For Case 1, an Isolation distance of 1500m as 2*ISD is considered to reflect the NTN-TN selection algorithm at the border, assuming TN connection will always be prioritized over NTN.

As defined in Figure 6.2.1.1-1, isolation distance is the distance between the blue-dotted line which represents TN cell boarder and the red line. No UEs deployed in the isolation region is assumed to reduce the calculation complexity.

Figure 6.2.1.1-1: Isolation distance for Case 1

Table 6.2.1.1-1:	Network and UE deployment

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN satellite | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped at the edge of TN clusters | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | All active TN clusters which has the NTN UE(s) at its edge. |
| 2 | TN with NTN satellite | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped at the edge of TN clusters | Consider an active rate of 20% for Rural and Urban of TN. | All active TN cells in central NTN beam |
| 3 | TN with NTN satellite | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: NTN UEs dropped outside or at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All in central NTN beam |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values) NTN UE: NTN UEs dropped outside or at the edge of TN clusters | TN clusters randomly placed in this NTN beam |  |
| 4 | TN with NTN satellite | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs dropped at the edge of TN clusters | TN randomly placed in this NTN beam | Option 1: All active TN clusters which has the NTN UE(s) at its edge. Option 2: Only the TN sectors which have NTN UE(s) at their edges. Option 1 is the baseline and it is not precluded companies can follow Option 2 to bring results |
| 5 | TN with NTN satellite | NTN UL | TN DL | NTN cell: Nadir point NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which has the NTN UE(s) at its edge |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which has the NTN UE(s) at its edge. |
| 6 | TN with NTN satellite | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN cell with satellite at low elevation to be further investigated. NTN UE: NTN UEs dropped outside or at the edge of TN clusters | Consider the active rate of 20% for Rural and Urban of TN. The Urban TN deployment for GEO in Case 6 is a mixture of urban and rural TN deployment. A representative percentage of urban TN could be considered for further study. | All active TN cells in central NTN beam |

6.2.2 System parameters

6.2.2.1	Satellite parameters

Set1 of satellite parameters are listed in Table 6.2.2.1-2 according to TR 38.821/ 36.763[6].

The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 6.2.2.1-1a: Transmission bandwidth configuration NRB, Ntone 15kHz and Ntone 3.75kHz in NB-IoT channel bandwidth

| NB-IoT | Standalone | In-band | Guard Band |
| --- | --- | --- | --- |
| Channel bandwidth BWChannel [kHz] | 200 | NA | NA |
| Transmission bandwidth configuration NRB | 1 | NA | NA |
| Transmission bandwidth configuration Ntone 15kHz | 12 | NA | NA |
| Transmission bandwidth configuration Ntone 3.75kHz | 48 | NA | NA |

Table 6.2.2.1-2: Set-1 satellite parameters for co-existence study

| Satellite orbit |  |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |  |
| Satellite EIRP density |  | 2GHz | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite max TX power in dBm | BW (MHz) |  | 0.2(NB-IoT) | 0.2(NB-IoT) | 0.2(NB-IoT) |
|  | SCS 15kHz |  | 31.01 | 33.01 | 27.01 |
|  | SCS 3.75khz |  | 31.01 | 33.01 | 27.01 |
| Satellite Tx max Gain |  |  | 51 dBi | 30 dBi | 30 dBi |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| ABS (Adjacent Beam Spacing) of adjacent beams from the central beam |  |  | 0.3474 deg | 3.8206 deg | 3.8206 deg |
| Satellite beam diameter |  |  | 250 km | 90 km | 50 km |
| Payload characteristics for UL transmissions |  |  |  |  |  |
| G/T |  | 2 GHz | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  |  | 51 dBi | 30 dBi | 30 dBi |

Table 6.2.2.1-4: Other parameters for NTN satellite

| Parameters | NB-IoT over NTN satellite | Remark |
| --- | --- | --- |
| Carrier frequency | 2GHz |  |
| The number of active UE (UL) for GEO and LEO ( Assumes 75% of NTN UE at TN for single-tone ) | 9 UEs for 15kHz SCS with single-tone 36 UEs for 3.75kHz SCS with single-tone | NOTE |
| The number of active UE (DL) | 1 | Same with TN |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | See Session 6.2.6 |  |
| NTN satellite Noise Figure in dB | See Table 6.2.2.1-5 |  |
| Handover margin | 3dB |  |
| NOTE: Scheduled PRB position for UE1 per satellite beam should be also fully aligned to simulate the worst case for co-channel interference and this is also aligned with full buffer case. |  |  |

Table 6.2.2.1-5: NTN satellite Noise Figure in dB

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 19 | 1.1 | 1.1 |
| G_Rx (dBi) | 51 | 30 | 30 |
| NF (dB) | 7.4 | 4.3 | 4.3 |

Note: there were 5 sets of

The NTN BS Noise Figure could be calculated from the gain-to-noise-temperature ration (G/T) and the Rx antenna gain, using following formula:

NF =10 Log10 (T_sysT_0)	, with T_0 = 290

And

GT=GRx- 10 Log10 (T_sys)

For the S-band, with Set 1 parameters, the BS NF are given in Table 2.

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 19 | 1.1 | 1.1 |
| G_Rx (dBi) | 51 | 30 | 30 |
| NF (dB) | 7.4 | 4.3 | 4.3 |

6.2.2.4	NTN UE parameters for standalone NB-IoT over NTN

NTN UE parameters are given in Table 6.2.2.4-1.

Table 6.2.2.4-1: NTN UE characteristics for system level simulations for standalone NB-IoT

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | (1, 1, 1) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Rx Antenna gain | 0 dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 9 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | 0 dBi per element |

6.2.2.5	TN parameters

TN parameters for co-existence study are given in Table 6.2.2.4-1, 6.2.2.4-2 and 6.2.2.4-3.

Table 6.2.2.4-1: Simulation assumptions of TN respectively based on NB-IoT and NR

|  | NB-IoT standalone | NR |
| --- | --- | --- |
| Carrier frequency in GHz | 2 | 2 |
| Size of each nominal channel BW in MHz | 0.2 | 20 |
| Transmission bandwidth in MHz | 0.18 | N/A |
| Environment | Urban macro Rural | Deployment scenario related, see Table 6.2.2.4-2. |
| Network layout | 19-sites [57 sectors] with wrap-around | 19-sites 57 sectors with wrap-around |
| Inter-site distance in meter | 500 for 2GHz band for UMA TBD For Rural | Deployment scenario related, see Table 6.2.2.4-2 |
| System loading and activity | Full buffer 100% | See Table 6.2.1.1-1 |
| Network location | TBD | See Table 6.2.1.1-1 |
| DL subcarrier spacing | 15kHz | 15kHz |
| UL | See RP-152284[7] | OFDMA |
| DL power control | No | No |
| UL power control | TR 36.942[8] section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB | TR 36.942[8] |
| Frequency reuse | 1 | 1 |
| Number of scheduled UE per cell (DL) | 1 | 1 |
| Number of scheduled UE per cell (UL) | 3 for multi-tone (60kHz per UE), 12 for 15kHz single-tone, 48 for 3.75kHz single-tone | 3 |
| UE antenna height in meter | 1.5 | 1.5m |
| UE TX power in dBm | -40 to 23 | -40 to 23 |
| UE antenna gain in dBi | 0 | 0 |
| Building penetration loss | TR 45.820[9] Annex D.1 | In pathloss model, TR 38.901[10] |
| Cell selection margin in dB | 3 | 3 |
| BS-MS min distance in meters | 35 | 35 |
| BS noise figure in dB | 5 | 5 |
| UE noise figure in dB | 9 | 9 |
| BS-UE path-loss model | TR 36.942[8] macro urban | TR 38.901[10] |
| Standard deviation of BS-UE log-normal shadow fading in dB | 10 | Deployment scenario related, referring to TR 38.901[10] |
| Shadowing correlation | Inter-cell 0.5 Intra-cell 1 | Inter-cell 0.5 Intra-cell 1 |
| Link-level performance model |  | See Section 6.2.8 Throughtput-SINR mapping |
| UE distribution |  | Uniform |
| Evaluation metrics | SINR vs ACS (as victim) | See Section 6.2.8 Throughtput or SNR loss criteria |

Table 6.2.2.4-2: Deployment-related parameters of TN (2 GHz)

|  | Urban Macro | Rural Macro | Remarks |
| --- | --- | --- | --- |
| ISD in meters | 750 | 7500 | ITU-R Report M.2292[11] |
| BS Antenna height in meters | 25 | 30 |  |
| UE Outdoor/indoor | 100% Outdoor |  |  |
| UE height in meter | 1.5 | 1.5 | RP-200559 [12] 3GPP LS to ITU-R WP5D and ITU-R WP5D [IMT_Parameters] [13] |

Table 6.2.2.4-3: ACLR/ACS for TN (2GHz)

|  |  | NR | NB-IOT |
| --- | --- | --- | --- |
| BS | ACLR | 45 dB | 40 dB |
|  | ACS | 46 dB | 46 dB |
| UE | ACLR | 30dB (ACLR1) 43dB (ACLR2) | 37 |
|  | ACS | 33 | 28 |

6.2.3	Antenna and beamforming pattern modelling

6.2.3.1	Satellite and UE Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling of satellite could be referred to section 6.4.1 in TR 38.811[5].

The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered.

1                for θ=0

4J1kasinθkasinθ2        for 0<θ≤90°

where:

-	J1(x) is the Bessel function of the first kind and first order with argument;

-	x, a is the radius of the antenna's circular aperture;

-	k = 2f/c is the wave number;

-	f is the frequency of operation;

-	c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam.

Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency.

The antenna patterns for LEO 600km, 1200km and GEO are shown in Figure 6.2.3.1-1 and 6.2.3.1-2.

Figure 6.2.3.1-1:  Antenna pattern for LEO 600KM and 1200KM (4.4127 deg for 3dB beamwidth)

Figure 6.2.3.1-2: Antenna pattern for antenna aperture of GEO (0.4011 deg for 3dB beamwidth)

The beam layout definition for a single satellite simulation in S-Band is defined in Table 6.2.3.1-1.

Table 6.2.3.1-1: Beam layout definition for single satellite simulation

| Beam layout definition | Baseline: Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. Only the 3dB beam width parameters should be used. The beam diameter and beam spacing values can be computed directly from the 3 dB beam width assumptions and should be considered as informative. |  |  |
| --- | --- | --- | --- |
| Number of beams | Baseline: 7-beam layout (i.e. 6 co-frequency beams surrounding the central beam) |  |  |
| UV plane illustration (extracted from [19]) |  |  |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after: The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |  |  |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern: ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2) with ABS [degree]=180/pi x ABS[rad] and with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern. |  |  |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam center is considered at nadir point Case 2: 45° for GEO and LEO |  |  |
|  | Option 1: FRF=1 | Option 2: FRF=3 | Option 3: FRF=2 |
| Polarization re-use | Option 1: Disable Option 2: Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |  |  |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |  |  |
| UE distribution | The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers. |  |  |
| UE configuration | S-band: Handheld |  |  |
| UE orientation | Handheld: Random |  |  |
| UE attachment | RSRP |  |  |
| NOTE 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. NOTE 2: For the calibration purpose, the ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude). |  |  |  |

6.2.3.2	TN BS and UE antenna and beam forming pattern modelling

For AAS antenna, it refers to the pattern in TR 38.921 [14].

Table 6.2.3.2-1: AAS antenna parameters for 2GHz

|  |  | Rural | Macro urban |
| --- | --- | --- | --- |
| 1 | Base Station Antenna Characteristics |  |  |
| 1.1 | Antenna pattern | TR 38.921[14] |  |
| 1.2 | Element gain (dBi) (Note 2) | 7.1 | 6.4 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 54º for V | 90º for H 65º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º | Linear ±45º |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 8 × 8 elements | 8 × 8 elements |
| 1.7 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.9 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 |
| 1.9 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 25 | 25 |
| 1.10 | Base station maximum coverage angle in the horizontal plane (degrees) | 120 | 120 |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-120 |
| 1.12 | Mechanical downtilt (degrees) | 3 | 10 |

For non-AAS antenna, the parameters in Table 6.2.3.2-1 are used for 2GHz BS antenna pattern in the NTN system simulation.

Table 6.2.3.2-1: FR1 BS Non-AAS antenna pattern for 2GHz

| Parameter for BS | Values |  |
| --- | --- | --- |
| Antenna vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=10°,SLAV=20dB |  |
| Antenna horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=20dB |  |
| Combining method for 3D antenna pattern (dB) | A“θ",φ”=-min-AE,Vθ"+AE,Hφ”, Am |  |
| Maximum directional gain of an antenna GE,max | 17 dBi |  |
| Conducted power | 46 dBm |  |
| Mechanical downtilt in degrees | Rural | 3 |
|  | Urban | 10 |

Only Non-AAS antenna can be used for NB-IoT.

For UE antennas, an omni-directional radiation pattern with antenna gain 0dBi is assumed.

6.2.4	ACIR model

ACLR modelling for TN and NTN co-existence study referring to clause 5.1.1.4.1 and 5.1.1.4.2 in TR 36.942[8] is used as baseline. The number of RBs refers to Table 6.2.2.1-4 and Table 6.2.2.4-1 respectively.

- In this system level simulation, effective ACLR is calculated based on section 6.3 of TR 36.802 and the main content is copied as bellow:

Additional information on ACLR and ACS is shown as follows,

•	Definition and assumption

•	LTE ACLR and ACS are defined for its own bandwidth size

•	NB-IoT ACLR and ACS are defined for its own bandwidth size

•	Attenuations are flat in the adjacent 10 MHz

•	ACLR and ACS are the inputs to the simulations

•	ACLR_e is the effective ACLR in the interference calculation

•	ACS_e is the effective ACS in the interference calculation

•	Interference from LTE to NB-IoT

•	ACLR_e = ACLR – 10*log10((180kHz/(# of IoT users))/9MHz)

•	ACS_e = ACS [pessimistic assumption]

•	Interference from NB-IoT to LTE

•	ACLR_e = ACLR – 10*log10((9MHz/(# of LTE users))/180kHz) [pessimistic assumption]

•	ACS_e = ACS

•	The effective ACLR (ACLR_e) and effective ACS (ACS_e) shall be applied in the coexistence simulation for the calculation of victim system interference

Figure 6.3-1 Illustration of flat assumption of ACLR and ACS

6.2.5	Propagation model

6.2.5.1	Propagation model between NTN SAN and UE

Propagation model between NTN SAN and UE should reference to section 6.6 in TR 38.811[5].

6.2.5.2	Propagation model between TN BS and UE

Propagation model between TN BS and UE should reference to section 7.4 in TR 38.901[10].

6.2.5.3	Propagation model between NTN SAN and TN BS

Propagation model between NTN SAN and TN BS should reference to TS 38.811[5] which is used for DL-UL cross link interference for S band.

6.2.6	Transmission power control model

6.2.6.1	TN UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942[8] is applied for TN with following parameters.

where:

-	Pmax = 23dBm,

-	Rmin = TBD dB,

-	CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y, where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

6.2.6.2	NTN UL TPC

For the coexistence study, the same TPC model of TN for NTN UL scenarios is adopted to align with UE UL power control parameters used in TR 38.821[6]. Follow 36.942 section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB.

6.2.6.3	NTN DL TPC

For downlink scenario, no power control scheme is applied.

6.2.7	Received powe-r model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

-	RX_PWR is the received power

-	TX_PWR is the transmitted power

-	G_TX is the transmitter antenna gain (directional array gain)

-	G_RX is the receiver antenna gain (directional array gain).

6.2.8	Performance metric

For NR, the average throughput loss and 5%-ile throughput loss should be less than 5%.

For NB-IoT over NTN, the average throughput loss and 5%-ile throughput loss should be less than 5%.

6.2.9	Throughput ~ SNR mapping for NR

Adopt Section 5.2.7 of TR 38.803[20] as the SINR-Throughput performance metrics.

Reference:

[10.3.1] R4-167721 SINR vs throughput mapping Huawei, Hisilicon

[10.3.1] R4-167722 TP for TR 38.803 SINR vs throughput mapping Huawei, Hisilicon

6.3	Co-existence simulation methodology

6.3.1	Simulation procedure

Adopt following simulation steps.

-	Step 1: Generate aggressor and victim networks.

-	NTN central beam is at satellite nadir, surrounded with 6 co-frequency beams. NTN FRFs higher than 1 need to be considered. Assume one NTN aggressor as default.

-	Deployment of TN network (19 cells with wraparound) refers to Table 6.2.1.1-1

-	Step2: UE associations

-	TN UE are generated randomly inside the TN network, make sure enough TN UEs are associated to each TN sectors based on coupling loss.

-	Deployment of NTN UE refers to Table 6.2.1.1-1.

-	Step 3: Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS/SAN-UE. This is done for both victim and aggressor networks.

-	Step 4: Throughput is computed in the victim systems without considering ACI as below:

ThputNO ACIbpshz=fSINRICI=fSN+IICI,

where:			IICI is the inter-cell interference.

For TN-NTN SINR calculation, the satellite receiver off angle should be considered in the satellite receiver gain calculation when calculating SINR. Note that such angle is not considered in TR 38.821[6] section 6.1.3 equations. Thus those equations should be used for SINR calculation.

-	Step 5: Throughput is computed considering ACI as below:

ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI,

where:			IACI is the adjacent channel interference.

-	Step 6: RF parameters are determined based on the degradation cause by ACI as below:

LossACI=1-ThputACIThputSINGLE

To simplify the simulation of interference from TN to NTN UL in Case 2 and 6, following method can be used. Consider the active TN cells from central NTN beam for the ACI evaluation from TN to NTN UL. The scaling factor is to be discussed and determined if any in next meeting. There is a view that simplifying such coexistence simulation work for Case 2 may even not be required.

-	Step 1: to drop NTN UE per beamprint randomly;

-	Step 2: to drop N clusters consisting of 57 sectors per beamprint randomly:

-	Step 3: to calculate the total ACI per beam to NTN UL by following scaling factor:

where:			active_TN = active_factor*round (the area per beam/the area of 57 sectors)

active_factor = 20% (or lower, particularly for urban scenarios)

-	Step 4: to calculate the total ACI from all beams (e.g. M=7 ) for NTN: