3GPP TSG-RAN WG4 Meeting#104bis-e                               R4-2216550

E-meeting,10th Oct – 19th Oct,2022

Agenda item:	7.4.4

Source: 	ZTE Corporation

Title: 	Further discussion on BS RF requirements for LTE based broadcast

Document for:	Approval

Introduction

LTE based broadcast [RP-211144] has been approved and completed in Rel-17, however due to the limited TU budget, its RAN4 work is postponed to Rel-18 in RAN4. In the last RAN4 meeting, there were some initial discussion on BS RF requirements. In this contribution, we want to share some further views from BS RF perspective for LTE based broadcast.

- For MBMS-dedicated cells:

  - Specify a PMCH allocation of 6/7/8 MHz and corresponding MBSFN reference signals [RAN1].

    - Specify corresponding signaling [RAN2, RAN3]

NOTE 0:	This part 1 REL-17 WID for RAN1/2/3 will enable a later RAN4 REL-18 part 2 WID:
		-	to specify band(s), within the portion of UHF spectrum allocated to broadcast 				systems (~470 - ~694/698 MHz, depending on the region), that support the 				channelization available to broadcasting operators
		in a REL-independent way starting from REL-17.

NOTE 1:	This work item shall not affect any functionality of unicast LTE, it targets only MBMS-			dedicated cells.

NOTE 2: 	The later RAN4 WI (part 2) shall assume that coexistence among different systems in the 		portion of the UHF band allocated to broadcast (~470 - ~694/698 MHz) is ensured through		coordination, in line with regional and national regulation.
NOTE 3:	New (dedicated) channel filters (e.g. 6/7/8 MHz filters) are not considered for the UE.

Discussion

## 2.1. BS type and BS class definition

First of all, from our understanding, the BS type 1-C is more applicable for frequency range between 470MHz and 698MHz. In the previous reply LS to ITU-R WP5D, it was also confirmed that BS type 1-H/BS type 1-O is not applicable for operating bands less then 1710MHz since it’s not easy to implement active antenna array due to its large size.

- There is no beamforming assumed for BS below 1.7 GHz. Parameters are therefore applicable for AAS BS in the range 1710 to 4990 MHz.

Based on the above considerations, only the BS type 1-C should be applicable for LTE based broadcast.

Proposal 1: to define the BS type 1-C for LTE based broadcast.

In addition, we need more discussions for BS class for LTE based broadcast. As we know that, there are multiple BS class (e.g. MPMT, HPHT-1, HPHT-2 and LPLT) introduced for CAS reception in Rel-16 RAN1 [5]. The existing Wide area BS class could be only applicable for LPLT, however for other BS types, we need new definitions for them. The main reason to define new BS class for HPHT-1, HPHT-2 and MPMT is due to its higher transmission power and higher antenna height which are different from the legacy TN wide area BS assumption. If we reuse the existing wide area BS definition for HPHT and MPMT, then UE maximum input power could exceed a lot. The maximum input power for the legacy UE is derived by 45dBm-70dBm MCL. If the transmission power is increased to 70dBm in the HPHT scenario, then maximum UE input power would be 0dB which blocked the UE receiver somehow if we reuse the legacy TN UE assumption for LTE based broadcast receiver.

Proposal 2: to reuse the Wide area BS class for LPLT and define new BS class for HPHT and MPMT.

Table 1: Scenarios for system-level evaluations [5]

| Parameter | MPMT | HPHT-1 | HPHT-2 | LPLT |
| --- | --- | --- | --- | --- |
| ISD | 50km | 125km | 173.2km | 15km |
| BS Power | 60dBm | 70dBm | 70dBm | 46 dBm |
| BS effective antenna height | 100m | 300m | 300m | 35m |
| CPs/Numerologies | CAS/SC-PTM: 4.7 us, 16.6us PMCH: 15kHz/7.5kHz/1.25kHz as baseline. |  |  |  |
| Carrier frequency | 700MHz |  |  |  |
| Channel BW | 10MHz as baseline |  |  |  |
| BS antenna gain | 10.5dBi | 13dBi | 13dBi | 15dBi |
| BS antenna pattern | 1Tx: rooftop and non-rooftop 2Tx: non-rooftop (FFS for rooftop) | 1Tx: rooftop and non-rooftop 2Tx: non-rooftop (FFS for rooftop) | 1Tx: rooftop and non-rooftop 2Tx: non-rooftop (FFS for rooftop) | 3 sector antenna pattern Each sector has a horizontal and vertical pattern as defined in: [4], Table A.1.1: 3GPP Case 1 and Case 3 (Macro Cell) 2Tx for non-rooftop (FFS for rooftop) |
| Cellular Layout | Hexagonal grid, 61 cell sites, 1 sector per site | Hexagonal grid, 61 cell sites, 1 sector per site | Hexagonal grid, 61 cell sites, 1 sector per site | Hexagonal grid, 61 cell sites,3 sector per site |
| Tx EVM | [8%] |  |  | 8% |

The definition for wide area BS are extracted as following with both MCL definition and mini distance separation.

Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum coupling loss equal to 70 dB. The Wide Area Base Station class has the same requirements as the base station for General Purpose application in Release 8.

For BS type 1-O and 2-O, BS classes are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

## 2.2. RF requirements

In the following section, we want to share some further views on BS RF requirements for LTE based broadcast and the details could be found as following.

Table 1. Further analysis how to specify BS RF requirements for LTE based broadcast

| Tx requirements | Applicability notes |
| --- | --- |
| BS output power | up to the declaration and no need to define any requirements |
| Output power dynamics |  |
| RE power control dynamic range | N/A, we don’t expect to have the RE power control to alleviate the co-channel interference by this approach |
| Total power dynamic range | N/A, we don’t expect the vision/sound signal of broadcast would be transmitted in the partial PRBs. |
| Transmit ON/OFF power | N/A |
| Transmitted signal quality |  |
| Frequency error | to reuse the same requirement defined in TS 36.104 |
| Modulation quality | The supported the modulation order up to 256QAM and reuse the EVM requirement in TS 36.104. 1024QAM should not be applicable for LTE based broadcast. |
| Time alignment error | FFS, this depend whether MIMO scheme is supposed to be supported for LTE based broadcast. If not, then this requirement is also not needed. |
| Unwanted emissions |  |
| Occupied bandwidth | to reuse the same requirement defined in TS36.104 which is following ITU-R Recommendation SM.328 |
| Adjacent Channel Leakage Power Ratio | this depends on the outcome of coexistence study. |
| Operating band unwanted emissions | this depends on the outcome of coexistence study and ITU/regional regulation which was captured in [4]. Further consideration how to accommodate these requirement into the TS 36.104 |
| Transmitter spurious emissions | FFS Coexistence requirement for the protection of other TN BS receiver should be considered. The legacy value is not applicable due to its aggressor transmitter power and potential MCL assumption. |
| Transmitter intermodulation | Not applicable for MPMT and HPHT since it’s not expected to have surrounding interfering gNB. FFS for LPLT. If the practical deployment, there are also no surrounding interfering gNB next to the victim BS, then this requirement should be also not applicable. |

Proposal 3: to start the discussion on BS RF requirement as proposed in table 1 for LTE based broadcast

## 2.3. How to capture the requirement for the legacy TN BS coexisting with LTE based broadcast

In addition, during the last RAN4 meeting, there were some initial discussions for the legacy TN BS coexistence requirement with LTE based broadcast. As extracted in the following section, both reference approach and explicitly list the regulatory requirement into the spec are feasible way in 3GPP, however from our understanding, from the benefit of spec implementation and further updates in case of other changes in regulatory requirements, it’s better to use the reference approach to avoid the further unnecessary maintenance work.

Proposal 4: to use the reference approach instead of explicitly list the regulatory requirement in the spec.

6.6.4.2.5.1	Limits in FCC Title 47

In addition to the requirements in clauses 6.6.4.2.1, 6.6.4.2.2, 6.6.4.2.3 and 6.6.4.2.4, the BS may have to comply with the applicable emission limits established by FCC Title 47 [8], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer.

6.6.4.2.5.2	Protection of DTT

In certain regions the following requirement may apply for protection of DTT. For BS type 1-C or BS type 1-H operating in Band n20, the level of emissions in the band 470-790 MHz, measured in an 8 MHz filter bandwidth on centre frequencies Ffilter according to table 6.6.4.2.5.2-1, a basic limits PEM,N is declared by the manufacturer. This requirement applies in the frequency range 470-790 MHz even though part of the range falls in the spurious domain.

Table 6.6.4.2.5.2-1: Declared emissions basic limit for protection of DTT

| Filter centre frequency, Ffilter | Measurement bandwidth | Declared emission basic limit (dBm) |
| --- | --- | --- |
| Ffilter = 8*N + 306 (MHz); 21 ≤ N ≤ 60 | 8 MHz | PEM,N |

Note:	The regional requirement is defined in terms of EIRP (effective isotropic radiated power), which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the BS needed to verify compliance with the regional requirement. Compliance with the regional requirement can be determined using the method outlined in TS 36.104 [13], annex F.

Conclusions

In this contribution, we want to share some further views on BS RF requirements for LTE based broadcast.and proposals are made as following:

Proposal 1: to define the BS type 1-C for LTE based broadcast.

Proposal 2: to reuse the Wide area BS class for LPLT and define new BS class for HPHT and MPMT.

Proposal 3: to start the discussion on BS RF requirement as proposed in table 1 for LTE based broadcast

Proposal 4: to use the reference approach instead of explicitly list the regulatory requirement in the spec.

References

- RP-220518, “Revised WID: New bands and bandwidth allocation for LTE based 5G terrestrial broadcast - part 2”, EBU, Qualcomm.

- TR36.775, US 600 MHz Band for LTE, v15.0.0

- R4-2214384, WF on List of expected changes to TR 36.104 due to introduction of LTE based 5G terrestrial broadcast band(s), Nokia

- R4-2214385, WF on Reuse of existing regulatory agreements, Huawei, ZTE, Approved.

- TR 36.776