3GPP TSG-RAN WG4 Meeting#104bis-e                               R4-2216554

E-meeting,10th Oct – 19th Oct,2022

Agenda item:	6.24.3

Source: 	ZTE Corporation

Title: 	Discussion on RRM requirements for NCR-MT in Rel-18

Document for:	Approval

Introduction

In the latest RAN#97e meeting, the work item [RP-222673] on NR network-controlled repeater was approved after the completion of study item phase led by RAN1. In this contribution, we want to share initial analysis on RRM requirement impacts from RAN4 perspective based on the listed feature from RAN1/RAN2 perspective.

Specify the signalling and behavior of the following side control information for controlling the NCR-Fwd [RAN1, RAN2]

- Beamforming

- UL-DL TDD operation

- ON-OFF information

Note: Power control aspect will be checked in RAN#98e.

Study the RRM functions to be supported and specify the RRM requirements of NCR-MT if necessary [RAN2, RAN4]

Study and specify the RF and EMC requirements of NCR if necessary [RAN4]

Note: The existing requirements defined in RAN4 can be reused if applicable.

Note: The work in RAN4 for beam related is expected to start on FR2 first.

Discussion

## 2.1. RF architecture for NCR

Firstly of all, before going into the details of Rel-18 NCR-MT RRM requirement discussion, it’s better to understand its RF architecture of NCR and then further discuss whether the RRM for NCR-MT is necessary or not. Compared with Rel-17 RF repeater, one NCR-MT entity is agreed by RAN1 during the study item phase to receive the control information from parent gNB to control the beamforming, UL-DL TDD operation and ON-OFF information of NCR-Fwd link. In addition, NCR-MT transmitter in UL is also necessary to conduct the random access to the network which has already been captured in solutions for repeater management in TR38.867 [2], provide the HARQ feedback for the received DL signal,etc.

In the following Figure 1/ 2, we illustrate how the NCR-MT entity is integrated into the legacy RF repeater in both FR1 and FR2.

For NCR-MT receiver in the downlink direction, NCR-MT is supposed to decode the in-band signals within the pass-band to obtain its side control information to assist the transmission/reception of NCR-Fwd link. For the following figure, NCR-MT could share some RF components with NCR-Fwd link from the cost efficiency perspective, however the dedicated/separated RF chain for NCR-MT receiver is also one possible solution which should be up to the vendor’s implementation.

For NCR-MT transmitter in the uplink direction, NCR-MT is supposed to transmit the in-band uplink signal as necessary behaviors (e.g. to conduct the random access to the network, provide the HARQ feedback for the received DL signal. etc). Similar as NCR-MT receiver, NCR-MT could somehow share some RF components with NCR-Fwd link from the cost efficiency perspective, however as we mentioned before, the dedicated/separated RF chain for NCR-MT receiver is also one possible solution which should be up to the vendor’s implementation.

Figure 1. the generic illustration of FR1 NCR

Figure 2. the generic illustration of FR2 NCR

## 2.2. RRM requirements for NCR-MT

From RRM perspective, as mentioned in the previous section 2.1, the NCR-MT entity should not only received the side control information from gNB, but also need to maintain the control link between DL input of NCR and the parent gNB.Indeed from our understanding, NCR-MT is somehow similar as low cost IAB-MT to receive the limited side control information from parent gNB instead of providing the backhaul traffic transmissions. To follow the similar principle for IAB-MT RRM requirement, RRM requirement is not applicable for wide area NCR-MT and only applicable for Local area NCR-MT since its expected propagation channel between gNB and wide area NCR-MT should be stable enough without causing too much radio link failure etc.

Proposal 1: RRM requirement is only applicable for Local area NCR-MT and not applicable for wide area NCR-MT.

In the following section, we provide the initial analysis for RRM requirement of NCR-MT on top of Rel-15 NR UE RRM requirement and its details could be found as following.

Table 1. Technical justification for RRM requirement of local area NCR-MT

| Requirements | Detailed RRM Scope | Is requirement needed for Rel-18 NCR- MT? | Technical justifications |
| --- | --- | --- | --- |
| Cell reselection | - Intra-frequency - Inter-frequency | FFS | The requirement for Idle/Inactive state mobility could be further discussed and RAN2 further input might be needed here. |
| Handover |  | FFS | The requirement for handover could be further discussed and RAN2 further input might be needed here. |
| RRC Connection Mobility Control | - RRC re-establishment - Random access - RRC release with redirection | Yes | Essential to make sure Rel-18 NCR-MT can remain RRC connection after some link failure happen. |
| SCell activation/deactivation delays |  | No | For Rel-18 NCR-MT, single carrier operation should be sufficient for the reception of the side control information defined by RAN1/2/3. Don’t see the strong necessity to have Scell configuration and activation. |
| Interruptions related to SCells addition, release, activation, deactivation | Interruptions on serving cells due to any of the addition, release, activation, or deactivation procedures | No | Similar reason as Scell activation/deactivation of CA. |
| PSCell addition/release delays |  | No | NR-DC is not expected to be supported for Rel-18 NCR-MT. |
| Active BWP switching | - Interruptions at active BWP switching - Active BWP switching delay | No | BWP switching for Rel-18 NCR-MT is not expected to happen frequently and don’t see its strong necessity. |
| TCI switching | TCI switching delay | No | It’s not expected to have frequent TCI switching in NCR-MT control link and don’t see its strong necessity. |
| RLM | Requirements for RLM, e.g., based on SSB, CSI-RS or both | Yes | Important feature to maintain NCR-MT Pcell and RRC restablish back to normal RRC connection after declare the RLF happens. Slightly different from Rel-16 IAB-MT. DRX configuration might be still needed since the traffic for side control information is not so high compared with Rel-16 IAB-MT as backhual link with high capacity demand. |
| Link recovery procedures | Requirements for SSB-based and CSI-RS based beam failure detection and candidate beam detection | Yes | Important feature to maintain NCR-MT beam management with its parent gNB. Similar as RLM, DRX might be considered as well. |
| Measurements requirements (time duration, number of cells, etc.) |  | No | Measurement requirements are not that important to Rel-18 NCR-MT due to its fixed deployment. |
| Measurement accuracy requirements |  | No | Similar as Rel-16 IAB-MT. No requirement is needed different from the normal UE. |
| MT timing related requirements | - MT MRTD/MTTD - MT transmit timing - MT timing advance | Yes | The same timing related requirement as Rel-15 shall be applied to Rel-18 NCR-MT. For MRTD/MTTD requirement, this is not applicable since CA/DC is not expected to be supported for Rel-18 NCR-MT. |

Based on the above analysis in table 1, we made the following proposals for further discussions.

Proposal 2: propose to define the following RRM requirement for local area NCR-MT as baseline.

- RRC Connection Mobility Control (RRC re-establishment,Random access,RRC release with redirection);

- RLM

- Link recovery procedures

- MT timing related requirements

Conclusions

In this contribution, we want to share initial views on NCR-MT from the RRM perspective and proposals are made as following:

Proposal 1: RRM requirement is only applicable for Local area NCR-MT and not applicable for wide area NCR-MT.

Proposal 2: propose to define the following RRM requirement for local area NCR-MT as baseline.

- RRC Connection Mobility Control (RRC re-establishment,Random access,RRC release with redirection);

- RLM

- Link recovery procedures

- MT timing related requirements

References

- RP-222673, New WID on NR network-controlled repeater, ZTE, Sanechips, Approved

- RP-222323, TR 38.867 for NR Network-Controlled repeater, RAN#97-e