3GPP TSG-RAN WG4 Meeting#104bis-e                               R4-2216560

E-meeting, Oct 10 – Oct 19,2022

Agenda item:	4.3.4.2

Source: 	ZTE Corporation

Title: 	Further discussion on BS conformance testing for 52.6-71GHz

Document for:	Approval

- Introduction

In the last RAN4#103e meeting, core part for 52.6-71GHz BS RF requirement is basically finalized. During the RAN4#104e meeting, lots of progress except for test environment issues left for further discussion with TE vendors. In this contribution, we want to share further inputs for the updates for EVM measurement in the Annex L based on the latest agreement.

- Discussion

In the last RAN4 meeting, we made the following agreement to have the fixed number of slots for EVM measurement in FR2-2, however based on the current specification, the DL symbols for measurement is limited to the number of slots with downlink symbols within 10 ms measurement period instead of fixed values, therefore the text in Annex L should be updated accordingly.

Issue 2-2-2: Test model data length

Agreement:

- Focus on using a fixed number of [80] slots for EVM measurement time length and test model data length and select the number of slots to ensure a good trade-off between the test time and MU for BS type 2-O transmitter testing in the frequency range between 52.6GHz and 71GHz.

L.7.2a	Averaged EVM (TDD)

For FR2-2 EVM measurement, let  be the 80 slots with downlink symbols, the averaging in the time domain can be calculated from  slots of different 80 slots measurement intervals.

is derived by: Square the EVM results in each 80 slots measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

Where  is the number of resource blocks with the considered modulation scheme in slot i.

The  is calculated, using the maximum of at the window W extremities. Thus  is calculated using  and  is calculated using (l and h, low and high; where low is the timing  and and high is the timing ).

The resulting is compared against the limit and is the total number of 80 slots for EVM measurement.

- Conclusions

In this contribution, we want to further considerations on EVM conformance testing for 60GHz and some proposals and observations are made as following:

Proposal 1: to propose update the Annex L for FR2-2 EVM measurement.

- References

- TS 38.104 17.6.0

- TS 38.141-2 17.6.0

- R4-2210638,WF on BS RF conformance testing for FR2-2,ZTE, approved.

- R4-2210514, Email discussion summary for [103-e][311] NR_exto71GHz_BSRF, Moderator (Nokia), Noted.

- R4-2210577,WF on system parameters for NR extension to 71GHz	Intel Corporation, approved.

- R4-2214374, WF on FR2-2 BS conformance testing, Nokia, approved.

- R4-2214373, WF on FR2-2 BS test environments, Ericsson, approved.