3GPP TSG-RAN WG4 Meeting#104bis-e                               R4-2216562

E-meeting,10th Oct – 19th Oct,2022

Source:	ZTE Corporation

Title:	TP for TS 38.181: Section 7.3

Agenda Item:	4.2.3.2.2

Document for:	Approval

- Introduction

In terms of the work split in the approved WF[1]. This contribution provides a text proposal to TS38.181 [2] on sub-clause 7.3.

- Reference

- R4-2214371, WF on NTN Solutions RF conformance, Ericsson, approved.

- R4-2211688,Draft TS 38.181 v0.1.0, CATT, agreed.

- Text Proposal

----- Start of TP -----

7.3	Dynamic range

7.3.1	Definition and applicability

The dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal at the antenna connector for TAB connector for SAN type 1-H inside the received SAN channel bandwidth. In this condition, a throughput requirement shall be met for a specified reference measurement channel. The interfering signal for the dynamic range requirement is an AWGN signal.

7.3.2	Minimum requirement

The minimum requirement for SAN type 1-H is in TS 38.108 [2], clause 7.3.2.

7.3.3	Test purpose

To verify that the each SAN type 1-H TAB connector receiver dynamic range, the relative throughput shall fulfil the specified limit.

7.3.4	Method of test

7.3.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1.

7.3.4.2	Procedure

The minimum requirement is applied to all connectors under test.

For SAN type 1-H the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested; see clause 7.1.

1)	Connect the connector under test to measurement equipment as shown in annex D.4.2 for SAN type 1-H.

2)	Set the signal generator for the wanted signal to transmit as specified in table 7.3.5-1 according to the appropriate SAN class.

3)	Set the Signal generator for the AWGN interfering signal at the same frequency as the wanted signal to transmit as specified in table 7.3.5-1 according to the appropriate SAN class.

4)	Measure the throughput according to annex A.2.

7.3.5	Test requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.2 with parameters specified in table 7.3.5-1 for SAN LEO class.

Table 7.3.5-1: SAN LEO class dynamic range

| SAN channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -76.1 | -88.2 | AWGN |
|  | 30 | G-FR1-A2-2 | -76.8 |  |  |
| 10 | 15 | G-FR1-A2-1 | -76.1 | -85.0 | AWGN |
|  | 30 | G-FR1-A2-2 | -76.8 |  |  |
|  | 60 | G-FR1-A2-3 | -73.8 |  |  |
| 15 | 15 | G-FR1-A2-1 | -76.1 | -83.2 | AWGN |
|  | 30 | G-FR1-A2-2 | -76.8 |  |  |
|  | 60 | G-FR1-A2-3 | -73.8 |  |  |
| 20 | 15 | G-FR1-A2-4 | -69.9 | -81.9 | AWGN |
|  | 30 | G-FR1-A2-5 | -69.9 |  |  |
|  | 60 | G-FR1-A2-6 | -70.2 |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. |  |  |  |  |  |

----- End of TP -----