3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216583

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RRM requirements without gaps for MG_enh2

Agenda item:	6.10.3.1

Document for:	Discussion

Introduction

This paper presents Nokia’s view on RRM aspects for further enhancements related to gapless measurements for Rel-18. In particular, it presents an overview of the corresponding objective of the work item. Based on that, observations and proposals are derived regarding measurements without gaps. The WID objectives related to this paper are highlighted below:

(2) Define RRM requirements for measurement without gaps for the following cases - NR SSB-based inter-frequency and intra-frequency measurements without gaps for UEs reporting NeedForGapsInfoNR IE [RAN4] - Study whether the additional interruption is allowed when UE reporting ‘NeedForGapsInfoNR'. Further define the interruption length, occasion and ratio, if the interruption is allowed - Define related requirements, such as CSSF, measurement period, scheduling restriction etc. - Inter-RAT measurements without gaps [RAN4] - Inter-RAT NR measurements - Inter-RAT LTE measurement

Discussion

NR requirements for measurements without gaps

During the last RAN4 meeting, the following was discussed regarding spare RF chain [1]:

- Issue 2-19: Available IDLE RF chain in measurement < Way forward >: - Proponents are encouraged to clarify the proposal “requirements need to be distinguished if the UE has at least one idle RF chain available or not under NCSG + NCSG case.”

This discussion has impact not only on the definition of Rel-18 requirements with concurrent gaps including NCSG, but also on the definition of Rel-18 requirements without gaps, since it may influence scheduling restrictions for the measurements without gaps. As part of the Rel-17 NCSG work, inter-frequency measurement requirements could be improved with the assumption that the UE has a spare idle receiver that can be used for inter-frequency measurements. As a result, the VIL was defined considering RF retuning of the spare chain and scheduling restrictions were defined for VIL1 and VIL2 depending on network synchronization and the IBM/CBM capability of the UE. Therefore, we need to clarify basic assumptions on the architecture which will guide the definition of the requirements without gaps.

Assume for measurements without gaps target architecture without idle receiver is available.

In Rel-16, the feature needForGaps was introduced for NR. One way that this feature can be configured is during the RRCReconfiguration procedure, where the Network sends a RRCReconfiguration including needForGapsConfigNR, which includes the NR bands for which the UE is requested to report the gap information. In response the UE sends a RRCReconfigurationComplete including needForGapsInfoNR, which includes the list of Intra-Frequency cells for which a measurement gap is needed. The UE indicates that information including the gapIndication-r16 to indicate whether “gap” or “no-gap” is required. It also includes the list of inter-frequency bands for which a measurement gap is needed.

In addition to the Rel-16 needForGaps, in Rel-17 needForNCSG was introduced in a similar manner, in which the UE might indicate the need of NSCG gaps. In the case of the NCSG, the UE includes additional information as part of the needForNCSG IE indicating “gap”, “ncsg”, “nogap-noncsg”.

Need for gaps include “gap and “no-gap”. Additional interruption might be needed even if gaps are not needed.

Need for NCSG gaps include “gap”, “ncsg”, “nogap-noncsg”.

Figure 1 Signalling diagram for the configuration of NeedForGaps during the RRCReconfiguration procedure.

It is in the scope of the Rel-18 NR_MG_enh2 WI that requirements for measurements without gaps are specified. As part of this work, it should be identified if a UE that is not capable of NCSG might be able to perform measurements without gaps, but still needs some interruption for performing such measurements. In that case, it is likely that some new signalling is needed in order to indicate that behaviour to the network. Additionally, in order to differentiate such feature from NCSG, it is beneficial to study if such interruptions can be smaller than VIL1+VIL2 used in NCSG.

A UE not capable of NCSG might not need gap(s) but some interruption.

If deriveSSBIndexFromCell-inter and deriveSSBIndexFromCell are signaled by the network, then the effective measurement duration can be reduced.

RAN4 to discuss if new signaling would be needed for needForGaps.

RAN4 to study feasible interruption times for measurement requirements without gaps.

As part of the discussion in the last RAN4 meeting, the following issue was discussed regarding interruptions when the UE reports ‘no-gap’[1]:

- Issue 3-1: [NFG] Whether interruption is expected when UE reports ’no-gap’ < Way forward >: - Option 1: Yes - Option 2: No - Option 3: Introduce additional UE capability to differentiate whether UE needs interruption

It is our understanding that when the UE signals no-gaps or no-gaps-noncsg as part of the needForGaps, or needForGapsNCSG implies in no additional interruption. Therefore, it is not desirable to change the interpretation of existing signaling. If Option 1 is agreed in the Issue 3-1 of the WF, that would mean that the gNB would expect different behavior from a Rel 17 UE indicating ‘no-gap’ in comparison to a Rel-18 UE indicating ‘no-gap’. Additionally, if a Rel-18 UE indicates ‘no-gap’ in a network containing Rel-17 gNBs, the gNB would not expect interruptions for that UE, which means that the it would try to schedule the UE during the new interruptions that are implied in Option 1.

A gNB supporting Rel-17 and Rel-16 interprets UEs signalling ‘no-gap’ and ‘nogap-noncsg’ as having no additional interruption.

A UE that requires interruption for performing measurements is not expected to signal ‘no-gap’ and ‘nogap-noncsg’ in Re-17.

Any change on the meaning of ‘no-gap’ and ‘nogap-noncsg’ in Rel-18 UEs would not be supported by Rel-17 gNBs.

Considering the analysis above, any change in the interpretation of the UE indications ‘no-gap’ and ‘nogap-noncsg’ would imply in a mismatch between the meaning of this signaling among devices (both UE and BS) supporting Rel-17 and Rel-18. Therefore, we propose that the interpretation of the existing indications are not changed.

Legacy behavior of existing indication in needForGaps and needForGapsNCSG shall not be changed in Rel 18 NR_MG_enh2.

Furthermore, including the information of whether the UE needs interruption as a UE capability is not solving the problem. If such UE capability is defined, the Rel-17 BS will also not be able to interpret it. As a result, the only feasible way to indicate interruption while performing measurements without gaps is through new Rel-18 specific indication in needForGaps, needForGapsNCSG or in a new information element.

If interruption is needed for a UE without gaps, it should be indicated using new indication as part of needForGaps, needForGapsNCSG or a new information element.

As part of the discussion in the last RAN4 meeting, the following issue was discussed regarding interruptions when the UE reports ‘no-gap’[1]:

Issue 3-4: [NFG] Requirement for intra-freq measurement without gap < Way forward >: - Option 1: Same as requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap) - Option 2: Take requirements NCSG requirements as a starting point - Option 3: Take requirements in Section 9.39 of TS38.133 (inter-freq wo/ gap) as a starting point - Option 4: The related frequency layer should be counted in CSSF outside gap - Option 5: While discussing the measurement period, since similar with the same issue in pre-configured MG, some solution proposed during pre-configured MG discussion can be used for reference. Issue 3-5: [NFG] Requirement for inter-freq measurement without gap < Way forward >: - Option 1: Take requirements in Section 9.39 of TS38.133 (intra-freq w/o gap) as a starting point - Option 2: Take requirements NCSG requirements as a starting point - Option 3: Take requirements in Section 9.3.9 of TS38.133 (inter-freq wo/ gap) as a starting point - Option 4: The related frequency layer should be counted in CSSF outside gap - Option 5: While discussing the measurement period, since similar with the same issue in pre-configured MG, some solution proposed during pre-configured MG discussion can be used for reference.

In our view, the focus of the requirements without gaps is on inter-frequency. The intra-frequency requirements are typically performed without measurement gaps since Rel-15, with the exception of a different numerology, and therefore inclusion of additional interruptions would imply worse performance in comparison to legacy requirements. Therefore, we propose that we focus in our discussion on measurements without gaps only on inter-frequency scenarios.

Intra-frequency measurements are typically performed without measurement gaps.

Intra-frequency measurements do not need retuning.

In our view, the starting point for such inter-frequency measurement requirements should be 38.133, clause 9.2.5.

For intra-frequency measurement requirements use requirements in 38.133, clause 9.2.5 as a starting point.

RAN4 to limit scope of the requirements without gaps with additional interruption to inter-frequency scenarios.

For the inter-frequency scenarios, measurements without gaps are already specified in 38.133, and NCSG was introduced for a UE that has an idle receiver for performing measurement which needs some interruption to retune. Form hardware point of view, the necessity for additional interruption on the measurement without gaps comes from the fact that the idle chain from the UE needs to retune. This is exactly the same reason why VIL is defined for NCSG.

According to proposal 1, no idle receiver is assumed for UE that receives with additional interruptions but without gap. Hence the target architecture consists of 1 Rx chain which needs additional interruption due to retuning to/from the target frequency and due to the measurement window. Taking into account observation 4, UE is enabled to measure the SSB index of the target cell on symbol level, The total additional interruption length hence consists of the retuning delay and the number of symbols for the SSB measurement. In our view, the starting point for such inter-frequency measurement requirements should be 38.133, clause 9.3.9 with the scope to reduce the total additional interruption length compared to NCSG, i.e. VIL1+VIL2..

For inter-frequency measurement requirements use requirements in 38.133, clause 9.3.9 as a starting point.

Consider smaller interruption length than VIL1+VIL2 from NCSG for a UE that requires additional interruptions for measurements without gaps.

Conclusion

This paper has presented Nokia’s views on Rel-18 measurement gaps enhancements. As part of this discussion, we have reached the following observations and made following proposals:

- Assume for measurements without gaps target architecture without idle receiver is available.

- Need for gaps include “gap and “no-gap”. Additional interruption might be needed even if gaps are not needed.

Need for NCSG gaps include “gap”, “ncsg”, “nogap-noncsg”.

A UE not capable of NCSG might not need gap(s) but some interruption.

If deriveSSBIndexFromCell-inter and deriveSSBIndexFromCell are signaled by the network, then the effective measurement duration can be reduced.

RAN4 to discuss if new signaling would be needed for needForGaps.

RAN4 to study feasible interruption times for measurement requirements without gaps.

A gNB supporting Rel-17 and Rel-16 interprets UEs signalling ‘no-gap’ and ‘nogap-noncsg’ as having no additional interruption.

A UE that requires interruption for performing measurements is not expected to signal ‘no-gap’ and ‘nogap-noncsg’ in Re-17.

Any change on the meaning of ‘no-gap’ and ‘nogap-noncsg’ in Rel-18 UEs would not be supported by Rel-17 gNBs.

Legacy behavior of existing indication in needForGaps and needForGapsNCSG shall not be changed in Rel 18 NR_MG_enh2.

If interruption is needed for a UE without gaps, it should be indicated using new indication as part of needForGaps, needForGapsNCSG or a new information element.

Intra-frequency measurements are typically performed without measurement gaps.

Intra-frequency measurements do not need retuning.

For intra-frequency measurement requirements use requirements in 38.133, clause 9.2.5 as a starting point.

RAN4 to limit scope of the requirements without gaps with additional interruption to inter-frequency scenarios.

For inter-frequency measurement requirements use requirements in 38.133, clause 9.3.9 as a starting point.

Consider smaller interruption length than VIL1+VIL2 from NCSG for a UE that requires additional interruptions for measurements without gaps.

- References

- R4-2214346, WF on further enhancements on measurement gaps and measurements without gaps, MediaTek inc.