3GPP TSG-RAN4 #104-bis-e	R4-2216584

Electronic Meeting, 10th –19th Oct 2022

Title:	FR2-1 UL 256QAM: EVM testing using PTRS

Source:	Anritsu Limited

Agenda Item:	6.7.2

Document for:	Discussion

# Introduction

During RAN4 #104-e meeting, UL 256QAM was discussed based on contributions such as [1] and several agreements were made as shown in the corresponding WF [2], among which it was agreed to configure PTRS in order to compensate CPE (while compensation for Inter Carrier Interference (ICI) is FSS). However, the meeting concluded with different views on the PTRS configuration to use including during testing with two options being considered:

- Option 1: PTRS configuration shall be aligned with the UE’s recommended PTRS configuration. (PTRS-DensityRecommendationUL function of UE)

- Option 2: Use a fixed PTRS configuration for all devices (PTRS-DensityRecommendationUL same for all devices)

The aim of this discussion paper is to give the pros and cons of each of those options.

# Discussion

## Option 1: PTRS-DensityRecommendationUL function of UE

| Pros | It will get the best EVM value for each device, and it would be the fair result. |
| --- | --- |
| Cons | Slightly complicated to implement. |
| Other comments | Some features for the implementation - To read the IE. - To convert from the recommendation IE into PTRS configuration parameters. - Flexible PTRS parameter setting is necessary in order to determine the PTRS-DensityRecommendationUL IE for who determine the IE (mobile platform vendor?). |

## Option 2: PTRS-DensityRecommendationUL same for all devices

| Pros | Simplicity. |
| --- | --- |
| Cons | Perhaps several patterns of PTRS configuration are needed, and the minimum result should be treated as the worst result. This is because the EVM may be worse when the PTRS configuration is not fully suitable for the UE. Total test time may be longer. |

# Conclusion

Based on the short analysis above, it can be concluded that any option can be chosen from a TE perspective. However, Option 1 would give better actual EVM and measurements would reflect that.

Observation 1: Option 1 is based on a PTRS-DensityRecommendationUL IE that if its parameters are set correctly can provide better CPE compensation. While for Option 2, the parameters may not suit the actual UE and lead to not proper CPE compensation.

Proposal 1: Option 1 should be chosen as it could give better EVM (assuming optimized PTRS-DensityRecommendationUL parameters) and measurements should reflect that.

# References

[1] R4-2212330, “On enabling FR2 UL256QAM”, Qualcomm Incorporated, RAN4 #104-e, Electronic meeting

[2] R4-2214453, “WF on UL 256QAM”, Xiaomi, RAN4 #104-e, Electronic meeting