3GPP TSG-RAN WG4 Meeting #104bis-e     	R4-2216597

E-meeting, October 10th  – 19th, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on offsets to cell-specific thresholds for 1 Rx RedCap

Agenda item:	4.6.3.1.2

Document for:	Discussion

- Introduction

- In the last RAN4 meeting, RAN4 sent an LS to RAN2 with the following conclusions:

- A RedCap UE with 1 Rx branch applies the offset to the following broadcasted cell-specific RSRP thresholds related to L3 measuremets in RRC_IDLE/INACTIVE state: - rsrp-ThresholdSSB, - msgA-RSRP-ThresholdSSB, - msgA-RSRP-Threshold, - absThreshSS-BlocksConsolidation, - sdt-RSRP-Threshold. - the following cell-specific RSRP thresholds used for Rel-16 low mobility and/or not at cell edge conditions, and Rel-17 stationary and not at cell edge conditions for RRC IDLE/INACTIVE state: - s-SearchDeltaP-r16, - s-SearchDeltaP-Stationary-r17 - s-SearchThresholdP-r16, - s-SearchThresholdQ-r16, - s-SearchThresholdP2-r17, - s-SearchThresholdQ2-r17. - the following cell (re)selection thresholds and ranking: - Qrxlevmin and Qqualmin - The offset is a fixed value in dB in the above cell-specific RSRP thresholds and will be specified in TS 38.133. The magnitude of the exact offset value is [1] dB. It is not expected to introduce new RAN2 signalling.

In this document we discuss the remaining issues in the discussion about the offsets to the cell-specific RSRP thresholds.

- Discussion

- In the last RAN4 meeting, RAN4 agreed that the offset is applied to cell-specific RSRP thresholds related to L3 measurements in RRC_IDLE and RRC_INACTIVE states.

The value of the offset is still in brackets in the LS, and in the last meeting it was also discussed whether the offset should be defined as a positive or as a negative value, depending on whether the threshold is an absolute threshold or a change threshold.

For absolute thresholds, our view is that the offset should assume a value equal to the accuracy degradation observed in measurements performed by 1 Rx UE and 2 Rx UE and agreed in RAN4 #103, as copied below.

In RAN4 #103-e, it was agreed that the absolute and relative accuracy levels for 1 Rx RedCap UE are relaxed by 1 dB in comparison to legacy requirements

Agreement in RAN4 #103-e SSB based L3 measurement with 1 Rx Relaxation of accuracy levels for FR1 - Absolute accuracy level for 1 Rx RedCap is relaxed by 1 dB compared to legacy requirements. - Relative accuracy level for 1 Rx RedCap is relaxed by 1 dB compared to legacy requirements.

For absolute thresholds, the offset to be applied by 1Rx RedCap UEs is equal to 1 dB .

- Conclusions

In this document, Nokia’s view regarding Rel-17 power saving applicability to RedCap UEs is shared. The following observations and conclusions are made:

- In RAN4 #103-e, it was agreed that the absolute and relative accuracy levels for 1 Rx RedCap UE are relaxed by 1 dB in comparison to legacy requirements

- For absolute thresholds, the offset to be applied by 1Rx RedCap UEs is equal to 1 dB .

References

- RP-220966, “Revised WID on support of reduced capability NR devices”

- R4-2214485 WF on RedCap RRM requirements, Ericsson (RAN4 #104)

- R4-2210592 WF on RedCap RRM requirements, Ericsson (RAN4 #103)