3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216600

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on NCD-SSB test cases for RedCap UEs

Agenda item:	4.6.4.1

Document for:	Discussion

Introduction

RedCap UEs support both CD-SSB and NCD-SSB in Rel-17. In the last RAN4 meeting, the following was captured in the WF [1]:

Issue 1: Whether to define test case for NCD-SSB? Candidate options: - Option 1: Yes - Option 2: No Issue 2: Which test case(s) should be used for NCD-SSB test in Handover if issue 1 is yes? Test Index; Test. 1; Intra-frequency handover from FR1 to FR1; known target cell for UE (1Rx, 2Rx). 2; Intra-frequency handover from FR1 to FR1; unknown target cell for UE (1Rx, 2Rx). 3; Inter-frequency handover from FR1 to FR1; unknown target cell for UE (1Rx, 2Rx). Issue 3: Which test case(s) should be used for NCD-SSB test in intra-frequency measurement if issue 1 is yes? Test Index; Test. 1; SA event triggered reporting tests without gap under non-DRX (1Rx, 2Rx). 2; SA event triggered reporting tests with per-UE gaps under non-DRX (1Rx, 2Rx). 3; SA event triggered reporting tests without gap under non-DRX with SSB index reading (1Rx, 2Rx). 4; SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading (1Rx, 2Rx). Issue 4: What’s the NCD-SSB configuration can be introduced if issue 1 is yes? Candidate options: - Option 1: NCD-SSB periodicity 80ms, NCD-SSB offset 5ms - Option 2: NCD-SSB periodicity 40ms, NCD-SSB offset 20ms - Option 3: Others

In this discussion, Nokia’s views are given regarding the introduction of test cases with NCD-SSB, discussed in RAN4 #104.

Discussion

In RAN4 #104, it has been discussed whether or not to introduce NCD-SSB test cases for RedCap UEs in Rel-17. Our view is that RAN4 has introduced requirements for scenarios with NCD-SSB and that it is important that the operation with NCD-SSB is tested in RAN4.

RAN4 has agreed to define requirements for scenarios with NCD-SSB for RedCap UEs in Rel-17.

RAN4 to define RRM test cases with NCD-SSB.

In the last meeting, it was also discussed whether to introduce NCD-SSB tests in Handover. Since RAN2 has agreed that it is possible to use NCD-SSB for the handover, our view is that yes, NCD-SSB test cases should be introduced for this feature.

Agreement from RAN2 #118 From RAN2 perspective, handover scenario 1 is supported. Scenario 1: Handover to a target cell’s specific Redcap BWP associated with NCD-SSB besides to the initial BWP associated with CD-SSB (i.e. UE directly sync to the NCD-SSB and perform RACH on that BWP)

However, our view is that not all cases of handover should be tested with NCD-SSB. In order to guarantee the test coverage for both NCD-SSB and CD-SSB, while still guaranteeing that the number of test cases is not increased, we propose the following:

RAN4 to define the following test cases for Handover:

| Test Index | Test |
| --- | --- |
| 1 | Intra-frequency handover from FR1 to FR1; known target cell for UE (1Rx, 2Rx) 1 Rx: CD-SSB 2 Rx: NCD-SSB |
| 2 | Intra-frequency handover from FR1 to FR1; unknown target cell for UE (1Rx, 2Rx) 1 Rx: NCD-SSB 2 Rx: CD-SSB |
| 3 | Inter-frequency handover from FR1 to FR1; unknown target cell for UE (1Rx, 2Rx) 1 Rx: CD-SSB 2 Rx: NCD-SSB |

For the third issue, we have a similar proposal as Proposal 2: to balance the CD-SSB and NCD-SSB test cases within the 1 Rx and 2 Rx test cases.

RAN4 to define the following test cases for intra-frequency measurements:

| Test Index | Test |
| --- | --- |
| 1 | SA event triggered reporting tests without gap under non-DRX (1Rx, 2Rx) CD-SSB |
| 2 | SA event triggered reporting tests with per-UE gaps under non-DRX (1Rx, 2Rx) NCD-SSB |
| 3 | SA event triggered reporting tests without gap under non-DRX with SSB index reading (1Rx, 2Rx) NCD-SSB |
| 4 | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading (1Rx, 2Rx) CD-SSB |

Regarding Issue 4 in the discussion from the last meeting, we would like to point out that for the offset equal to 5 ms there might be issues with measurement gaps. Furthermore, since the periodicity of the NCD-SSB cannot be less than the periodicity of the CD-SSB we believe that it is sufficient to test NCD-SSB periodicity with 40 ms.

Introduce NCD-SSB configuration as:  NCD-SSB periodicity 40ms, NCD-SSB offset [20 ms]

Conclusion

In the paper, the following Observations and Proposals were made:

- RAN4 has agreed to define requirements for scenarios with NCD-SSB for RedCap UEs in Rel-17.

- RAN4 to define RRM test cases with NCD-SSB.

RAN4 to define the following test cases for Handover:

| Test Index | Test |
| --- | --- |
| 1 | Intra-frequency handover from FR1 to FR1; known target cell for UE (1Rx, 2Rx) 1 Rx: CD-SSB 2 Rx: NCD-SSB |
| 2 | Intra-frequency handover from FR1 to FR1; unknown target cell for UE (1Rx, 2Rx) 1 Rx: NCD-SSB 2 Rx: CD-SSB |
| 3 | Inter-frequency handover from FR1 to FR1; unknown target cell for UE (1Rx, 2Rx) 1 Rx: CD-SSB 2 Rx: NCD-SSB |

RAN4 to define the following test cases for intra-frequency measurements:

| Test Index | Test |
| --- | --- |
| 1 | SA event triggered reporting tests without gap under non-DRX (1Rx, 2Rx) CD-SSB |
| 2 | SA event triggered reporting tests with per-UE gaps under non-DRX (1Rx, 2Rx) NCD-SSB |
| 3 | SA event triggered reporting tests without gap under non-DRX with SSB index reading (1Rx, 2Rx) NCD-SSB |
| 4 | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading (1Rx, 2Rx) CD-SSB |

- References

- R4-2214485, “WF on RedCap RRM Requirements”, RAN4 #104-e, Ericsson

- R2-2206901, “Report of 3GPP TSG RAN WG2 meeting #118-e”; RAN2

- Annex

Agreements in  RAN4 #102-e

The measurement scenarios for NCD-SSB and CD-SSB

- Define RedCap UE’s measurement requirements based on the following scenarios:

  - Case A: Serving cell active BWP includes CD-SSB

  - Case B: Serving cell active BWP includes NCD-SSB

    - Case B-1: All neighbour cells include NCD-SSB on the same frequency location as serving cell NCD-SSB/[CD-SSB]

- FFS whether to support Case B-2

  - Case B-2: Some neighbour cells include NCD-SSB, and some neighbour cells without NCD-SSB on the same frequency location as serving cell NCD-SSB/CD-SSB

  - Note: if the scenario is supported then no new requirements or minimum changes shall be introduced comparing to Case A and B-1 requirements

Whether to define neighbour cell measurement requirements for NCD-SSB

RAN4 to define requirements for RRM measurement on neighbour cell based on CD-SSB or NCD-SSB for the agreed scenarios mentioned above.

Agreements in  RAN4 #103-e

Whether to define requirements for scenario B-2

Case B-2 is supported if no additional requirements or minimum changes shall be introduced compared with other scenario.

Reference SSB to decide measurement type (intra- or inter-frequency)

The issue can be closed based on the following RAN2’s latest agreements.

From RAN2 signalling point of view,

- a BWP-specific servingCellMO is defined under BWP-DownlinkDedicated, the SSB defined in this servingCellMO is the reference SSB to be used for serving cell measurements when the UE is in this active BWP;

- if the field is absent, SSB defined in servingCellMO under ServingCellConfig is the reference SSB to be used for serving cell measurements.

- This reference SSB is used to define intra-frequency measurements.

Definition of SSB based intra-frequency measurement

The centre frequency of the reference SSB of the serving cell and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. The reference SSB is the SSB defined in BWP-specific servingCellMO under BWP-DownlinkDedicated of active DL BWP. If the field is absent, the reference SSB is the SSB defined in servingCellMO under ServingCellConfig.

Agreements in RAN4 #104-e

Requirements for HO to a RedCap specific BWP with NCD-SSB (no CD-SSB) (Scenario 1)

RAN4 to reuse legacy HO requirements for handover directly to RedCap specific BWP with NCD-SSB only without measurement except Tsearch relaxation from 1 Rx reception.