3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216618

# Electronic Meeting, Oct 10 – 19, 2022

Title:	Discussion on lower humidity limit in normal temperature test environment

Source:	ZTE Corporation

Agenda item:	8.3.1

Document for:	Approval

- 1.	Introduction

In RAN5#94-e meeting, an LS on lower humidity limit in normal temperature test environment was sent to RAN4 [1]. It is identified that there are inconsistencies on the lower humidity limit in the normal temperature test environment among 2G/3G/4G/5G specifications. The following question was raised by RAN5.

To RAN4 group.

ACTION:

- RAN WG5 respectfully requests RAN WG4 to provide the reason why the lower humidity limit in normal temperature test environment differs in GERAN/UTRAN/E-UTRAN/NR. Are there any detail considerations to keep the lower humidity limit for normal test environment?

The lower humidity limit was first introduced into 3GPP TS 05.05 for radio transmission and reception of GSM/EDGE with relative humidity of 25% to 75% for normal conditions. Later, a CR in [2] to remove the lower limit for humidity was approved for GSM spec, while for the upper limit the CR suggested to keep it unchanged since the upper limit was considered do have an effect on RF performance. The 3G/4G/5G specs inherit the description of humidity from GSM spec. In previous RAN4 meetings, some solutions are discussed to solve the inconsistencies between RAN4 and RAN5. The main disagreement lies in the aspects in which RAN group the inconsistencies will be solved. In this contribution, we further provide our considerations on solving the inconsistencies with the humidity limit between RAN4 and RAN5. A draft reply LS to RAN5 is attached in the following.

- 2. 	Discussion

During the discussion in RAN4#104-e meeting, three options to solve the inconsistencies between RAN4 and RAN5 on humidity limit in normal temperature test environment were proposed [3].

- Proposals

  - Option 1: remove the explicit humidity range and normal temperature test is required to be performed under room humidity condition unless otherwise stated.

  - Option 2: RAN4 will not update the definition of test environment. RAN5 is encouraged to continue discussing and deciding whether the same approach could apply to 5G NR in the context of conformance testing.

  - Option 3: keep the current description in RAN4 spec with the relative humidity range of “25% ~ 75%” as the solution to resolve the inconsistencies among the specs.

To our understanding, for different RATs the test environment should be the same. The relative humidity of “25% to 75%” for normal test conditions should be uniformly specified for GERAN/UTRA/E-UTRA/NR systems in RAN4 specifications. The high or low humidity may affect the stability and reliability of devices. From the perspective of the propagation characteristics of wireless signals, humidity may also have potential impacts on the radio test results. Considering that the definition of test environment has been specified in RAN4 for long years, to keep the core spec stable is preferable to the industry.

With regard to the option 1, it seems not clear what exactly the room humidity condition refers to. There is no explicit definition for the room humidity condition in the specifications. Actually we know in summer or in winter, in north area or in south area, the room condition are all different. For a test environment specified in the specification, an explicit declaration is necessary. Otherwise, it will be no difference with nothing to be declared.

With regard to the option 2, the main idea is similar to option 3 in which the content in core specification in RAN4 is kept unchanged. The difference is that for option 3 RAN5 is encouraged to continue discussing the issues within the working group. The question is that if RAN5 after further discussion decides to keep the humidity limit as “up to 75%”, then the inconsistencies between RAN4 and RAN5 will remain.

Observation 1:	The relative humidity for normal test conditions should be uniformly specified for GERAN/UTRA/E-UTRA/NR systems.

Observation 2:	It is observed that to keep the core specification stable is preferable to the industry.

As stated in [4], the high or low temperature and humidity may affect the stability, reliability and life of communication devices. In general, when the relative humidity is 100%, the air is considered to be saturated; when the relative humidity is lower than 40%, the air is considered to be dry; when the relative humidity is higher than 80%, the air is considered to be humid. When the relative humidity is greater than 65%, the surface of the object will be covered by a water film of 0.001~0.01μm thick. Such water film is invisible but it affects the operation of the communication devices. When the air is saturated, the water film will be much thicker such as to 10μm. If the relative humidity is not changed, the higher the temperature, the greater impact of water vapor on the devices. This is because water vapor pressure increases and water molecules can easily enter into the material. The higher the relative humidity, the thicker the water film formed on the surface of electronic components, resulting in the formation of conductive water. Furthermore, the plastic and rubber products will be deformed or damaged due to water absorption. When the relative humidity is increased from 20% to 80%, it is said that the paper size will be increased by 0.8%. Although the humidity requirement of the test environment is not as obvious as the requirement of temperature, its importance cannot be underestimated. If the humidity is too high, the metal material will be oxidized and corroded. As a result, the metal parts and the connectors of the equipment will be corroded, and the insulation strength of the material will be reduced. In some serious cases, short circuits may occur. If the humidity is too low, static electricity may be easily generated, which threatens the safe operation of communication devices. To ensure the relative humidity of the test environment meets the requirements, a humidifier or a dehumidifier could be configured as required. Apart from the above mentioned, it is also noted that from the perspective of the propagation characteristics of wireless signals, temperature and humidity may have potential impacts on the radio test results. Therefore, under normal conditions, it is more appropriate to maintain the range of 25%~75% for the humidity test environment.

Observation 3:	The high or low humidity limit for the normal conditions may have potential impacts on some test items and cannot be ignored.

Proposal 1:	It is suggested to select Option 3, i.e. to keep the current description in RAN4 spec with the relative humidity range of “25% ~ 75%” as the solution to resolve the inconsistencies among the specs.

A draft reply LS to RAN5 is attached as follows.

# References

- R5-221604, LS on lower humidity limit in normal temperature test environment R5-221066, ZTE Corporation, Samsung R&D Institute UK

- GP-080557, Removal of lower limit for humidity in Annex A1.2, TS 51.010-1, Nokia

- R4-2214251, Email discussion summary for [104-e][140] NR_reply_LS_UE_RF, Apple

- R4-2213618, Draft Reply LS on lower humidity limit in normal temperature test environment, ZTE Corporation

3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-22xxxxx

Electronic Meeting, 10 – 19 Oct, 2022

Title:	[Draft] Reply LS on lower humidity limit in normal temperature test environment

Response to:	R5- 221604

Release:	Release 15

Work Item:	5GS_NR_LTE-UEConTest

Source:	RAN4

To:	RAN5

Cc:	N/A

Contact Person:

Name:	Zhifeng Ma

- E-mail Address:	ma.zhifeng@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 thanks for RAN5 LS R5- 221604.

Regarding the following question from RAN5:

RAN WG5 respectfully requests RAN WG4 to provide the reason why the lower humidity limit in normal temperature test environment differs in GERAN/UTRA/E-UTRA/NR. Are there any detail considerations to keep the lower humidity limit for normal test environment?

- D.2.1	Temperature (GSM 400, GSM 900 and DCS 1800)

The MS shall fulfil all the requirements in the full temperature range of:

+15C  -  +35C	for normal conditions (with relative humidity of 25 % to 75 %);

-10C  -  +55C 	for DCS 1 800 MS and small MS units extreme conditions
(see IEC publications	 68-2-1 and 68-2-2);

-20C  -  +55C 	for other units extreme conditions (see IEC publications 68-2-1 and 68-2-2).

Outside this temperature range the MS, if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the MS exceed the transmitted levels as defined in 3GPP TS 05.05 for extreme operation.

RAN4 discussed the issue of lower humidity limit in normal temperature test environment. After further check with the history of GSM/WCDMA/LTE specs, it is found that the lower humidity limit was first introduced into 3GPP TS 05.05 for radio transmission and reception of GSM/EDGE shown as below.

The definition of lower humidity limit in normal temperature test environment in GSM specifications has been specified as “25 % to 75 %” since version 7.0.0 for more than 20 years. The content of this part has been continued to be used in RAN4 specifications for UTRA, LTE and NR systems. A narrow range of relative humidity of “25 % to 75 %” is more reasonable since “up to 75%” indicates that UE should also support the condition under relative humidity of “0% to 25%” which is an extra requirement to the current UEs.

Although for most of the test cases under normal temperature conditions the humidity does not affect the test performance of UE as much as temperature, the high or low humidity environment may have potential impacts on some other test items, such as ESD (Electro-Static Discharge) do have strict requirements in humidity. The humidity may also affect the stability, reliability and radio propagation of communication devices. For some instruments or system components in mmW dark chamber, the requirements for relative humidity should also be considered.

RAN4 concludes that the relative humidity of “25% to 75%” for normal test conditions should be uniformly specified for GERAN/UTRA/E-UTRA/NR systems in RAN4 specifications.

2. Actions:

To RAN5

ACTION: 	RAN4 kindly asks RAN5 to take above RAN4 answers into consideration in their future work.

3. Date of Next TSG-RAN WG4 Meetings:

3GPP RAN4#105	14 Nov – 18 Nov 2022	Toulouse, France

3GPP RAN4#106	27 Feb – 03 Mar 2023	Athens, Greece