# 3GPP TSG-RAN WG4 Meeting # 104-bis-e					R4-2216635

# Electronic Meeting, 10th  –19th October, 2022

Source:		Ericsson

Title:			IoT UE remaining RF issue

Agenda item:		7.5.5

Document for:		Approval

# Introduction

In [1], there are several remaining issues including:

- A-MPR:

For b255

- Option 1: Depends on outcome of SEM/ACLR discussion and spurious emission for UE coexistence.

- Option 2: Reuse n255 requirements from NR NTN

- Option 3: Already clear that A-MPR is not needed.

For b256

- Option 1: Depends on outcome of SEM/ACLR discussion and spurious emission for UE coexistence.

- Option 2: Reuse n256 requirements from NR NTN

- Option 3: Already clear that A-MPR is not needed.

- Frequency error

Agreement:

  - Reuse 0.1 and 0.2 ppm requirements of frequency error and further discuss the condition where the requirements are applied.

- SEM

  - All categories: Option 1: Assume TN as baseline, and reconfirm after coexistence verification

- UE co-existence

  - Option 1: Further input needed but consider NR NTN and existing E-UTRA TN as baselines.

- Reference sensitivity

  - Cat-M1 (1.4MHz):

    - Use b24 value for b255

    - FFS on b256

- Out-of-band blocking

  - For b256: Option 2: wait for NR NTN outcome in RAN4#104-e.

In this paper, we present our view on the IoT UE RF requirement overview.

# Discussion

## A-MPR

The A-MPR relates to the additional SEM/spurious requirement relating the regulatory requirement for different bands. In TR 38.863 [2], the FCC requirement of 47 C.F.R § 25.216 (h) which is related to n255/255 is captured as the 3GPP requirement in clause 6.5.3.3.2 in TS 38.101-5 for network signalling NS_57N, however, the FCC requirement of  47 C.F.R. § 25.202 (f) is not captured in TS 38.101-5. As the NTN IoT follow the NTN discussion and it is a question whether such requirement will be needed for NTN UE or the SEM mask could cover this FCC requirement. Figure 1 and Figure 2 shows the overlaid mask with both 3GPP LTE SEM for Cat-M1 and 3GPP NB-IoT SEM for NB-IoT. It shows that at the closed frequency offset to the channel edge, the FCC requirement is more stringent.

The FCC 25.202(f) requirement is more stringent than the 3GPP SEM mask for Cat-M1 and NB-IoT.

Figure 1: Fcc 25.202(F) mask overlaid with 3GPP LTE SEM Mask.

Figure 2: Fcc 25.202(F) mask overlaid with 3GPP NB-IoT SEM Mask.

Thus we propose to consider to introduce the additional SEM requirement for FCC 25.202 (f), as stated in TR38.863, the FCC technical rules apply from 1GHz to 3GHz and thus such requirement should apply both n255 and n256. The Text proposal is below:

6.5.2.3	Additional spectrum emission mask

6.5.2.3.1	Requirements for network signalling value "NS_xy"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_xy" is indicated in the cell, the power of any UE emission shall be attenuated below the levels below the mean output power of transmitter specified in Table 6.5.2.3.1-1.

Table 6.5.2.3.1-1: Additional requirements for "NS_xy"

| Spectrum emission limit (dBr) / Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 1.4MHz MHz for NTN Cat-M1 and 200 kHz for NTN NB-IoT | Measurement bandwidth (MBW) |
| ± 0- (BWChannel / 2) | -25 | 4 kHz |
| ± (BWChannel / 2) – 2BWChannel | -35 |  |
| > ± 2BWChannel | – 43 +23-30 = -50 for PC3 -43+20-30= -53 for PC5 |  |
| NOTE : The measurement bandwidth shall be 4kHz. |  |  |

Consider introducing the above additional SEM in the NTN IoT UE specification.

For additional SEM/spurious requirement in TS 38.101-5, the same NS signalling is reused if the additional requirement is the same with the TN. The same principle could be applied here also if the additional SEM/spurious requirement is the same with TN in TS 36.101.  For FCC 25.202 and 25.216, these are new requirements specific for NTN and thus the new NS signalling will be needed. These two requirements can share the same NS value as both additional SEM and additional spurious requirement will be applied at the same time.

The same NS value can be applied to the additional SEM and additional spurious requirement in TS 38.101-5

- As the SEM baseline is the TN requirement, so here we assume the SEM is the same with LTE SEM for NTN Cat-M1 and LTE NB-IoT SEM for NTN NB-IoT. There is no A-MPR consideration for NB-IoT as agreed in last meeting and A-MPR for NTN Cat-M1 is not decided according to WF[1]. From the discussion above, we think additional SEM requirement should be applied both for n255 and n256 and as such the A-MPR requirement need to investigate considering this new requirement. For the additional spurious requirement for band 255, we believe there should be no A-MPR needed following the measurement results [3] and the A-MPR simulation can be done only for the additional SEM requirement. The A-MPR for band 256 should investigate with both additional SEM requirement proposed in this paper and the existing additional spurious requirement (NS_24) in TS 36.101.

Further investigate the A-MPR with the new additional SEM requirement for b255 and b256.

## Frequency error

The NTN IoT frequency error requirement could reuse the NTN frequency error requirement as the pre-compensation of the UL frequency for doppler frequency is the same with NTN UE and NTN IoT UE. The difference is that for NTN IoT UE, the doppler frequency compensation should be done per segment due to the long UL transmission.

Currently the NTN frequency error requirement is stated below in TS 38.101-5, it is tested with two points, one is with Doppler frequency compensation and one without (pre-compensation is 0 Doppler frequency).

### 6.4.1	Frequency error

The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

[NOTE:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift. For the test case, the location of the UE is explicitly provided to the UE from the test equipment.]

Requirement will be verified for at least two cases of which one has zero Doppler conditions

In the current specification, there is unclearness on the “ideally pre-compensated reference uplink carrier frequency”, as the NTN discussion regarding the UE pre-compensated doppler frequency algorithm is up to UE implementation, so it means there could be doppler frequency compensation accuracy performance between different UE implementations. TE must have ability to derive the ideally doppler frequency otherwise this “ideally” doppler frequency accuracy could be deviated between TEs and is not ideal doppler frequency anymore. In this sense, it seems there is need to define the “ideally” doppler frequency derivation in TE so the UE with vendor specific algorithm can be tested.

There is a need to specify the ideally doppler frequency derivation for TE

Another aspect is on the test coverage of UE pre-compensated doppler frequency functionality and this relates to the question how to emulate the satellite movement in the test system. In real NTN network, the DL signal received by NTN IoT UE will experience varying doppler frequency due to the satellite movement, UE will calculate the pre-compensated doppler frequency according to received ephemeris data and its position retrieved from GNSS. It may be difficult to emulate the satellite movement in test system and so the test coverage of the pre-compensated doppler frequency is limited.

In current FE requirement, the implication is that TE will receive a UE transmitted signal without the doppler shift emulation, in such case, to test the frequency error against the ideal pre-compensated frequency, the TE needs to set the LO at such frequency and using the legacy FE measurement procedure to derive the FE.  As mentioned earlier, the ideal doppler frequency is varying with time and thus TE needs to adjust its LO in time.  The ability to set the LO with changing frequency without disturbing the test process needs to be confirmed with TE vendors. Another way to implement the test is to set the TE LO with nominal UL frequency as the same as the legacy FE test, in such a case, TE needs to measure and compensate the doppler frequency shift relative to UL configured frequency point F0_UL, such compensation of FE with ideal frequency shift could be implemented in baseband and thus bring the advantage of the software update with TE hardware change.

It will be easier for TE to measure and compensate the pre-compensated doppler shift by UE relative to the nominal UL frequency

It should be noted that currently frequency error test is assuming any constant frequency error in UE should be within 0.1 ppm and preferably within sub-SCS range for connected mode test. The doppler shift could be up to around +/- 50kHz, for a SCS of 15khz, it could be then up to 3 times integer 15kHz and as TE needs to set up RRC connection with UE, the TE should be able to synchronize to the UE. RAN4 should check whether the current testing procedure in Annex F can apply without any changes assuming the frequency error to be detect is bigger than legacy considering the doppler frequency shift.

Discuss further whether the current test methodology is good enough to measure the additional frequency error caused by frequency pre-compensation in UE.

Current measurement in annex F is based on IQ data which is baseband processing and with the doppler frequency pre-compensation, the received signal may be outside its assigned bandwidth for NB-IoT considering the maximum doppler of +/- 50kHz as guard band in NB-IoT carrier is 10kHz. This is not issue for CAT-M1 as the guard band in Cat-M1 is 160kHz. One possible solution could be increasing the “baseband” signal bandwidth for NB-IoT to 300kHz so to capture he baseband signal with the doppler shift. Another solution is to measure the coarse frequency error in terms of the integer number of SCS and frequency error of fractional SCS.

- The remaining question could be how to align the TE and UE understanding on the doppler frequency shift set by TE and pre-compensated by UE. RAN1 and RAN2 has designed the mechanism based on GNSS + ephemeris data (satellite position + sattleite velocity), the same method could be assumed to be used by TE and UE. One thing to align this aspect is that how to derive the UE position, As rel-17 only assume NTN UE with GNSS capability, so UE should derive its UE position from GNSS integrated in the UE, however, TE should also assume the same UE position by accessing a GNSS receiver; alternatively the AT command could be designed to make sure UE and TE use the same UE position, however, this method does not cover the UE pre-compensation function with accessing its position data via GNSS and thus the test coverage is limited. To access a GNSS receiver could impact the TE hardware design which makes the legacy TE difficult to test this case. RAN4 could leave this issue to RAN5 decide.

Whether to have GNSS access at TE could leave to RAN5 to decide.

- The UE position could be assumed pre-known to TE and the doppler frequency derivation from the ephemeris data should be specified in Annex so that the frequency error caused by the deviation from the UE estimated amount and TE pre-set would be minimized. The coordinate system for non-GEO could be the same as the 6.3 in TR 38.811.

Figure 6.3-1: Illustration of the coordinate system

- Assuming the satellite position is S1=(Sx1, Sy1,Sz1) and satellite velocity is V1=( (Vx1, Vy1,Vz1), and UE position is  U =(Ux, Uy, Uz) at time T1,  the satellite position is  S2 =(Sx2, Sy2,Sz2) and satellite velocity is V2 =(Vx2, Vy2,Vz2) at time T2,

- fdoppler(T1)=(S1-U),V1S1-U∙fcc

- The doppler frequency at time T could be linear extrapolation with information of T1 and T2, the time between T1 and T2 could be set to be short so the uncertainty could be reduced.

The doppler frequency should be specified in annex  so that frequency error caused by the deviation from the UE estimated amount and TE pre-set would be minimized.

- With the above discussion, the current frequency error requirement could be improved with below, TE need to compensate with the ideal pre-compensated doppler frequency.

### 6.4.1	Frequency error

The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals after compensation with ideally pre-compensated doppler frequency compared with nominal reference uplink carrier frequency.

[NOTE:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift. For the test case, the location of the UE is explicitly provided to the UE from the test equipment.]

RAN4 discuss the above changes for the frequency error requirement considering the test discussion above.

- The proposed annex is below:

- Annex Z: Frequency error compensation for pre-compensated doppler shift

- For the frequency error measurement for NTN IoT UE, TE needs to compensate the measured frequency error with ideal doppler frequency. The frequency error magnitude to be compensated for doppler shift is described in Z.1 and frequency offset measurement due to doppler frequency is described in Z.2.

- Z.1. Derive the doppler frequency for post-compensation

- Assuming the satellite position is S1=(Sx1, Sy1,Sz1) and satellite velocity is V1=( (Vx1, Vy1,Vz1), and NTN IoT UE position is  U =(Ux, Uy, Uz) at time T1,  the satellite position is  S2 =(Sx2, Sy2,Sz2) and satellite velocity is V2 =(Vx2, Vy2,Vz2) at time T2,

- fdoppler(T1)=(S1-U),V1S1-U∙fcc

- And

- fdoppler(T2)=(S2-U),V2S2-U∙fcc

- The doppler frequency fdopplerT at transmission time T for post-compensation could be linear extrapolation with fdoppler(T1)of and fdoppler(T2) , the time between T1 and T2 could be set to be short so the uncertainty could be reduced. fc is the UL configured frequency.

- Z.2. Frequency error measurement due to doppler frequency after post-compensation

Measure the frequency offset Δf in Annex F, then the frequency error measured after the TE post-compensation of doppler frequency:

∆f(T)f=Δf-fdopplerTfc

Discuss the above annex for the doppler frequency measurement.

## SEM and ACLR

Our companion paper[6] in coexisting simulation shows that it is fine to reuse the TN ACLR for NB-IoT. There is some difficult ACIR if the isolation distance is not used, for example, the TN DL aggressor and NTN IoT victim and this is the same with NTN and TN coexisting. There would be no issue if isolation distance would be assumed. The TN SEM can be reused also. This the same for cat-M1 as the coexisting simulation can depend on the NB-IoT as the ACLR model is flat in our simulation. The question is that how to handle the Foffset specified in TN specification for ACLR_GSM. In NTN coexisting simulation, there is no additional 100kHz guard band configured for NB-IoT carrier to the TN adjacent carrier, this is different with the 36.802 coexisting simulation assumption in Table 6.2-1.  So the NTN IoT UE could specify the ALCR without the 100kHz gurad band.  The question is then whether the legacy device could be reused when first adjacent channel ACLR requirement is changed without 100kHz guard band. If the legacy device could be reused directly without any change is preferred, the legacy requirement should be followed.

Legacy ACLR requirement should be reused if to reuse the legacy device directly to the NTN.

## UE co-existence

The spurious requirement for UE co-eixstence is to protect the coexisting bands in the same region. The NTN IoT operating in band 255 and band 256 should be same requirement to protect the coexisting bands. Therefore,  the NTN IoT UE spurious requirement for coexisting could be reused with the protected band modified to 255 and 256. And this will apply both to NB-IoT and Cat-M1 in band 255 and 256.

Reuse the NTN spurious requirement for coexisting with modification of operating band number.

## Reference sensitivity

Last meeting it is agreed to use B24 REFSENS for B255 for cat-M1, but FFS on B256. The reason is that there is similar discussion for NTN regarding the filter used by n256. It is agreed in NTN WI that 30MHz passband filter (dedicated for n256) and 90 MHz passband filter (n65) should be both considered and thus the OOB is further relaxed for n256 requirement.

- Agreement:

- Both 30MHz and 90MHz duplexer could be accommodated.

- Note:  Requirement itself would be applicable for n256 instead of referring to band n65 in the TS 38.101-5 spec

- Note:  Above assumption agreed under the condition there is no impact on the competition of NTN UE RF core requirements.

Based on such agreement, NTN IoT operating in 256 could reuse the assumption and derive the REFSENS. There is no similar/same band in LTE supported for cat-M1 and the REFSESN could be derived using the previous cat-M1 REFSENS WF. The REFSESN for 2 RX UE in B65 for 1.4MHz BW is – 104.2 dBm, with scaling with 2.5 dB [7], the 1 Rx UE REFSENS is -101.7 dBm. The HD-FDD REFSENS is -104.2 + 1.7 = -102.5 dBm. However, when B65 REFSENS is discussed in TR 36.862, the Rx filter loss is -0.4 dB to 0.4 dB compared to average value of B1, in the end B65 REFSENS is relaxed 0.5 dB compared to B1 REFSENS when B65 filter was not available. As the NTN IoT UE will experience low signal value, we think B256 REFSENS could be tightened with 0.5 dB with reference to B65 REFSENS with 0.5 dB tightening.

Consider the NTN Cat-M1 REFSENS based on B65 REFSENS with half dB tightening in TS 36.101.

## Out-of-band blocking

It is agreed in RAN4#104e that the OOB requirement for NTN UE:

Sub-topic 2-1 OOBB requirements for n256

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Void NOTE 3: Void NOTE 4: Void |  |  |  |  |  |

- Agreement: Option 1

The understanding of such agreement is based on filter design performance considering both 30MHz and 90MHz filter. Assume the same for NTN IoT UE and filter design for UE is implementation specific, there is no issue to reuse the same OOBB requirement.  Additionally, as there is additional relax in NOTE 2 for the range 3 blocker level for NB-IoT in TS 36.101, the same relaxation should be applied to NTN NB-IoT.

OOB requirement for NB-IoT:

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -152 |
| 255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: For operating bands which downlink band frequency range is between 1475.9 MHz < f < 2690 MHz the power level of the interferer (PInterferer) for Range 3 shall be modified to: -20 dBm for the frequency range which is bounded by FDL_low- 200 MHz of the lowest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz and FDL_high + 200 MHz of the highest band that UE supports supports in frequency range 1475.9 MHz < f < 2690 MHz.” |  |  |  |  |  |

OOB requirement for Cat-M1:

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations |  |  |  |  |  |

Considering the above OOB for NTN cat-M1 and NTN NB-IoT.

# Conclusions

In this contribution, we present our view on the remaining RF requirement overview for IoT UE with below observations and proposal:

Observation 1 The FCC 25.202(f) requirement is more stringent than the 3GPP SEM mask for Cat-M1 and NB-IoT.

Proposal-2: The same NS value can be applied to the additional SEM and additional spurious requirement in TS 38.101-5

Proposal-3:Further investigate the A-MPR with the new additional SEM requirement for b255 and b256.

Observation 3:It will be easier for TE to measure and compensate the pre-compensated doppler shift by UE relative to the nominal UL frequency

Observation 4: Discuss further whether the current test methodology is good enough to measure the additional frequency error caused by frequency pre-compensation in UE.

Proposal-4:Whether to have GNSS access at TE could leave to RAN5 to decide.

Proposal-5:The doppler frequency should be specified in annex  so that frequency error caused by the deviation from the UE estimated amount and TE pre-set would be minimized.

Proposal-6:RAN4 discuss the above changes for the frequency error requirement considering the test discussion above.

Proposal-7:Discuss the above annex for the doppler frequency measurement.

Proposal-8:Legacy ACLR requirement should be reused if to reuse the legacy device directly to the NTN.

Proposal-9:Reuse the NTN spurious requirement for coexisting with modification of operating band number.

Proposal-10:Consider the NTN Cat-M1 REFSENS based on B65 REFSENS with half dB tightening in TS 36.101.

Proposal-11:Considering the above OOB for NTN cat-M1 and NTN NB-IoT.

# References

- R4-2214467, Way forward on NB-IoT/eMTC NTN agenda item 12.5.1 & 12.5.4, MediaTek Inc

- TR 38.863.

- R4-2207967, Measurements for n255 A-MPR evaluation, Ligado Networks

- TR 38.811

- TS 38.521-1

- R4-22xyz, IoT NTN coexisting initial results, Ericsson

- R4-156923, Way forward for rel-13 eMTC REFSENS, Sony

- R4-2214372,	WF for NTN UE RF maintenance, ZTE