# 3GPP TSG-RAN WG4 Meeting # 104-bis-e					R4-2216640

# Electronic Meeting, 10th  –19th October, 2022

Source:		Ericsson

Title:			On NTN Frequency error requirement

Agenda item:		4.2.4

Document for:		Approval

# Introduction

In this paper, we present our view on the improvement of NTN Frequency error requirement.

# Discussion

Currently the NTN frequency error requirement is stated below in TS 38.101-5, it is tested with two points, one is with Doppler frequency compensation and one without (pre-compensation is 0 Doppler frequency).

### 6.4.1	Frequency error

The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

[NOTE:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift. For the test case, the location of the UE is explicitly provided to the UE from the test equipment.]

Requirement will be verified for at least two cases of which one has zero Doppler conditions

In the current specification, there is unclearness on the “ideally pre-compensated reference uplink carrier frequency”, as the NTN discussion regarding the UE pre-compensated doppler frequency algorithm is up to UE implementation, so it means there could be doppler frequency compensation accuracy performance between different UE implementations. TE must have ability to derive the ideally doppler frequency otherwise this “ideally” doppler frequency accuracy could be deviated between TEs and is not ideal doppler frequency anymore. In this sense, it seems there is need to define the “ideally” doppler frequency derivation in TE so the UE with vendor specific algorithm can be tested.

There is a need to specify the ideally doppler frequency derivation for TE

Another aspect is on the test coverage of UE pre-compensated doppler frequency functionality, and this relates to the question how to emulate the satellite movement in the test system. In real NTN network, the DL signal received by NTN IoT UE will experience varying doppler frequency due to the satellite movement, UE will calculate the pre-compensated doppler frequency according to received ephemeris data and its position retrieved from GNSS. It may be difficult to emulate the satellite movement in test system and so the test coverage of the pre-compensated doppler frequency is limited.

In current FE requirement, the implication is that TE will receive a UE transmitted signal without the doppler shift emulation, in such case, to test the frequency error against the ideal pre-compensated frequency, the TE needs to set the LO at such frequency and using the legacy FE measurement procedure to derive the FE.  As mentioned earlier, the ideal doppler frequency is varying with time and thus TE needs to adjust its LO in time.  The ability to set the LO with changing frequency without disturbing the test process is a question to TE vendors. Another way to implement the test is to set the TE LO with nominal UL frequency as the same as the legacy FE test, in such a case, TE needs to measure and compensate the doppler frequency shift relative to UL configured frequency point F0_UL, such compensation of FE with ideal frequency shift could be implemented in baseband and thus bring the advantage of the software update with TE hardware change.

It will be easier for TE to measure and compensate the pre-compensated doppler shift by UE relative to the nominal UL frequency

The currently requirement asking the frequency error to be derived with relative to the ideal pre-compensated frequency is not align with the legacy FE requirement which compare to the UL configured frequency. We think it is reasonable to compare with the nominal UL frequency instead of the ideal pre-compensated frequency as there will be error associated with the ideal pre-compensated frequency while not the case for configured UL configured frequency.

- Another question is that how to align the TE and UE understanding on the doppler frequency shift set by TE and pre-compensated by UE. RAN1 and RAN2 has designed the mechanism based on GNSS + ephemeris data (satellite position + sattleite velocity), the same method could be assumed to be used by TE and UE. One thing to align this aspect is that how to derive the UE position, As rel-17 only assume NTN UE with GNSS capability, so UE should derive its UE position from GNSS integrated in the UE, however, TE should also assume the same UE position by accessing a GNSS receiver; alternatively the AT command could be designed to make sure UE and TE use the same UE position, however, this method does not cover the UE pre-compensation function with accessing its position data via GNSS and thus the test coverage is limited. To access a GNSS receiver could impact the TE hardware design which makes the legacy TE difficult to test this case. RAN4 could leave this issue to RAN5 decide.

Whether to have GNSS access at TE could leave to RAN5 to decide.

- The UE position could be assumed pre-known to TE and the doppler frequency derivation from the ephemeris data should be specified in Annex so that the frequency error caused by the deviation from the UE estimated amount and TE pre-set would be minimized. The coordinate system for non-GEO could be the same as the 6.3 in TR 38.811.

Figure 6.3-1: Illustration of the coordinate system

- Assuming the satellite position is S1=(Sx1, Sy1,Sz1) and satellite velocity is V1=( (Vx1, Vy1,Vz1), and UE position is  U =(Ux, Uy, Uz) at time T1,  the satellite position is  S2 =(Sx2, Sy2,Sz2) and satellite velocity is V2 =(Vx2, Vy2,Vz2) at time T2,

- fdoppler(T1)=(S1-U),V1S1-U∙fcc

- The doppler frequency at time T could be linear extrapolation with information of T1 and T2, the time between T1 and T2 could be set to be short so the uncertainty could be reduced.

The doppler frequency should be specified in annex  so that frequency error caused by the deviation from the UE estimated amount and TE pre-set would be minimized.

- With the above discussion, the current frequency error requirement could be improved with below, TE need to compensate with the ideal pre-compensated doppler frequency.

### 6.4.1	Frequency error

The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals after compensation with ideally pre-compensated doppler frequency compared with nominal reference uplink carrier frequency.

[NOTE:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift. For the test case, the location of the UE is explicitly provided to the UE from the test equipment.]

RAN4 discuss the above changes for the frequency error requirement considering the test discussion above.

- The proposed annex is below:

- Annex Z: Frequency error compensation for pre-compensated doppler shift

- For the frequency error measurement for NTN IoT UE, TE needs to compensate the measured frequency error with ideal doppler frequency. The frequency error magnitude to be compensated for doppler shift is described in Z.1 and frequency offset measurement due to doppler frequency is described in Z.2.

- Z.1. Derive the doppler frequency for post-compensation

- Assuming the satellite position is S1=(Sx1, Sy1,Sz1) and satellite velocity is V1=( (Vx1, Vy1,Vz1), and NTN IoT UE position is  U =(Ux, Uy, Uz) at time T1,  the satellite position is  S2 =(Sx2, Sy2,Sz2) and satellite velocity is V2 =(Vx2, Vy2,Vz2) at time T2,

- fdoppler(T1)=(S1-U),V1S1-U∙fcc

- And

- fdoppler(T2)=(S2-U),V2S2-U∙fcc

- The doppler frequency fdopplerT at transmission time T for post-compensation could be linear extrapolation with fdoppler(T1)of and fdoppler(T2) , the time between T1 and T2 could be set to be short so the uncertainty could be reduced. fc is the UL configured frequency.

- Z.2. Frequency error measurement due to doppler frequency after post-compensation

Measure the frequency offset Δf in Annex F, then the frequency error measured after the TE post-compensation of doppler frequency:

∆f(T)f=Δf-fdopplerTfc

Discuss the above annex for the doppler frequency measurement.

# Conclusions

In this contribution, we present our view on the NTN frequency error requriement with below observations and proposal:

Observation 1 There is a need to specify the ideally doppler frequency derivation for TE

Observation 2 It will be easier for TE to measure and compensate the pre-compensated doppler shift by UE relative to the nominal UL frequency

Proposal-1: Whether to have GNSS access at TE could leave to RAN5 to decide.

Proposal-2: The doppler frequency should be specified in annex  so that frequency error caused by the deviation from the UE estimated amount and TE pre-set would be minimized.

Proposal-3:RAN4 discuss the above changes for the frequency error requirement considering the test discussion above.

Proposal-4:Discuss the above annex for the doppler frequency measurement.