3GPP TSG-RAN WG4 Meeting #104b-e	R4-2216654

Electronic Meeting, 10-19 October 2022

Source:	Huawei, Hisilicon

Title:	Discussion on Multi-carrier enhancements with multiple TAGs

Agenda item:			6.19.2

Document for:	Approval

# 1. Introduction

In the RAN4#104e meeting, multi-carrier enhancements with multiple TAG was discussed in [1][2]. RAN4 did not identify and come to a conclusion for the impact of Tx switching with multiple TAGs on RAN1 specification in the very first meeting for the topic, which is also aligned with the conclusion in RAN#97e. Therefore, the draft LS[3] on UL Tx switching with multiple TAGs was not sent to RAN1. Besides, there is no additional information in the draft LS apart from the agreement in RAN Plenary.

And in the RAN#97e, the revised WID was updated with Note: The number of TAGs is limited to up to 2. And the following conclusion was captured in the meeting report of RAN#97e.

- Conclusion: for the work on UL Tx switching with 2 TAGs, RAN1/2 discussion can be triggered by RAN4 LS. No RAN1 spec impact is expected.

In this contribution, we would like to focus on the RAN4 impact from Tx switching with multiple TAGs.

# 2. Discussion

## 2.1. UL switching time with multiple TAGs

Tx chain switching is within band pair with two bands, no matter the two bands are chosen from 3 or 4 bands. The Tx switching with multiple TAGs is limited to 2 TAGs as captured in RAN#97e. So there is no difference of Tx switching with multiple TAGs between with 2 bands and with 3/4 bands. There is no additionally cases for 3/4 bands.

In the discussion of single TAG in the previous release, switching time was introduced to describe the UE capability of Tx switching gap with certain band pair and switching pattern, i.e., 1Tx-2Tx or 2Tx-2Tx. In the scenario of Tx switching of multiple TAGs, the switching time within the same band pair and switching pattern should be same as that of single TAG. The number of TAG(s) is related to BS deployment with collocated or non-collocated, and it should not impact on UE capability.

Proposal 1：UL switching time is the same for single TAG and multiple TAGs.

## 2.2. Outage time for multiple TAGs

The existence of TA difference is the main factor to differentiate multiple TAGs from single TAG. Whether to consider TA difference or MTTD in outage time, companies had various views in the last RAN WG4 meeting.

Actually, we have an example in Rel-17 spec, which is similar to Tx switching with multiple TAGs. Due to timing advance difference of V2X and Uu, Rel-17 intra-band V2X and Uu concurrent operation has the similar issue. It was captured in 38.101-1:

6.3E.3.4 Transmit ON/OFF time mask for V2X con-current operation For the inter-band con-current NR V2X operation, the requirements specified in clause 6.3.3 shall apply for the uplink in licensed band and the requirements specified in clause 6.3E.3.2 and 6.3E.3.3 shall apply for the sidelink in licensed band or Band n47. … Figure 6.3E.3.4-2: Time mask for switching between Uu and SL for different carrier case In the real field, there is a timing advance difference, i.e. NTA∙Tc between NR Uu slot and NR SL slot due to different timing advance of NR Uu and NR SL. The switching time masks do not include timing advance difference but the timing advance difference should be considered with the switching time for same carrier case and different carrier case.

We should consider the time mask of multi-TAG Tx switching with reference to intra-band V2X con-current operation. In the case of switching between NR Uu slot and NR SL slot, TA difference is not captured in the switching time mask. The same principle can be followed for UL Tx switching with dual-TAG.

Proposal 2：The principle of intra-band V2X con-current operation time mask can be followed for UL Tx switching with multiple TAGs.

For the operation of UL Tx switching with multiple TAGs, the scheduling restriction is necessary to the uplink transmissions if TA difference is taken into consideration:

For the case of Tx switching with multi-TAG for the two uplink carriers, the UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols that overlaps with the switching period on both the carriers.

Proposal 3：The impact of Tx switching with multiple TAG can be considered as scheduling restriction in RAN4 specification. There is no impact on RAN1 specification is foreseen from RF perspective.

Proposal 4：For the case of Tx switching with multi-TAG for the two uplink carriers, the UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols that overlaps with the switching period on both the carriers.

# 3. Conclusions

This contribution presents discussions on potential RAN4 impact to support the UL Tx switching among 3 bands/4 bands with multiple TAGs.

Proposal 1：UL switching time is the same for single TAG and multiple TAGs.

Proposal 2：The principle of intra-band V2X con-current operation time mask can be followed for UL Tx switching with multiple TAGs.

Proposal 3：The impact of Tx switching with multiple TAG can be considered as scheduling restriction in RAN4 specification. There is no impact on RAN1 specification is foreseen from RF perspective.

Proposal 4：For the case of Tx switching with multi-TAG for the two uplink carriers, the UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols that overlaps with the switching period on both the carriers.

# 4. Reference

[1]	R4-2214249, Email Discussion Summary for [104-e][138] NR_MC_enh, TSG RAN4 Meeting #104e, August. 15 – 26, 2022.

[2]	R4-2215163 WF on UL Tx switching with multiple TAGs, Ericsson, TSG RAN4 Meeting #104e, August. 15 – 26, 2022.

[3]   R4-2214466 LS on UL Tx switching with multiple TAGs, Ericsson, TSG RAN4 Meeting #104e, August. 15 – 26, 2022.