3GPP TGS-RAN WG4 Meeting #104Bis-e	     R4-2216684
Electronic Meeting, October 10 – 19, 2022

Agenda item:	4.3.1

Source:	Intel Corporation

Title:	System parameters maintenance for NR ext. to 71GHz

Document for:	Discussion

# 1	Introduction

In this paper we review system parameter maintenance items for the NR extension to 71GHz work item (WI).

# 2	Maintenance items

The core part of the NR extension to 71GHz WI was concluded in RAN 97e [1] and includes single carrier operation and intra-band contiguous DL CA. For system parameters, the following aspects were agreed to be further discussed during maintenance [2]:

  - NR CA bandwidth class

Agreement:

- No new bandwidth classes are specified for n*100 MHz

- The discussion on the new channel bandwidth classes for FR2-2 is allowed in the maintenance of this WI

  - Configurations for intra-band contiguous CA

Agreement:

- This is related to NR CA bandwidth classes and therefore can also be discussed in maintenance stage

CA bandwidth class

To extend support to 2000 MHz, new CA bandwidth classes within FBG1 were proposed and discussed for FR2-2 during RAN4 #104e [3,4]. As shown below, these cover 4*400 MHz and 5*400 MHz.

| NR CA BW class | Aggregated channel bandwidth | # cont. CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4,5 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| V (Note 4) | 1200 MHz < BWChannel_CA ≤ 1600 MHz | 4 |  |
| W (Note 4) | 1600 MHz < BWChannel_CA ≤ 2000 MHz | 5 |  |
| NOTE 3: In this release of the specification, the minimum requirements for intra-band contiguous CA configurations apply for aggregated channel bandwidths up to 1600 MHz for FR2-1 (this note is not relevant for UE capability parsing by the network). NOTE 4: In this release of the specification, this bandwidth class is applicable only for operating bands within FR2-2. |  |  |  |

Given the agreement to discuss new CA bandwidth classes for FR2-2 in maintenance and need to extend support to 2000 MHz, RAN4 should further discuss the CA bandwidth classes proposed above. Based on the outcome of this discussion, additional configurations for intra-band contiguous CA can be added.

Proposal 1: Further discuss new CA bandwidth classes for FR2-2 in this meeting and consider the proposed classes captured below [3,4]:

| NR CA BW class | Aggregated channel bandwidth | # cont. CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4,5 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| V (Note 4) | 1200 MHz < BWChannel_CA ≤ 1600 MHz | 4 |  |
| W (Note 4) | 1600 MHz < BWChannel_CA ≤ 2000 MHz | 5 |  |
| NOTE 3: In this release of the specification, the minimum requirements for intra-band contiguous CA configurations apply for aggregated channel bandwidths up to 1600 MHz for FR2-1 (this note is not relevant for UE capability parsing by the network). NOTE 4: In this release of the specification, this bandwidth class is applicable only for operating bands within FR2-2. |  |  |  |

# 3	Conclusions

In this contribution we discussed system maintenance items for the NR ext. to 71GHz WI. The following proposal was made:

Proposal 1: Further discuss new CA bandwidth classes for FR2-2 in this meeting and consider the proposed classes captured below [3,4]:

| NR CA BW class | Aggregated channel bandwidth | # cont. CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4,5 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| V (Note 4) | 1200 MHz < BWChannel_CA ≤ 1600 MHz | 4 |  |
| W (Note 4) | 1600 MHz < BWChannel_CA ≤ 2000 MHz | 5 |  |
| NOTE 3: In this release of the specification, the minimum requirements for intra-band contiguous CA configurations apply for aggregated channel bandwidths up to 1600 MHz for FR2-1 (this note is not relevant for UE capability parsing by the network). NOTE 4: In this release of the specification, this bandwidth class is applicable only for operating bands within FR2-2. |  |  |  |

# 4	References

- RP-222655, “SR for Extending current NR operation to 71GHz,” Qualcomm Inc., RAN #97e, September 2022

- R4-2214422, “WF on system parameters for FR2-2,” Intel Corporation, RAN4 #104e, August 2022

- R4-2212845, “Contiguous DL CA system parameters for FR2-2,” Nokia, Nokia Shanghai Bell, RAN4 #104e, August 2022

- R4-2214228, “Email discussion summary for [104-e][110] NR_ext_to_71GHz_Part_1,” Moderator (Intel), RAN4 #104e, August 2022

- 3GPP TS 38.101-2, 17.7.0, September 2022

- R4-2214430, “Draft CR for TS 38.101-2 on system parameter updates for FR2-2,” Nokia, Nokia Shanghai Bell, RAN4 #104e, August 2022