3GPP TSG-RAN WG4 Meeting #104b-e              	R4-2216687

Electrical Meeting, 10th Oct-19th Oct, 2022

Agenda item:	4.4.2.2

Source:	Samsung

Title:	Simulation results for PUCCH demodulation requirement for Rel-17 coverage enhancement

Document for:	Discussion

# 1	Introduction

In the previous RAN4 meeting, it was agreed to introduce PUCCH demodulation requirement with JCE. The detail simulation assumptions for each requirement were discussed to facilitate the requirement definition. The related agreement was captured into the WF [1]

In this contribution, the ideal and impairment results are provided for requirement derivation.

# 2	Initial simulation results

In this section, based on the agreed simulation assumption in the last meeting, the simulation results are provided for simulation alignment purpose.

Table 1. Ideal and impairment simulation results for PUCCH format1 with JCE

| Case | SCS | slot | Format | BW | PRB | Payload | Tx/Rx | Channel | Metric | SNR (ideal) | SNR (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz SCS FDD | 8 | 1 | 5MHz | 1 | 2bits | 1T2R | TDLA30-10 low | ACK missed detection | -10.8 | -8.8 |
|  |  |  |  |  |  |  |  |  | NACK to ACK detection | -10.6 | -8.6 |
| 2 | 30KHz SCS FDD | 8 | 1 | 10MHz | 1 | 2bits | 1T2R | TDLA30-10 low | ACK missed detection | -10.7 | -8.7 |
|  |  |  |  |  |  |  |  |  | NACK to ACK detection | -9.9 | -7.9 |
| 3 | 15KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 | 1 | 5MHz | 1 | 2bits | 1T2R | TDLA30-10 low | ACK missed detection | -6.1 | -4.1 |
|  |  |  |  |  |  |  |  |  | NACK to ACK detection | -5.2 | -3.2 |
| 4 | 30KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 | 1 | 10MHz | 1 | 2bits | 1T2R | TDLA30-10 low | ACK missed detection | -6.3 | -4.3 |
|  |  |  |  |  |  |  |  |  | NACK to ACK detection | -4.9 | -2.9 |
| 5 | 60KHz SCS TDD (DDSUU, S=10D:2G:2U) | 2 | 1 | 50MHz | 1 | 2bits | 1T2R | TDLA30-75 low | ACK missed detection | -6.2 | -4.2 |
|  |  |  |  |  |  |  |  |  | NACK to ACK detection | -5.4 | -3.4 |
| 6 | 120KHz SCS TDD (DDSUU, S=10D:2G:2U) | 2 | 1 | 50MHz | 1 | 2bits | 1T2R | TDLA30-75 low | ACK missed detection | -6.2 | -4.2 |
|  |  |  |  |  |  |  |  |  | NACK to ACK detection | -5.8 | -3.8 |

Table 2. Ideal and impairment simulation results for PUCCH format3 with JCE

| Case | SCS | slot | Format | BW | PRB | Payload | Length | Tx/Rx | Channel | DMRS | SNR (ideal) | SNR (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz SCS FDD | 8 | 3 | 5MHz | 1 | 16bits | 14 | 1T2R | TDLA30-10 low | Without additional DMRS | -6.9 | -4.9 |
|  |  |  |  |  |  |  |  |  |  | With additional DMRS | -7.6 | -5.6 |
| 2 | 30KHz SCS FDD | 8 | 3 | 10MHz | 1 | 16bits | 14 | 1T2R | TDLA30-10 low | Without additional DMRS | -7.0 | -5.0 |
|  |  |  |  |  |  |  |  |  |  | With additional DMRS | -7.6 | -5.6 |
| 3 | 15KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 | 3 | 5MHz | 1 | 16bits | 14 | 1T2R | TDLA30-10 low | Without additional DMRS | -1.7 | 0.3 |
|  |  |  |  |  |  |  |  |  |  | With additional DMRS | -2.3 | -0.3 |
| 4 | 30KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 | 3 | 10MHz | 1 | 16bits | 14 | 1T2R | TDLA30-10 low | Without additional DMRS | -2.1 | -0.1 |
|  |  |  |  |  |  |  |  |  |  | With additional DMRS | -2.6 | -0.6 |
| 5 | 60KHz SCS TDD (DDSUU, S=10D:2G:2U) | 2 | 3 | 50MHz | 1 | 16bits | 14 | 1T2R | TDLA30-75 low | Without additional DMRS | -2.1 | -0.1 |
|  |  |  |  |  |  |  |  |  |  | With additional DMRS | -2.5 | -0.5 |
| 6 | 120KHz SCS TDD (DDSUU, S=10D:2G:2U) | 2 | 3 | 50MHz | 1 | 16bits | 14 | 1T2R | TDLA30-75 low | Without additional DMRS | -2.1 | -0.1 |
|  |  |  |  |  |  |  |  |  |  | With additional DMRS | -2.6 | -0.6 |

# 3	Conclusion

In this contribution, the ideal and impairment simulation results are provided for requirement derivation.

# References

R4-2214315, Email discussion summary for coverage enhancement