3GPP TSG-RAN WG4 Meeting #104b-e              	R4- 2216694

Electrical Meeting, 10th Oct-19th Oct, 2022

Agenda item:	4.3.7.3.1

Source:	Samsung

Title:	Initial simulation results on PUSCH demodulation requirement for Rel-17 71GHz

Document for:	Discussion

# 1	Introduction

In the previous RAN4 meeting, the detail test setup for PUSCH demodulation requirement with 71GHz was agreed. The related agreement was captured into the WF [1].

In this contribution, the ideal and impairment results are provided for requirement derivation.

# 2	Initial simulation results

In this section, based on the agreed simulation assumption in the last meeting, the simulation results for different are provided for simulation alignment purpose

Table 1: ideal and impairment simulation results for PUSCH with CP-OFDM (to be updated)

| Case | SCS | CBW | MCS | Channel model | DMRS | Mapping type | Data length | PTRS | Antenna configuration | SNR (ideal) | SNR (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 120 | 100 | 4 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 1x2 | -2.5 | -0.5 |
| 1 | 120 | 100 | 4 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 2x2 | 1.8 | 3.8 |
| 2 | 120 | 100 | 16 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 1x2 | 8.7 | 10.7 |
| 3 | 120 | 100 | 16 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 2x2 | 16.5 | 18.5 |
| 4 | 120 | 100 | 18 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 1x2 |  |  |
| 5 | 120 | 100 | 18 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 2x2 |  |  |
| 6 | 120 | 100 | 20 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 1x2 | 10.6 | 12.6 |
| 7 | 480 | 400 | 4 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 1x2 | -2.7 | -0.7 |
| 8 | 480 | 400 | 4 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 2x2 | 1.7 | 3.7 |
| 9 | 480 | 400 | 16 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 1x2 | 8.3 | 10.3 |
| 10 | 480 | 400 | 16 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 2x2 | 16.1 | 18.1 |
| 11 | 480 | 400 | 18 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 1x2 |  |  |
| 12 | 480 | 400 | 18 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 2x2 |  |  |
| 13 | 480 | 400 | 20 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 1x2 |  |  |

Table 2: ideal and impairment simulation results for PUSCH with DFT-s-OFDM

| Case | SCS | PRB | MCS | Channel model | DMRS | Mapping type | Data length | PTRS | Antenna configuration | SNR (ideal) | SNR (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 120 | 64 | 4 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 1x2 | -2.2 | -0.2 |
| 1 | 480 | 64 | 4 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 1x2 | -2.3 | 0.3 |

# 3	Conclusion

In this contribution, the ideal and impairment simulation results for 71GHz PUSCH requirement are provided requirement derivation.

# References

R4-2214388, WF on PUSCH demodulation requirements for FR2-2