3GPP TSG-RAN WG4 Meeting #104b-e              	R4-2216695

Electrical Meeting, 10th Oct-19th Oct, 2022

Agenda item:	4.3.7.3.2

Source:	Samsung

Title:	Initial simulation results on PUCCH demodulation requirement for Rel-17 71GHz

Document for:	Discussion

# 1	Introduction

In the previous RAN4 meeting, the detail test setup for PUCCH demodulation requirement with 71GHz was agreed. The related agreement was captured into the WF [1].

In this contribution, the ideal and impairment results are provided for requirement derivation.

# 2	Initial simulation results (To be updated)

In this section, based on the agreed simulation assumption in the last meeting, the simulation results for different are provided for simulation alignment purpose

Table 1. Ideal and impairment simulation results for PUCCH format 0

| Number of Tx antennas | Number of demodulation branches | Propagation condition and correlation matrix | Number of bits | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Hopping | Test metric | SNR (dB) Ideal) | SNR (dB) (impair) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-650 Low | 1 | 120 | 100 | 1 | 16 | Disabled | Prob(ACK miss)<1% | 7.8 | 9.8 |
|  |  |  |  |  |  | 2 | 16 | Enabled | Prob(ACK miss)<1% | -10.2 | -8.2 |
|  |  |  |  | 480 | 400 | 1 | 16 | Disabled | Prob(ACK miss)<1% |  |  |
|  |  |  |  |  |  | 2 | 16 | Enabled | Prob(ACK miss)<1% |  |  |

Table 2. Ideal and impairment simulation results for PUCCH format 1

| Number of Tx antennas | Number of demodulation branches | Propagation condition and correlation matrix | Number of bits | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Hopping | Test metric | SNR (dB) Ideal) | SNR (dB) (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-650 Low | 2 | 120 | 100 | 14 | 1 | Enabled | Prob(NACK to ACK)<0.1% | -4.8 | -2.8 |
|  |  |  |  |  |  |  | 1 | Enabled | Prob(ACK miss)<1% | -6.2 | -4.2 |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(NACK to ACK)<0.1% | -17.4 | -15.4 |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(ACK miss)<1% | -18.6 | -16.6 |

\

Table 3. Ideal and impairment simulation results for PUCCH format 2

| Number of Tx antennas | Number of demodulation branches | Propagation condition and correlation matrix | Number of bits | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Test metric | SNR (dB) Ideal) | SNR (dB) (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-650 Low | 2 | 120 | 100 | 1 | 4 | 4 | Enabled | Prob(NACK to ACK)<0.1% | 3.2 | 5.2 |
|  |  |  |  |  |  | 2 | 9 | 22 | Enabled | UCI BLER <1% | -2.0 | -0.0 |

Table 4. Ideal and impairment simulation results for PUCCH format 3

| Number of Tx antennas | Number of demodulation branches | Propagation condition and correlation matrix | Number of bits | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Additional DMRS | Test metric | SNR (dB) Ideal) | SNR (dB) (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-650 Low | 2 | 120 | 100 | 14 | 1 | 16 | Enabled | Without DMRS | UCI BLER <1% | -1.4 | -0.6 |
|  |  |  |  |  |  | 14 | 1 | 16 | Enabled | With DMRS | UCI BLER <1% | -1.5 | -0.5 |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | Without DMRS | UCI BLER <1% | -0.4 | 1.6 |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | With DMRS | UCI BLER <1% | -0.4 | 1.6 |

Table5. Ideal and impairment simulation results for PUCCH format 4

| Number of Tx antennas | Number of demodulation branches | Propagation condition and correlation matrix | Number of bits | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Additional DMRS | Test metric | SNR (dB) Ideal) | SNR (dB) (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-650 Low | 2 | 120 | 100 | 14 | 1 | 22 | Enabled | Without DMRS | UCI BLER <1% | 0.5 | 2.5 |
|  |  |  |  |  |  | 14 | 1 | 22 | Enabled | With DMRS | UCI BLER <1% | 0.5 | 2.5 |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | Without DMRS | UCI BLER <1% | -11.2 | -9.2 |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | With DMRS | UCI BLER <1% | -11.9 | -9.9 |

# 4	Conclusion

In this contribution, the ideal and impairment simulation results of PUCCH with 71GHz are provided for requirement derivation.

# References

R4-2214500, WF on PUCCH demodulation requirements for FR2-2