3GPP TSG-RAN WG4 # 104bis-e	R4-2216703

Electronic Meeting, October 10 – October 19, 2022

Agenda item:	4.6.5.2.1

Source:	MediaTek Inc.

Title:	Discussion on CQI Requirements for RedCap

Document for:	Discussion

- Introduction

In RAN4#104-e, a way forward on RedCap UE requirements was agreed [1]. In this contribution, we provide simulation results and views on reaming issues for CQI requirements.

- Discussion

It was agreed in RAN4#104-e to use the same CQI feedback scheduling pattern and requirements for both FD-FDD and HD-FDD duplex mode.

Agreement: CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD - Configure the following parameters for CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD: - CSI-RS periodicity and offset: 10/1 - CSI-Report periodicity and offset: 10/9 - CQI/RI/PMI delay: [14ms] - Interested companies are encouraged to evaluate the performance difference between CQI delay 14ms and 10ms in RAN4#104-bis-e. If significant performance degradation is observed compared with CQI delay 10ms, RAN4 will revisit the CQI/RI/PMI delay - If CQI delay 10ms is applied, consider CSI periodicity and offset is set to 10/5.

However, according to [2], there is a TP ratio difference for different CQI/RI/PMI delay. We proivde our results in Table 1 and it can be shown that there is indeed a TP ratio differenece between CQI/RI/PMI delay 10ms and 14ms. Also, with CQI delay 14ms, the TP ratio are less than 1.05 for 15/16 dB. Therefore, we propose to use the follwing CQI feedback scheduling pattern:

- CSI-RS periodicity and offset: 10/5

- CSI-Report periodicity and offset: 10/9

- CQI/RI/PMI delay: 10ms

Table 1. CQI simulation results for 2T1R, HD-FDD in fading condition

|  | CQI delay=10ms |  |  | CQI delay=14m |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR | Percentage of reported CQI Index not in {medCQI-1, medCQI+1} | TP ratio | BLER | Percentage of reported CQI Index not in {medCQI-1, medCQI+1} | TP ratio | BLER |
| 9 | 0.52 | 1.11 | 0.06 | 0.53 | 1.06 | 0.08 |
| 10 | 0.53 | 1.11 | 0.07 | 0.54 | 1.10 | 0.09 |
| 15 | 0.57 | 1.07 | 0.10 | 0.57 | 1.02 | 0.13 |
| 16 | 0.56 | 1.08 | 0.11 | 0.56 | 1.04 | 0.13 |

Proposal 1: For HD-FDD in static/fading condition, use the following configurations for CQI reporting tests:

- CSI-RS periodicity and offset: 10/5

- CSI-Report periodicity and offset: 10/9

- CQI/RI/PMI delay: 10ms

- Conclusion

In this contribution, we provide simulation results for RedCap CQI requirements with different CQI delays and also the following proposal for configuration.

Proposal 1: For HD-FDD in static/fading condition, use the following configurations for CQI reporting tests:

- CSI-RS periodicity and offset: 10/5

- CSI-Report periodicity and offset: 10/9

- CQI/RI/PMI delay: 10ms

- Reference

- R4-2210672, “WF on RedCap demodulation and CQI reporting requirements”, Ericsson

- R4-2212894, “CQI reporting requirements for RedCap”, Ericsson