3GPP TSG-RAN WG4 Meeting #104-bis-e                                                       R4-2216709

Electronic Meeting, October 10th – 19th, 2022

Source:	Samsung

Title:	Enhanced NR Support for FR2 HST on RF requirements for intra-band CA

Agenda item:			6.12.2

Document for:	Discussion

# 1. Introduction

In RAN#95e meeting, the Rel-18 RAN4-led work item on enhanced NR support for high speed train scenario in FR2 has been approved as [1]. This Rel-18 can be regarded as the continuous enhancement over the Rel-17 feature of NR support of FR2 HST, in which Rel-17 WI RAN4 has focused on train roof-mounted high-power devices for NR SA single carrier scenario in FR2, by studying the FR2 HST deployment scenario and specifying the channel modelling, RF, RRM and demodulation requirements for FR2 HST.

As one of the objectives of Rel-18 enhanced NR support for FR2 HST, the intra-band carrier aggregation (CA) operation, which is of interests to operators but not yet covered in Rel-17 work item scope, is provided in WID [2] as follows.

- Specify the RF requirements for intra-band carrier aggregation (CA) scenario, and investigate and specify the RRM requirements for intra-band carrier aggregation (CA) scenario [RAN4]

In this contribution, we would like to share our initial viewpoints to trigger the discussion on the RF requirements for intra-band CA for FR2 HST enhancement.

# 2 Discussion

## 2.1 Rel-17 NR Support for FR2 HST: RF requirement overview

This Rel-17 WI on NR support for high speed train (HST) scenario in Frequency Range 2 (FR2) is targeted to specify NR UE RF requirements, RRM requirements and BS/UE performance requirements for train roof-mounted high-power devices in HST scenario with speed up to 350km/h and applicable frequency is up to 30GHz.

In Rel-17 work item, for UE RF requirement, FR2 power class 6 for band n257, n258 and n261 is newly introduced, which is corresponding to the UE type of high speed train roof-mounted UE. Relevant UE TX and RX RF requirements for power class 6 are specified, in which the TX and EIS spherical coverage requirements are defined over the newly introduced spherical coverage evaluation areas. For UE beam correspondence requirement, the applicability rule and requirement side conditions are specified.

In the approved CR [4] to introduce UE RF requirement for FR2 Power Class 6, the following clauses are impacted or newly introduced, in which the changes are summarized and their relationship with PC5 are highlighted:

Table-1: Summarized Rel-17 RF Impact by Introducing FR2 Power Class 6 for HST

| Impacted Clause | Description | Relationship with PC5 |
| --- | --- | --- |
| 6.2.1.0 General (UE maximum output power) | To introduce PC6 | New change |
| 6.2.1.6 UE maximum output power for power class 6 | Peak EIRP, EIRP Spherical coverage (new framework), other limits | Newly defined framework, and requirement |
| 6.2.2.6 UE maximum output power reduction for power class 6 | MPR | Same as PC5, but PC5 also refer to PC3 |
| 6.2.3.3.6 A-MPR for NS_202 for power class 6 | A-MPR for NS_202 | Same as PC5, but PC5 also refer to PC3 |
| 6.2.3.4.6 A-MPR for NS_203 for power class 6 | A-MPR for NS_203 | Same as PC5, but PC5 also refer to PC3 |
| 6.2D.1.6 UE maximum output power for UL MIMO for power class 6 | UL-MIMO maximum power | Newly defined to refer to PC6 single TX, different from PC5 |
| 6.2D.2.6 UE maximum output power reduction for modulation / channel bandwidth for UL MIMO for power class 6 | UL-MIMO MPR | Newly defined to refer to PC6 UL-MIMO MOP, different from PC5 |
| 6.2D.3.6 UE maximum output power reduction with additional requirements for UL MIMO for power class 6 | UL-MIMO A-MPR | Newly defined to refer to PC6 UL-MIMO MOP, different from PC5 |
| 6.3D.1.3 Minimum output power for UL MIMO for power class 5 and 6 | Minimum output power for UL-MIMO | Same as PC5, and added into PC5 clause |
| 6.3.1.3 Minimum output power for power class 5 and 6 | Minimum output power | Same as PC5, and added into PC5 clause |
| 6.4.2.2.7 Carrier leakage for power class 6 | Carrier leakage | Same as PC5 |
| 6.4.2.3.7 In-band emissions for power class 6 | In-band emissions | Same as PC5 |
| 6.6.7 Beam correspondence for power class 6 | Beam correspondence requirement | Newly defined BC requirement |
| 7.3.2.6 Reference sensitivity power level for power class 6 | Reference sensitivity | Newly defined Refsens requirement |
| 7.3.4.6 EIS spherical coverage for power class 6 | EIS spherical coverage | Newly defined framework, and requirement |

Observation 1: Rel-17 RF requirement impact by Introducing FR2 Power Class 6 for HST is summarized in Table-1.

## 2.2 Expected RF Impact for Intra-band CA

As provided in WID, UE RF requirement for intra-band carrier aggregation (CA) scenario is intended to be specified. Based on our understanding, we see no stronge reason to exclude either intra-band contiguous or non-contiguous CA for both UL and DL.

Proposal 1: Intra-band contiguous and non-contiguous CA for both UL and DL are in the scope of Rel-18 FR2 HST enhancement work item.

By enabling the feature of intra-band CA for FR2 power class 6 UE, at least the changes on the following clauses are expected.

Proposal 2: The expected changes on intra-band CA relevant clauses by Rel-18 feature for FR2 HST enhancement are summarized and provided in Table-2:

Table-2: Expected changes on intra-band CA relevant clauses by Rel-18 feature for FR2 HST enhancement

| Clause in TS38.101-2 |  | Expected Changes or Not |
| --- | --- | --- |
| 6.2A Transmitter power for CA |  |  |
|  | 6.2A.1 UE maximum output power for CA | No changes expected because only considering intra-band CA |
|  | 6.2A.2 UE maximum output power reduction for CA | New sub-clause to be added for PC6, the same requirement as PC5 expected |
|  | 6.2A.3 UE maximum output power with additional requirements for CA | New sub-clause for CA_NS_202 and CA_NS_203 for PC6, the same requirement as PC5 expected |
|  | 6.2A.4.1 Configured transmitted power for intra-band UL CA | No changes expected because the configured power requirement is PC-agnostic. |
| 6.3A Output power dynamics for CA |  |  |
|  | 6.3A.1 Minimum output power for CA | New sub-clause to be added for PC6, the same requirement as PC5 expected but for Band n257, n258 and n261 |
|  | 6.3A.2/3/4 other clauses for output power dynamics for CA | No changes expected because the relevant requirement is PC-agnostic. |
| 6.4A Transmit signal quality for CA |  |  |
|  | 6.4A.2.2 Carrier leakage | New sub-clause to be added for PC6, the same requirement as PC5 expected |
|  | 6.4A.2.3 Inband emissions | New sub-clause to be added for PC6, the same requirement as PC5 expected |
|  | Other sub-clauses under 6.4A | No changes expected because the relevant requirement is PC-agnostic. |
| 6.5A Output RF spectrum emissions for CA |  | No changes expected because the output RF spectrum emission requirement is PC-agnostic. |
| 6.6A Beam correspondence for CA |  | No changes expected because the CA beam correspondence requirement is PC-agnostic. |
| 7.3A Reference sensitivity for DL CA |  | No changes expected because the DL CA Reference sensitivity requirement is PC-agnostic. |
| 7.4A Maximum input level for DL CA |  | No changes expected because the DL CA maximum input level requirement is PC-agnostic. |
| 7.5A Adjacent channel selectivity for DL CA |  | No changes expected because the DL CA ACS requirement is PC-agnostic. |
| 7.6A Blocking characteristics for DL CA |  | No changes expected because the DL CA blocking characteristics requirement is PC-agnostic. |

# 3. Conclusion

In this contribution, we provided our viewpoints on the RF requirements for intra-band CA for FR2 HST enhancement for this work item, accordingly the following observations and proposals are obtained:

Observation 1: Rel-17 RF requirement impact by Introducing FR2 Power Class 6 for HST is summarized in Table-1.

Proposal 1: Intra-band contiguous and non-contiguous CA for both UL and DL are in the scope of Rel-18 FR2 HST enhancement work item.

Proposal 2: The expected changes on intra-band CA relevant clauses by Rel-18 feature for FR2 HST enhancement are summarized and provided in Table-2:

Table-2: Expected changes on intra-band CA relevant clauses by Rel-18 feature for FR2 HST enhancement

| Clause in TS38.101-2 |  | Expected Changes or Not |
| --- | --- | --- |
| 6.2A Transmitter power for CA |  |  |
|  | 6.2A.1 UE maximum output power for CA | No changes expected because only considering intra-band CA |
|  | 6.2A.2 UE maximum output power reduction for CA | New sub-clause to be added for PC6, the same requirement as PC5 expected |
|  | 6.2A.3 UE maximum output power with additional requirements for CA | New sub-clause for CA_NS_202 and CA_NS_203 for PC6, the same requirement as PC5 expected |
|  | 6.2A.4.1 Configured transmitted power for intra-band UL CA | No changes expected because the configured power requirement is PC-agnostic. |
| 6.3A Output power dynamics for CA |  |  |
|  | 6.3A.1 Minimum output power for CA | New sub-clause to be added for PC6, the same requirement as PC5 expected but for Band n257, n258 and n261 |
|  | 6.3A.2/3/4 other clauses for output power dynamics for CA | No changes expected because the relevant requirement is PC-agnostic. |
| 6.4A Transmit signal quality for CA |  |  |
|  | 6.4A.2.2 Carrier leakage | New sub-clause to be added for PC6, the same requirement as PC5 expected |
|  | 6.4A.2.3 Inband emissions | New sub-clause to be added for PC6, the same requirement as PC5 expected |
|  | Other sub-clauses under 6.4A | No changes expected because the relevant requirement is PC-agnostic. |
| 6.5A Output RF spectrum emissions for CA |  | No changes expected because the output RF spectrum emission requirement is PC-agnostic. |
| 6.6A Beam correspondence for CA |  | No changes expected because the CA beam correspondence requirement is PC-agnostic. |
| 7.3A Reference sensitivity for DL CA |  | No changes expected because the DL CA Reference sensitivity requirement is PC-agnostic. |
| 7.4A Maximum input level for DL CA |  | No changes expected because the DL CA maximum input level requirement is PC-agnostic. |
| 7.5A Adjacent channel selectivity for DL CA |  | No changes expected because the DL CA ACS requirement is PC-agnostic. |
| 7.6A Blocking characteristics for DL CA |  | No changes expected because the DL CA blocking characteristics requirement is PC-agnostic. |

# 4. Reference

[1] RP-220985, “New WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung.

[2]	RP-222272, “Revised WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung.

[3] RP-220316, “WI summary for NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung.

[4]	R4-2206521, CR to introduce UE RF requirement for FR2 Power Class 6, Samsung.