3GPP TSG-RAN WG4 Meeting #104-bis-e                                                       R4-2216714

Electronic Meeting, October 10th – 19th, 2022

Source:	Samsung

Title:	Further Discussion on Switching Time and other RF Aspects for UL Tx Switching Across 3 or 4 Bands

Agenda item:			6.19.2

Document for:	Discussion

# 1. Introduction

In RAN#94e meeting, the Rel-18 RAN1-led work item on multi-carrier enhancement has been approved as [1], which has been further revised into [2], in which there are two major objectives focusing on (1) a solution for multi-cell PUSCH/PDSCH scheduling with a single DCI and (2) enhancements for multi-carrier UL operation, respectively.

Specifically, the detailed objectives for the above-mentioned (2) enhancements for multi-carrier UL operation are provided in WID [2] as follows:

2. Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4] - UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4) - UE capability and RRC configuration related signalling (RAN2) - Note: strive for RAN1/2 design agnostic with the number of bands, i.e., common design between 3 and 4 bands - Note: no additional TAG is introduced for UL transmission on a carrier without corresponding DL carrier - Note: this objective does not target to extend the SUL framework to support more than 1 SUL for 1 NUL - Note: Extension of TX switching for 2 bands to multiple TAG configurations is included in the scope. The work is limited to RAN4. - Switching time and other RF aspects, and RRM requirements for above UL Tx switching schemes across up to 3 or 4 bands (RAN4) - Note: Prioritize UL Tx switching across up to 3 bands is to be addressed first and then that for up to 4 bands can also be addressed

Starting from last meeting, RAN1 has started the discussion on UL Tx switching across 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs at RAN1#109-e meeting (May, 2022). By replying RAN1 LS (R1-2205502), RAN4 has extensively discussed various technical aspects with the reply LS approved in [4].

In this contribution, we would like to share our initial viewpoints to trigger the discussion on this work item, especially providing our analysis relevant to two questions in RAN1 LS.

# 2 Discussion

In the approved WF on UL Tx switching across 3/4 bands with single TAG [5], the following aspects remain to be further considered in RAN4:

#### Issue 2-1-3: Exact value of Tx switching period - Further discuss the two options in the next meeting: - Option 1: Reuse the same switching period for each band pair as UE reported in Rel-16/17, i.e., UE does not need to report new or larger switching period per band pair for Rel-18 Tx switching. - Option 2: Although the set of switching periods is the same as in Rel-16/17, a different value can be reported for each band pair in Rel-18 Tx switching with 3/4 bands configured. #### Issue 2-2-2: Impact on the band with the number of Tx chain unchanged due to switching GTW Agreement: - As baseline UE assumption, neither of Tx chains is expected to be used for transmission on band C during the switching period. - RAN4 will further discuss optional advanced features to allow the other Tx chain can be expected to be used for transmission on band C during the switching period as advanced/optional UE assumption.

## 2.1 Exact value of Tx switching period

Although the set of values for TX switching period, RAN4 has agreed to reuse the same set of values as in Rel-16/17, i.e., {35 us, 140 us, 210 us} for UL CA and SUL. For UE architecture discussed in Rel-16 TX switching specifically, different periods of TX switching time may be desired based on whether:

- TX PA is commonly used between two UL carriers:

if yes, processing time is desired to reload PA from one carrier to another.

- PLL is commonly used between two UL carriers:

if yes, processing time is desired to reload PLL from one frequency to another.

For TX switching across 3 or 4 bands, if dedicated PA and PLL are prepared for the UL carrier in the switch-to band, we see no extra switching period is required, i.e., 35us is usually regarded as the switching time required if dedicated PA and PLL are prepared. If no dedicated PA and PLL are prepared and reload processing time is needed, we expect additional switching period would be of necessity since UE complexity is expected to be increased to handle additional bands in RF switches, for TX switching across 3 or 4 bands in comparison to 2 bands. Considering this, between the two options given in the approved WF [5], Option 2 is more preferred, which provide more flexibility to different kinds of UE implementation, while the required switching period across 3/4 bands is still known to gNB by using the Rel-18 reporting.

Proposal 1: RAN4 adopt Option 2 for the exact value of TX switching period across 3/4 bands:

- Option 2: Although the set of switching periods is the same as in Rel-16/17, a different value can be reported for each band pair in Rel-18 Tx switching with 3/4 bands configured.

## 2.2 Impact on the band with the number of Tx chain unchanged due to switching

In last RAN4 meeting, RAN4 has discussed the UE assumption/behavior considering the following two cases, with the conclusion on the impact on the unchanged TX chain [4]:

- Case 1: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on either band A or band B. For Case 1, RAN4 agreed that neither of Tx chains is expected to be used for transmission during the switching period. - Case 2: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on a different band (named “band C”). For Case 2, RAN4 agreed that, as baseline UE assumption, neither of Tx chains is expected to be used for transmission on band C during the switching period.

For the Case 2, although it is regarded as baseline that neither of Tx chains is expected to be used for transmission on band C during the switching period, it is regarded as the pessimistic assumption by many companies. Therefore, “RAN4 will further discuss optional advanced features to allow the other Tx chain can be expected to be used for transmission on band C during the switching period as advanced/optional UE assumption” as indicated in the approved WF [5].

In this Case 2, TX switching on the 1st TX chain (from Band A to Band B) may or may not incur interruption to 2nd TX chain (continuously remaining on Band C). Whether or not the interruption occurs depends on whether or not the following factors exist:

- the coupling effects of reloaded PLL to the 2nd TX chain;

- retuning of other RF parts or BB clock base need to be modified on the 2nd TX chain

- DAC in the 2nd TX chain have to be halted, because TX switched DAC sampling rate is changed.

Considering we may not want to preclude any advanced implementation without impact on TX transmission on the 2nd TX chain, we have the following proposal:

Proposal 2: For Case 2 (One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on a different band (named “band C”).)

      - Advanced UE implementation can allow the other Tx chain can be expected to be used for transmission on band C during the switching period as advanced/optional UE assumption

      - Applying the baseline or the advanced implementation assumption depends on the particular UE implementation for the given band pair of each band combination.

# 3. Conclusion

In this contribution, we provided our viewpoints on the remaining RF aspects for this work item, accordingly the following observations and proposals are obtained:

Proposal 1: RAN4 adopt Option 2 for the exact value of TX switching period across 3/4 bands:

- Option 2: Although the set of switching periods is the same as in Rel-16/17, a different value can be reported for each band pair in Rel-18 Tx switching with 3/4 bands configured.

Proposal 2: For Case 2 (One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on a different band (named “band C”).)

      - Advanced UE implementation can allow the other Tx chain can be expected to be used for transmission on band C during the switching period as advanced/optional UE assumption

      - Applying the baseline or the advanced implementation assumption depends on the particular UE implementation for the given band pair of each band combination.

# 4. Reference

[1] RP-213577, “New WID on Multi-carrier enhancements”, NTT DoCoMo, Inc.

[2]	RP-221435, “New WID on Multi-carrier enhancements”, NTT DoCoMo, Inc.

[3] R4-2211508 (R1-2205502), “LS on UL Tx switching across 3 or 4 bands in Rel-18”, NTT DoCoMo, Inc.

[4] R4-2214464, “Reply LS on UL Tx switching across 3 or 4 bands”, RAN4.

[5] R4-2214463, “WF on UL Tx switching across 3/4 bands with single TAG”, China Telecom.

[6] R4-2215163, “WF on UL Tx switching with multiple TAG”, Ericsson.