3GPP TSG-RAN WG4 Meeting #104bise                                             R4-2216718

Electronic Meeting, 10th– 19th Oct, 2022

Agenda Item:    6.17.2.2.2

Source:         Samsung

Title:            Discussion on UE aspect for SBFD operation

Document for:   Approval

# Introduction

In RAN4#104e RAN4 informed RAN1 regarding preliminary agreement on RAN4 feasibility study and RF requirement impact for SBFD operation [1].

## From UE perspective Agreement: - Using existing UE RF requirements to estimate UE performance and if needed extrapolating them for system level studies

This contribution will provide further assessment on SBFD operation from UE perspective based on above agreement.

# Co-channel inter-subband UE-UE CLI modelling

There are several candidates for co-channel inter-subband UE-UE CLI case as agreed in [2].

Candidate considerations for UE-UE CLI model: - TX model can refer to existing UE requirement in TS38.101-1 and TS38.101-2 - In band emission as starting point - FFS is not precluded for other candidates such as ACLR - RX model can refer to existing UE requirement in TS38.101-1 and TS38.101-2 - Maximum input power as threshold based on above specification - FFS is not precluded for other candidates such as ACS, ICI, and estimated RX model based on legacy UE.

For co-channel inter-subband UE-UE CLI model, according to existing UE RF requirement there are two candidates: one is IBE based model with no receiver in-channel selectivity considered, and the other one is ACIR based model. Concern was shared in previous meeting that the ACIR model could not be applied since this is in-channel case. But there are also clarifications as that the applicability may be dependent on how the subband of SBFD configured. It may be premature to conclude this since this is related to PHY layer design. But from RAN4 perspective, one solution to resolve the issue can be to choose the worst case as starting point for interference level calculation. Hence, we suggest to compare the leakage ratio of IBE based model and ACIR base model first. For in-band emission, the corresponding requirement tables in TS38.101-1 and TS38.101-2 are pasted as below

Table1: Requirements for in-band emissions in TS38.101-1

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25 | 0 dBm ≤ Output power ≤ 10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power < 0 dBm |  |
|  |  | -10 | -40 dBm ≤ Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier leakage frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed either in the RB containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see clause 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see clause 5.3). NOTE 8: EVM is the limit specified in Table 6.4.2.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: For almost contiguous allocations defined in clause 6.2.2, LCRB = NRB_alloc + NRB_gap with no in-gap emission requirement. |  |  |  |  |

Table 6.4.2.3.4-1: Requirements for in-band emissions for power class 3 in TS38.101-2

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 | Output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 0 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 0 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed in the RBs containing the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Clause 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Clause 5.3). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB = 1 or RB = -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. |  |  |  |  |

If the UL transmission is allocated in middle of the channel as one of the candidate SBFD configurations, the leakage ratio in each non-allocated RB is 30dB and 25dB for FR1 and FR2 respectively according to IBE based model. Under such assumption, corresponding interference signal level falling in DL RBs received by victim UE should be derived by transmitted power, path loss between aggressor and victim and related leakage ratio according to IBE requirement. And then the impact on SINR of victim UE receiver could be calculated.

The IBE based model and ACIR based model are compared in table below. It seems ACIR based model will be concluded with more pessimistic result from co-channel co-existence perspective.

Table 3: comparison between IBE based model and ACIR based model

| Frequency range | UE-UE IBE model | UE-UE ACIR model |
| --- | --- | --- |
| FR1 (4GHz as example) | 30dBc | 28dBc |
| FR2 (30GHz as example) | 25dBc | 16.5dBc |

Observation 1: It seems ACIR based model will be concluded with more pessimistic result from co-channel co-existence perspective.

To facilitate the evaluation on co-channel inter-subband SLS, we suggest to take the worse model based on UE-UE ACIR. But the maximum input level, which plays the same role as in-band blocking agreed as check point for BS self-interference, could be applied as checking point for UE co-channel inter-subband CLI model. To summary, our proposals are as below:

Proposal 1: For co-channel inter-subband UE-UE CLI modelling, it’s suggested to agree maximum input level as one checking point for receiver saturation of victim UE.

Proposal 2: For co-channel inter-subband UE-UE CLI modelling, it’s suggested to apply below simplified model based on existing RF requirement as starting point.

  - InterferenceUE-UE_co =PtxaggressordBm-PL+antenna gain-28(dBc) for FR1

  - InterferenceUE-UE_co =PtxaggressordBm-PL+antenna gain-16.5(dBc) for FR2

# Adjacent channel UE-UE CLI modelling

Agreement on feasibility and how to model UE-UE CLI modelling considering unwanted emission and receiver selectivity: - Model as starting point: UE ACLR based model on TX and UE ACS based model on RX which is the same ACIR model as Rel-16 CLI study. - FFS on below model - UE ACLR model with 2step size(FR1 example: ACLR1/2=28/33dB) on TX - UE ACS based model on RX if blocker is smaller than maximum input level of UE, and additional SNR degradation at the victim receiver due to receiver gain backoff - FFS on how the per-sub-band/RB aspect is characterised. Other aspect is also not precluded

In case of adjacent channel UE-UE CLI modelling, since we have already had legacy UE-UE ACIR based model as starting point. We would like to see more evaluation result before further improvement on basic CLI model. For finer step size on UE ACLR mode we tend to agree the motivation considering subband will be configured for SBFD operation. Hence we are open for the discussion on this direction. However, regarding the check point of maximum input level, it is not convinced yet the necessity since this is never considered in legacy study for adjacent channel co-existence.

Proposal 3: To align with legacy CLI and co-existence study, no receiver check point needed for adjacent channel UE-UE CLI model.

# Conclusion

For Co-channel inter-subband UE-UE CLI model, we compared two candidate models and suggest a merged one with stricter model from interference level calculation perspective as below:

Proposal 1: For co-channel inter-subband UE-UE CLI modelling, it’s suggested to agree maximum input level as one checking point for receiver saturation of victim UE.

Proposal 2: For co-channel inter-subband UE-UE CLI modelling, it’s suggested to apply below simplified model based on existing RF requirement as starting point.

  - InterferenceUE-UE_co =PtxaggressordBm-PL+antenna gain-28(dBc) for FR1

  - InterferenceUE-UE_co =PtxaggressordBm-PL+antenna gain-16.5(dBc) for FR2

For adjacent channel UE-UE CLI model, we have below proposal.

Proposal 3: To align with legacy CLI study, no receiver check point needed for adjacent channel UE-UE CLI model.

# Reference

[1] R4-2214376, Reply LS on interference modelling for duplex evolution

[2] R4-2214377, WF on feasibility study from RF perspective

[3] R4-2212486, Discussion on interference modeling requested by RAN1

[4] TS38.101-1: NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

[5] TS38.101-2: NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone