3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216725

Online Meeting, Oct 10 – 19, 2022

Agenda item:	6.18.2

Source: 	Qualcomm Incorporated

Title: 	Study of PRS/SRS bandwidth aggregation – RF aspects

Document for:	Discussion

# Introduction

RAN4 started the discussion of RF aspects of the Rel-18 study of expanded and improved NR positioning in RAN4#104-e [1]. Regarding the feasibility study of PRS/SRS bandwidth aggregation, RAN4 reached some initial agreements captured in a WF [2].

## Intra-band CA scenario

Agreement:

- Intra-band contiguous CA scenario will be prioritized in study.

## Initial conclusion on feasibility

Agreement:

- PRS/SRS bandwidth aggregation for intra-band contiguous carrier is feasible for single chain Tx/Rx architectures

## Scope of study based on PRS/SRS bandwidth aggregation

Agreement:

- Deprioritize power imbalance discussion

Way forward: Further discuss the following in the next RAN4 meeting:

- RF architecture – can we focus on a single RF architecture (i.e., single Tx/Rx chain), align on target architectures

- Studying RF impairment model (timing/group delay/frequency/phase) first to assess performance and accuracy gain with realistic impairments

- Studying achievable accuracy gain when TAE is within specified requirement for intra-band contiguous CA

- Notifying RAN1 of the UE transmit power limitation due to potential prioritization

It was agreed that the study of PRS/SRS aggregation will focus on the intra-band contiguous PRS/SRS scenario. Additionally, RAN4 concluded that it is feasible to support intra-band contiguous PRS/SRS aggregation for single-chain Tx/Rx architectures.

In this contribution, we provide our views on the initial agreements/conclusion and about further aspects that RAN4 should investigate as part of this study.

# Discussion

## Views on initial conclusion

RAN4 concluded in the previous meeting that it is feasible to support intra-band contiguous PRS/SRS aggregation for single-chain Tx/Rx architectures. For DL aggregation this means that when the contiguous PRS signals are transmitted using a single Tx chain at the TRP and received using a single Rx chain at the UE, RAN4 understands there are no major impediments to coherently aggregating the signals at the UE. Similarly for UL aggregation, when the contiguous SRS signals are transmitted using a single Tx chain at the UE and received using a single Rx chain at the TRP, RAN4 understands there are no major impediments to coherently aggregating the signals at the TRP.

For the above scenarios where single Tx/Rx chain are assumed, no adverse effects due to relative timing errors, phase errors nor frequency errors between signals are expected. At least for aggregation of simultaneous PRS/SRS transmissions, RAN4 does not need to study those impairments for single-chain Tx/Rx architectures.

Observation 1: RAN4 has concluded that PRS/SRS bandwidth aggregation for intra-band contiguous carrier is feasible for single-chain Tx/Rx architectures. No adverse effects on measurement accuracy are expected due to relative timing errors, phase errors nor frequency errors between signals/carriers for such architectures.

Proposal 1: For aggregation of simultaneous PRS/SRS transmissions, no further study of relative timing errors, phase errors nor frequency errors between signals/carriers is required for single-chain Tx/Rx architectures.

One relevant aspect mentioned in the previous meeting by several companies is group delay calibration. We agree that group delay calibration accuracy does affect measurement accuracy and it may be studied further, even for single-chain architectures. However, this issue is not entirely new and specific to PRS/SRS aggregation. In Rel-16 NR positioning, RAN4 accounted for the effect of group delay errors by adding margins to the RSTD and UE Rx-Tx accuracy requirements [5].

In our prior contribution, we provided our initial views on how to account for group delay errors in the context of PRS/SRS aggregation [4]. We understand that RAN4 can continue studying this issue but we do not expect RAN4 agreements on e.g. group delay margin values as part of this Rel-18 study.

Observation 2: RAN4 can further study the impact of group delay errors in the context of PRS/SRS aggregation.

Proposal 2: No RAN4 agreements on group delay margin values are expected as part of the Rel-18 study on PRS/SRS bandwidth aggregation.

## Issues for further study

In light of the initial (positive) conclusion about the feasibility of PRS/SRS carrier aggregation for single-chain Tx/Rx architectures, RAN4 should discuss which issues to prioritize for the rest of the Rel-18 study. Several topics were mentioned during the discussion in RAN4#104-e [2, 3]. In our view, RAN4 should discuss at least the following:

- Additional RF architectures to be studied.

- Relevant RF impairments for the target RF architectures, including relative timing errors, phase errors, and frequency errors.

- Methodology for evaluating performance gains, including impairment models.

Regarding additional RF architectures, our view is that RAN4 should study modular architectures consisting of separate Tx/Rx chains per layer/carrier (e.g. Figure 1 and  Figure 2). This type of architecture is typical in commercial devices and infrastructure, so it would make sense to include it as part of this study.

Figure 1: Separate Tx chains (baseband + RF)

Figure 2: Separate Rx chains (baseband + RF)

For modular architectures, RAN4 should study the effects of relative timing errors, phase errors and frequency errors between layers/carriers. Our assumption is that devices and infrastructure nodes that feature modular transceiver architectures would need to develop solutions to mitigate/minimize these errors in order to enable PRS/SRS bandwidth aggregation. We understand that RAN4 may not have enough time to fully develop detailed solutions as part of this study item; they would be left up to implementation. The primary goal of this study would be to determine what is the tolerable range of errors by evaluating their impact on performance.

Proposal 3: RAN4 studies modular architectures featuring separate Tx/Rx chains per layer/carrier as part of the Rel-18 study on PRS/SRS bandwidth aggregation.

Proposal 4: For modular architectures featuring separate Tx/Rx chains per layer/carrier, RAN4 will study the effects of relative timing errors, phase errors and frequency errors between layers/carriers.

Proposal 5: RAN4 should discuss the methodology to evaluate the impact of relative timing errors, phase errors and frequency errors between layers/carriers, including the following:

- Scenarios to be simulated

  - System level simulations vs link level simulations

  - FR, SCS, RS BW, number of layers/carriers (2, 4?)

  - Propagation channels

  - Es/Iot side-condition, number of samples

  - Measurements: DL-TDOA or Rx-Tx time difference

- Metric

  - Measurement accuracy with coherent processing vs. single layer/carrier

- Impairment models

  - Relative timing error – uniformly distributed between [-t, t] nsec

  - Relative phase error – uniformly distributed between [-theta, theta] degrees

  - Frequency error – uniformly distributed between [-d, d] ppm

  - How to generate errors

    - Draw a random realization for each error independently per measurement occasion/sample.

    - Correlated/dependent timing and phase errors, frequency errors independent from timing and phase errors,

    - Correlated/dependent timing, phase and frequency errors.

Regarding the above proposal, it may be better to discuss points 1 and 2 in the RRM session and point 3 in the RF session.

# Conclusions

Observation 1: RAN4 has concluded that PRS/SRS bandwidth aggregation for intra-band contiguous carrier is feasible for single-chain Tx/Rx architectures. No adverse effects on measurement accuracy are expected due to relative timing errors, phase errors nor frequency errors between signals/carriers for such architectures.

Proposal 1: For aggregation of simultaneous PRS/SRS transmissions, no further study of relative timing errors, phase errors nor frequency errors between signals/carriers is required for single-chain Tx/Rx architectures.

Observation 2: RAN4 can further study the impact of group delay errors in the context of PRS/SRS aggregation.

Proposal 2: No RAN4 agreements on group delay margin values are expected as part of the Rel-18 study on PRS/SRS bandwidth aggregation.

Proposal 3: RAN4 studies modular architectures featuring separate Tx/Rx chains per layer/carrier as part of the Rel-18 study on PRS/SRS bandwidth aggregation.

Proposal 4: For modular architectures featuring separate Tx/Rx chains per layer/carrier, RAN4 will study the effects of relative timing errors, phase errors and frequency errors between layers/carriers.

Proposal 5: RAN4 should discuss the methodology to evaluate the impact of relative timing errors, phase errors and frequency errors between layers/carriers, including the following:

- Scenarios to be simulated

  - System level simulations vs link level simulations

  - FR, SCS, RS BW, number of layers/carriers (2, 4?)

  - Propagation channels

  - Es/Iot side-condition, number of samples

  - Measurements: DL-TDOA or Rx-Tx time difference

- Metric

  - Measurement accuracy with coherent processing vs. single layer/carrier

- Impairment models

  - Relative timing error – uniformly distributed between [-t, t] nsec

  - Relative phase error – uniformly distributed between [-theta, theta] degrees

  - Frequency error – uniformly distributed between [-d, d] ppm

  - How to generate errors

    - Draw a random realization for each error independently per measurement occasion/sample.

    - Correlated/dependent timing and phase errors, frequency errors independent from timing and phase errors,

    - Correlated/dependent timing, phase and frequency errors.

# References

[1] RP-213588, Study on expanded and improved NR positioning, RAN#94-e

[2] R4-2214462, WF on expanded and improved NR positioning study, RAN4#104-e

[3] R4-2214248, Email discussion summary for [104-e][137] FS_NR_pos_UERF, RAN4#104-e

[4] R4-2212210, On the feasibility of PRS/SRS bandwidth aggregation for enhanced positioning accuracy, Qualcomm Inc., RAN4#104-e

[5] TS 38.133, v16.13.0