3GPP TSG-RAN WG4 Meeting # 104bis-e 	R4-2216738

Electronic Meeting, October 10th - 19th, 2022

Agenda Item: 6.10.3.1

Source: MediaTek Inc.

Title: Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR

Document for: Discussion

# Introduction

In existing Rel-15/16 NR, two measurement gaps have been identified, which are per-UE and per-FR measurement gap. Then, in Rel-16 NeedForGap feature have been introduced, however, there has been no requirements in RAN4 to support this feature. Later in Rel-17 NR, three measurement gap enhancement have been considered, which are: (i) pre-configured MG pattern(s) per configured BWP (fast MG configuration), (ii) multiple concurrent and independent MG patterns, and (iii) network controlled small gap (NCSG). Now in Rel-18, further work objective to complete the requirements for measurement without gaps is given in the work item description (WID) [1] as below:

- Define RRM requirements for measurement without gaps for the following cases - NR SSB-based inter-frequency and intra-frequency measurements without gaps for UEs reporting NeedForGapsInfoNR IE [RAN4] - Study whether the additional interruption is allowed when UE reporting ‘NeedForGapsInfoNR'. Further define the interruption length, occasion and ratio, if the interruption is allowed - Define related requirements, such as CSSF, measurement period, scheduling restriction etc.

Furthermore, the open issues from the previous meeting (meeting RAN4#104-e) are given below [2]:

## Sub-topic 3-1: [NFG] NeedForGapsInfoNR ### Issue 3-1: [NFG] Whether interruption is expected when UE reports ’no-gap’ < Way forward >: - Option 1: Yes - Option 2: No - Option 3: Introduce additional UE capability to differentiate whether UE needs interruption ### Issue 3-2: [NFG] Interruption requirement, if allowed < Way forward >: - Option 1: (Visible) Take NCSG as a starting point. FFS the exact values - Option 2: (Invisible) Adopt deactivated SCell’s interruption requirement as a start point. FFS the interruption ratio (data dropping rate). ### Issue 3-4: [NFG] Requirement for intra-freq measurement without gap < Way forward >: - Option 1: Same as requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap) - Option 2: Take requirements NCSG requirements as a starting point - Option 3: Take requirements in Section 9.39 of TS38.133 (inter-freq wo/ gap) as a starting point - Option 4: The related frequency layer should be counted in CSSF outside gap - Option 5: While discussing the measurement period, since similar with the same issue in pre-configured MG, some solution proposed during pre-configured MG discussion can be used for reference. ### Issue 3-5: [NFG] Requirement for inter-freq measurement without gap < Way forward >: - Option 1: Take requirements in Section 9.39 of TS38.133 (intra-freq w/o gap) as a starting point - Option 2: Take requirements NCSG requirements as a starting point - Option 3: Take requirements in Section 9.3.9 of TS38.133 (inter-freq wo/ gap) as a starting point - Option 4: The related frequency layer should be counted in CSSF outside gap - Option 5: While discussing the measurement period, since similar with the same issue in pre-configured MG, some solution proposed during pre-configured MG discussion can be used for reference.

The analysis and discussion are given in the next section.

# Discussion on the issues from the WF

Issue 3-1: Whether interruption is expected when UE reports ’no-gap’

From the existing NR intra-frequency measurement, the following requirements for Rel-16 NeedForGap are ambiguous on whether interruption is needed:

The UE can perform intra-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) if - the UE indicates ‘no-gap’ via intraFreq-needForGap for intra-frequency measurement, or - the SSB is completely contained in the active BWP of the UE, or - the active downlink BWP is initial BWP[3].

Let’s assume the following scenario A, as shown in the figure below, is a UE indicating ‘no-gap’ and operating in an active BWP without SSB during data transmission/reception on non-SSB symbols. Now, in order for the UE to measure the SSB then the UE RF BW shall be increased to cover the SSB outside the active BWP (i.e. to measure the SSB symbols), as depicted in scenario B. Although the UE indicates no need for gap, the UE still need an interruption time to enlarge the RF BW to measure the SSB. Nevertheless, such interruption requirements have not been identified in existing requirements. Therefore, RAN4 shall define the interruption length, occasions and ratio in Rel-18 discussion.

Figure 1: A UE operating in an active BWP without SSB. Scenario A is when the UE transmitting/receiving data on non-SSB symbols, while scenario B is when the UE measuring SSB symbols.

Another scenario is for inter-frequency measurements. UE may be configured with many inter-frequency MOs but UE only has limited number of RF chains. Even if UE can measure individual inter-frequency layer without gap, UE still needs to measurement all of them one-by-one. Hence, UE has no choice but to retune the RF frequency on some RF chains in order to measure all the inter-frequency layers.

- Existing Rel-16 NeedForGap requirements are not complete regarding the need for interruption and its details.

- RAN4 shall allow interruption for the Rel-16 NeedForGap capability when a UE indicates ‘no-gap’.

- RAN4 shall define requirements for the interruption length, occasions and ratio to complete Rel-16 NeedForGap requirements.

Issue 3-2: [NFG] Interruption requirement, if allowed

Given that the new interruption mechanism for NeedForGap has already been covered in NCSG requirements, therefore, RAN4 shall use the existing Rel-17 NCSG requirements as a baseline to define the interruption related requirements in NeedForGap.

- RAN4 shall leverage the existing Rel-17 NCSG requirements to define the new interruption requirements for NeedForGap.

Issue 3-4: [NFG] Requirement for intra-freq measurement without gap

In general for measurements without gap, the overall measurement delay has the following structure: (Delay of a single layer) x CSSFoutside_gap. Now, the delay of a single layer can reuse existing requirements from either Section 9.2.5 or NCSG requirements, which depends on the conclusion from previous issues (issues 3-1 and 3-2).

- RAN4 shall define the requirements for intra-frequency measurement without gap (delay of a single layer) x CSSFoutside_gap, where the delay requirements can be reused from either Section 9.2.5 or NCSG.

Issue 3-5: [NFG] Requirement for inter-freq measurement without gap

As commented in the previous issue, the overall measurement delay has the following structure: (Delay of a single layer) x CSSFoutside_gap. Now, the delay of a single layer can reuse existing requirements from either Section 9.3.9 or NCSG requirements, which depends on the conclusion from previous issues (issues 3-1 and 3-2).

- RAN4 shall define the requirements for inter-frequency measurement without gap (delay of a single layer) x CSSFoutside_gap, where the delay requirements can be reused from either Section 9.3.9 or NCSG.

# Summary

In this contribution, discussion on general and RRM requirement in RedCap UEs are provided and we have the following observation:

Observation 1: Existing Rel-16 NeedForGap requirements are not complete regarding the need for interruption and its details.

Also, we have the following proposal:

Proposal 1: RAN4 shall allow interruption for the Rel-16 NeedForGap capability when a UE indicates ‘no-gap’.

Proposal 2: RAN4 shall define requirements for the interruption length, occasions and ratio to complete Rel-16 NeedForGap requirements.

Proposal 3: RAN4 shall leverage the existing Rel-17 NCSG requirements to define the new interruption requirements for NeedForGap.

Proposal 4: RAN4 shall define the requirements for intra-frequency measurement without gap (delay of a single layer) x CSSFoutside_gap, where the delay requirements can be reused from either Section 9.2.5 or NCSG.

Proposal 5: RAN4 shall define the requirements for inter-frequency measurement without gap (delay of a single layer) x CSSFoutside_gap, where the delay requirements can be reused from either Section 9.3.9 or NCSG.

# References

- R4-222337, “New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, MediaTek Inc, Intel Corporation, September 2022.

- R4-2214346, “WF on further enhancements on measurement gaps and measurements without gaps”, MediaTek inc., August.

- RP-220867, “Moderator’s summary for discussion [95e-07-RAN4-R18-MeasGap]”, March 2022.