3GPP TSG-RAN WG4 Meeting # 104bis-e	R4- 2216742

# Electronic Meeting,  October 10 – October 19, 2022

Agenda Item:	4.8.2

Source: 		Qualcomm Incorporated

Title: 			Discussion on RRM performance requirement for CG-SDT

Document for:	Discussion

# 1.	Introduction

In this paper, we discuss the test framework for RRM performance requirements of CG-SDT from last RAN4 meeting.

# 2. 	Discussion

RAN5 confirmed the test feasibility of CG-SDT in LS. We duplicated the LS [1].

For UE to initiate reliable MO Small Data transmission in RRC_INACTIVE, RAN5 discussed the testability aspects of SDT and agreed on a solution to use ‘UE Test Loop Function operating in UE test loop mode B’ defined in TS 36.509 with “T_delay_modeB” timer configured, as per test requirement.

RAN5 will trigger UL data for CG-SDT transmission. From the RAN4-104e meeting [2], we suggested combining pass and fail TA validation in a single test to increase test reliability. As the test framework does not subject to frequency band, we propose the test framework and test case is proposed in FR1.

Proposal 1. CG-SDT test framework will follow following steps.

- Step1: two CG-SDT resources are configured.

- Step2: UE measured reference RSRP1 when UE RRCrelease with CG-SDT configuration is received when changing from RRC_CONNECTED to RRC_INACTIVE state.

- Step3: RSRP is changed within the threshold boundary.

- Step4: UE measure RSRP2 for TA validation

- Step5: UE transmit CG-SDT

  - No sub-sequent transmissions.

- Step6: RSRP is changed to beyond the threshold boundary

- Step7: UE measure RSRP2 for TA validation and fail.

The detail framework is illustrated in Figure.1. There are seven time points which are defined as followings:

Figure1. CG-SDT test framework for testing pass and fail TA validation in one test.

Proposal 2. Definition of time points in the test framework.

- Time point A: RRCrelease with CG-SDT configuration.

- Time point B: RSRP dropped within the -cg-SDT-RSRP-change-Threshold-r17

- Time point C: UE measure RSRP2 for TA validation

- Time point D: UE transmits first CG-SDT

- Time point E: RSRP dropped beyond the -cg-SDT-RSRP-change-Threshold-r17

- Time point F: UE measure RSRP2 for TA validation.

- Time point G: UE do not transmit second CG-SDT

- Time duration (T1,T2,T3,T4,T5,T6) is TBD

# 3. Conclusions

Proposal 1. CG-SDT test framework will follow following steps.

- Step1: two CG-SDT resources are configured.

- Step2: UE measured reference RSRP1 when UE RRCrelease with CG-SDT configuration is received when changing from RRC_CONNECTED to RRC_INACTIVE state.

- Step3: RSRP is changed within the threshold boundary.

- Step4: UE measure RSRP2 for TA validation

- Step5: UE transmit CG-SDT

  - No sub-sequent transmissions.

- Step6: RSRP is changed to beyond the threshold boundary

- Step7: UE measure RSRP2 for TA validation and fail.

Proposal 2. Definition of time points in the test framework.

- Time point A: RRCrelease with CG-SDT configuration.

- Time point B: RSRP dropped within the -cg-SDT-RSRP-change-Threshold-r17

- Time point C: UE measure RSRP2 for TA validation

- Time point D: UE transmits first CG-SDT

- Time point E: RSRP dropped beyond the -cg-SDT-RSRP-change-Threshold-r17

- Time point F: UE measure RSRP2 for TA validation.

- Time point G: UE do not transmit second CG-SDT

- Time duration (T1,T2,T3,T4,T5,T6) is TBD

# 4.    References

[1] R5-225279, Reply LS on the feasibility of testing UE initiated SDT data transmission in RRC_INACTIVE

[2] R4-2214341, Way forward on RRM requirements and test cases for NR SDT, ZTE, RAN4 #104-e