3GPP TSG-RAN WG4 Meeting # 104bis-e	R4- 2216745

# Electronic Meeting,  October 10 – October 19, 2022

Agenda Item:	6.9.3

Source: 		Qualcomm Incorporated

Title: 			Discussion on RRM requirements for remaining issues about FR1+FR1 NR-DC

Document for:	Discussion

# 1.	Introduction

In this paper, we further share our view about the RRM requirements for FR1+FR1 NR-DC from the last RAN4 104-e meeting [1].

# 2. 	Discussion

In the last RAN4 meeting, many issues are left under FFS. However, we think the current requirements can support FR1+FR1 in most cases.

From current RF spec, it is interpreted as 6 NR DL CCs in total with 3 UL. If case of the number of serving carriers will be updated the late phase of Rel-18, RAN4 should revise the requirement accordingly later.

Proposal 1 : For FR1+FR1 NR-DC, the number of serving carriers in RRM requirement is up to 6 NR DL CCs in total, with 1 UL in PCell, 1 UL in PSCell and 1 UL in SCell.

We think the current RRM requirement can support FR1+FR1 in most cases.

Proposal 2 : The existing requirements can be applicable to following RRM requirements for FR1+FR1 NR-DC

- Conditional PScell addition delay requirement

- Existing scheduling availability requirement for performing RLM/BFD/CBD/L1-RSRP

- HO with PScell (Legacy Tprocessing +5ms for parallel processing, +10ms for sequential processing)

# 3. Conclusions

Proposal 1 : For FR1+FR1 NR-DC, the number of serving carriers in RRM requirement is up to 6 NR DL CCs in total, with 1 UL in PCell, 1 UL in PSCell and 1 UL in SCell.

Proposal 2 : The existing requirements can be applicable to following RRM requirements for FR1+FR1 NR-DC

- Conditional PScell addition delay requirement

- Existing scheduling availability requirement for performing RLM/BFD/CBD/L1-RSRP

- HO with PScell (Legacy Tprocessing +5ms for parallel processing, +10ms for sequential processing)

# 4.    References

[1] R4-2214345, WF on R18 eFeRRM, Apple, RAN4 #104-e