3GPP TSG-RAN WG4 Meeting # 104bis-e	R4- 2216746

# Electronic Meeting,  October 10 – October 19, 2022

Agenda Item:	6.10.3

Source: 		Qualcomm Incorporated

Title: 			RRM requirements for measurements without gap

Document for:	Discussion

# 1.	Introduction

In this paper, we further share our view about the RRM requirements for measurement without gaps from the last RAN4 104-e meeting [1].

# 2. 	Discussion

- NeedforGAP

A measurement is defined as an inter-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) for UE capable of interFrequencyMeas-NoGap provided - the UE supports interFrequencyMeas-Nogap-r16 [15], and - the SSB is completely contained in the active BWP of the UE.

In 38.306 spec, the NeedforGapsNR-r16 is defined to report for inter frequency measurement. However, there is no requirement to use NeedforGap for inter-frequency measurement in 38.133 spec.

It is unclear about R16 UE behaviour if the requirements are defined to report no-gap via Needforgap. Some UE may have interruption and some UE may not have interruption. Since the requirement was not defined, our position was not making any change from the new requirement with R16 signalling for R16 UE. If there is no affect to R16 UE, then we can compromise to have interruption requirements when UE report no-gap for R18 UE. For R18 UE who does not need interruption, optional capability can be considered.

Observation1: In current 38.133 spec, there is no requirement to report via needforGap for inter-frequency measurement.

Proposal 1. RAN4 define the requirement for inter-frequency measurement when UE report via needforgap.

Observation2: It is not clear about R16 UE behaviour from defining a new requirement. Some R16 UE may cause interruption and some UE may not cause interruption when UE report no-gap via needforGAP. It is not clear how the R16 UE meet the requirements.

Proposal 2. The requirement shall apply only for R18 UE who report no-gap. No impact on other release UE.

Proposal 3. For R18 UE, we can compromise to define interruption requirement when UE report no-gap. We are open to discuss having an optional UE capability to indicate the R18 UE whether interruption is needed.

- Inter-RAT LTE measurement requirements

During last RAN4 meeting about NCSG [2], RAN4 agreed to define the requirements for inter-RAT measurement without gap when UE indicates ‘nogap-noncsg’ via NeedForNCSG-InfoEUTRAN for the inter-RAT measurement. Thus, no further requirements in Rel-18 are needed.

R4-2214336, RAN4 #104-e NR-LTE inter-RAT measurement with ‘nogap-noncsg’ Agreements - Define requirements for NR – LTE inter-RAT measurement without gap (UE reports ‘nogap-noncsg’ for inter-RAT measurement).

Proposal 4: No other requirements are needed in Rel-18 for inter-RAT LTE measurement as RAN4 agreed to define the requirements for inter-RAT measurement without gap when UE indicates ‘nogap-noncsg’ via NeedForNCSG-InfoEUTRAN in Rel-17.

# 3. Conclusions

Observation1: In current 38.133 spec, there is no requirement to report via needforGap for inter-frequency measurement.

Proposal 1. RAN4 define the requirement for inter-frequency measurement when UE report via needforgap.

Observation2: It is not clear about R16 UE behaviour from defining a requirement. Some R16 UE may cause interruption and some UE may not cause interruption when UE report no-gap via needforGAP. It is not clear how the R16 UE meet the requirements.

Proposal 2. The requirement shall apply only for R18 UE, there should be no impact on other release UE.

Proposal 3. For R18 UE, we can compromise to define interruption requirement when UE report no-gap. We are open to discuss having an optional UE capability to indicate the R18 UE whether interruption is needed.

Proposal 4: No other requirements are needed in Rel-18 for inter-RAT LTE measurement as RAN4 agreed to define the requirements for inter-RAT measurement without gap when UE indicates ‘nogap-noncsg’ via NeedForNCSG-InfoEUTRAN in Rel-17.

# 4.    References

[1] R4-2214346, WF on further enhancements on measurement gaps and measurements without gaps, MediaTek, RAN4 #104-e

[2] R4-2214336, WF on NCSG, Apple, RAN4 #104-e