3GPP TSG-RAN4 Meeting #104-bis-e	R4-2216755
Electronic Meeting, 10th October – 19th October 2022

Agenda item:	6.5.3

Source:	Keysight Technologies

Title:	On QoQZ Validation and MU for Multi-AoA Systems for Multi-Chain Operation

Document for:	Approval

# Introduction

This contribution discusses the impact of positioner blocking on MU element of Quality of Quiet Zone.

# Discussion

In [1], discussions were held on the positioner blocking and it was agreed  to study this effect on the MU element of Quality of Quiet Zone going forward [2].

Issue 3-1: MU impacts for Multi-Rx test system - Proposals - Option 1: RAN4 to study the impact on MU element of Quality of Quiet Zone and positioner blocking. - Option 2: specify other option if any - Agreement: RAN4 to study the impact of positioner blocking on MU element of Quality of Quiet Zone

As the test system for multi-chain operation has not been defined yet and various features are being discussed, e.g., full degrees of freedom for both AoAs vs full degrees of freedom for first AoA with limited/fixed angular separations for the second AoA [3], the QoQZ validation procedure cannot be defined yet, i.e., the details of how positioner blocking is affecting the MU element cannot be addressed yet.

Proposal 1: Defer the effect of positioner blocking on QoQZ MU until the test system for multi-chain operation has been defined.

It would be ideal if an existing multi-probe/multi-AoA test systems could be leveraged for the multi-DL chain test cases as the existing QoQZ procedure could be applied readily. This procedure and the effect of positioner blocking on the MU element of QoQZ is investigated next.

The QoQZ procedure for FR2 is defined in [4], specifically Annex O. The effect of positioner blocking is considered and taken into account in the QoQZ procedure as the reference antenna rotations must utilize rotations in full 3D, e.g., for the Combined-Axes System, the antenna rotation angles are defined as “5 different angles , i.e.,   = -90o, -45o, 0o, 45o, and 90o and rotated around the y axis with 8 different angles  = 0o, 45o, 90o, 135o, 180o, 225o, 270o, and 315o” with additional clarification “Due to the pedestal of the 2-axis positioner, combined-axes systems are not able to measure towards the  = 180o direction; for those systems, the reference measurements at this reference AUT orientation can be skipped.”

These antenna rotations/orientations are further illustrated in Figure O.2.6.2-1 of [4]

Figure O.2.6.2-2: Sample reference AUT orientations for position 4, P4, for reference antenna polarization pol = 90o

Sample reference antenna orientations of the QoQZ validation with significant positioner blocking are highlighted in Figure 1 with the reference antenna pointing towards the positioner.

Figure 1: QoQZ reference antenna orientation with blocking effect (left in the mounting position, right in the measurement position for peak EIRP).

Observation 1: The current FR2 QoQZ validation procedure takes the positioner blocking effects into account.

The positioner blocking can be avoided during the DUT measurements and the QoQZ validation if the re-positioning concept, introduced in Clause N.3 [4], is adopted, e.g.,

Near-field coupling effects between the antenna and the pedestals/positioners/fixtures generally cause increased signal ripples. Re-positioning the DUT by directing the beam peak away from those areas can reduce the effect of signal ripple on EIRP/EIS measurements.

Specifically, the blocking of the positioning can be eliminated by re-positioning the device in two separate orientations so that both hemispheres are tested without any blockage, e.g.,

Figure N.3-2: DUT re-positioning for an example of combined-axes system

This is also clarified in Annex O of [4], e.g.,

If the device re-positioning approach outlined in Annex N is adopted for all EIRP/EIS/TRP based conformance test cases, the quality of quiet zone analysis is sufficient only° for  = 0°, 45°, 90°, 270°, and 315°.

Observation 2: The positioner blocking effect can be avoided if the DUT measurements and QoQZ validation procedures take the re-positioning concept into account.

The FR2 multi-probe/multi-AoA test systems defined so far are the 2 AoA RRM systems:

- DFF, e.g., Figure B.2.2.1-2 of [5]

Figure B.2.2.1-2: Example RRM baseline system with two simultaneously active AoA using a DFF setup.

- Enhanced IFF, e.g., Figure B.2.6.1-1of [5]

Figure B.2.6.1-1: Example RRM baseline system with two simultaneously active AoA using an Enhanced IFF setup

- Hybrid IFF+DFF, e.g., Figure B.2.7.1-1of [5]

Figure B.2.7.1-1: Example RRM baseline system with two simultaneously active AoA using an IFF+DFF setup

and the FR2 MIMO OTA system, i.e., Figure B.2.1-2 of [6].

Figure B.2.1-2: Channel Model Coordinate Axes in FR2 3D-MPAC system

For the RRM systems supporting 2 AoA, Clause 7.1.3.2.4 of [5] specifies that the QoQZ validation defined in Annex O of [4] applies, e.g.,

7.1.3.2.4 Quality of the quiet zone For RRM, the quality of the quiet zone validation defined in Annex O of TS 38.521-2 [15] needs to assess only the single-directional EIRP and EIS metrics. For measurement setups with multiple probes, the QoQZ procedure needs to be performed with all probes present and in the conditions used for RRM testing. The quality of the quiet zone for the RRM measurement setup based on DFF is described in B.2.2.3. The QoQZ validation needs to be performed only with the reference probe P0. The quality of the quiet zone for the RRM measurement setup based on simplified DFF is described in B.2.3.3. The quality of the quiet zone for the RRM measurement setup based on IFF is described in B.2.4.3. The quality of the quiet zone for the RRM measurement setup based on enhanced IFF is described in B.2.6.3. The QoQZ validation needs to be performed only with the reference reflector, P0, if same sized IFF reflectors are used.. The quality of the quiet zone for the RRM measurement setup based on IFF+DFF is described in B.2.7.3. The QoQZ validation needs to be performed only with the one probe among all DFF probes and one probe among all IFF probes.

In [7], it is furthermore specified that the multi-probe system for FR2 MIMO OTA utilizes the QoQZ validation defined in Annex O of [4]

The quality of quiet zone test procedure, equipment, and test frequencies are defined in Annex O.2 of TS 38.521-2 [9]. For NR FR2 MIMO OTA, only the single-directional EIRP and EIS metrics need to be assessed and the procedure needs to be performed using just a single 3D MPAC probe.

Therefore, the multi-probe/multi-AoA OTA test systems defined in 3GPP for FR2 testing reference the QoQZ validation procedure defined in Annex O of [4].

Observation 3: The FR2 QoQZ validation procedure defined in Annex O of [4] currently applies to all singe-probe and multi-probe FR2 UE RF and RRM test systems.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: The current FR2 QoQZ validation procedure takes the positioner blocking effects into account.

Observation 2: The positioner blocking effect can be avoided if the DUT measurements and QoQZ validation procedures take the re-positioning concept into account.

Observation 3: The FR2 QoQZ validation procedure defined in Annex O of [4] currently applies to all singe-probe and multi-probe FR2 UE RF and RRM test systems.

Proposal 1: Defer the effect of positioner blocking on QoQZ MU until the test system for multi-chain operation has been defined.

# References

- R4-2214323, Email discussion summary for [104-e][334] FS_NR_FR2_OTA_enh, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting # 104-e, August 2022

- R4-2214357, WF on NR FR2 OTA testing enhancements, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting # 104-e, August 2022

- R4-2215540, Testing Considerations for Multi AoA UE RF Spherical Coverage Test Case and FR2 Demodulation, Keysight Technologies, 3GPP TSG-RAN4 Meeting #104-bis-e, October 2022

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

- TS 38.508-1, User Equipment (UE) conformance specification; Part 1: Common test environment, V17.5.0 (2022-06)

- TS 38.151, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements, V17.2.0 (2022-09)

- TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE), V17.8.0 (2022-09)