3GPP TSG-RAN WG4 Meeting #104bis-e	R4- 2216778

Electronic Meeting, 10 – 19 October 2022

Source: 	Huawei, HiSilicon

Title:	Discussions on IoT NTN system parameters

Agenda Item:	7.5.2

Document for:	Agreement

# 1	Introduction

Issue 1-3:    Channel spacing

- Agreement:

- Reuse TN specification 36.101/36.104 norminal channel spacing for Cat-M1 and NB-IoT

  - Further discuss the frequency gap for NB-IoT over NTN deployment

- Removal of in-band/guard band item.

## Issue 1-4-2:    M_DL for standalone NB-IoT

Further discussion below options in next meeting:

- Option 1: To follow the same MDL values as TN FDD as baseline

- Option 2: Update the M_DL value to improve spectral efficiency (R4-2213243)

- Option 3: Others are not precluded.

Issue 1-4-3:    EARFCN:

Further discussion in next meeting:

- Start from the highest EARFCN  262143 value and descending manner for B256 and B255

For the IoT-NTN WI [1], a WF [2] on system parameters was agreed in RAN4#10e-4. Among the agreements, there’re three issues to be further discussed, which are duplicated below:

In the following, we share our views on the remaining three issues.

# 2	Discussion

Based on the discussions in the last meeting, we share some further analysis on the three pending issues.

## 2.1 Channel spacing and guard band for NB-IoT

For NB-IoT standalone operation, NB-IoT requirements for receiver and transmitter shall apply with a frequency offset Foffset  as defined in Table 5.6-3A.

Table 5.6-3A: Foffset for NB-IoT standalone operation

| Lowest or Highest Carrier | Foffset |
| --- | --- |
| Standalone NB-IoT | 200 kHz |

It has been pointed out that a 100kHz guard band is defined for NB-IoT standalone deployment in TS 36.104.

Given that the channel bandwidth is defined as 200kHz for standalone NB-IoT, the Foffset of 200kHz means an extra 100kHz guard-band is needed in addition to the nominal channel spacing of 200kHz. The need of the guard-band comes from the coexistence with GSM systems, which was one of the basic assumptions when NB-IoT was developed in Rel-13.

Although the coexistence with GSM systems is not needed for NTN bands, we believe the guard-band is still needed to minimize the interference between asynchronous NB-IoT carriers, such as those belonging to different operators. The requirement for the guard-band may be relaxed in the future if it’s confirmed by coexistence study of different NTN NB-IoT carriers. Note that no such study is planned in the current WID [1].

For NB-IoT anchor and non-anchor carriers from the same operator, the guard-band is not necessary, and the channel spacing can be minimize to 180kHz, provided that the carriers are synchronous in time and frequency.

Proposal 1: Reuse the frequency offset Foffset of 200kHz as specified for NB-IoT TN at least for the current WI.

## 2.2 M_DL for standalone NB-IoT

In NB-IoT TN, the offset of channel number to DL/UL EARFCN is defined, i.e. M_DL/M_UL, in order to maintain the frequency orthogonality between anchor and non-anchor carriers. It was agreed in [2] that the channel raster is kept at 100kHz for the two proposed IoT-NTN bands. Hence, we believe the existing M_DL/M_UL values for TN FDD are sufficient for NTN.

Proposal 2: Follow the same M_DL values as TN FDD for NTN.

## 2.3 ARFCN for IoT-NTN

It was agreed in [2] that the channel raster is to be defined as 100kHz for both NTN bands, and the channel number will start from the highest EARFCN of 262143 and in descending manner for band B256 and B255.

Based on our proposals in [3], some minor modifications are made to support 100kHz channel raster.

Following the convention of the existing EARFCN, the uplink EARFCN starts from 262143 (=2^18-1) for the highest carrier frequency for band 256, and then continue in descending order to assign the EARFCN for the uplink of the rest of band 256, and then band 255. For the downlink, the assignment of EARFCN may start from 229375, which is equal to 2^18-2^15-1. The details of the new EARFCN are shown in Table 1.

Table 1: IoT-NTN channel numbers

| NTN Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| 256 | 2170 | 229076 | 229076 – 229375 | 1980 | 261844 | 261844 – 262143 |
| 255 | 1525 | 228736 | 228736 – 229075 | 1626.5 | 261504 | 261504 – 261843 |
| NOTE : The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |  |  |  |

Proposal 3: Define the ARFCN for IoT-NTN as shown in Table 1.

# 3	Conclusion

In this paper, we have shared our further analysis on the three remaining issues in system parameters. The following proposals are made:

Proposal 1: Reuse the frequency offset Foffset of 200kHz as specified for NB-IoT TN at least for the current WI.

Proposal 2: Follow the same M_DL values as TN FDD for NTN.

Proposal 3: Define the ARFCN for IoT-NTN as shown in Table 1.

# References

- RP-221556 WID on NB-IoT/eMTC core & performance requirements for Non-Terrestrial Networks, MediaTek, RAN#96

- R4-2214380 WF on system parameter for IoT over NTN, Ericsson, RAN4#104-e

- R4-2212454 Discussions on IoT NTN UE RF requirements, Huawei, HiSilicon, RAN4#104-e