3GPP TSG RAN WG4 Meeting #104bis-e	R4-2216857

Electronic Meeting, 10-19 October 2022

Agenda Item:	7.5.1

Source: 	Ericsson

Title:	On band grouping for RRM requirements for IoT with satellite access

Document for:	Discussion

- Introduction

The RRM core requirements for NB-IoT and UE category M1 (Cat-M1) for satellite access (i.e. NTN) are being introduced in Rel-18, TS 36.133. This also includes specification of the applicable frequency bands and the band grouping, in TS 36.133.

In this paper we analyze and propose the frequency band grouping for IoT (NB-IoT and Cat-M1) for satellite access in TS 36.133.

# Analysis of satellite access band grouping for NB-IoT

The following two FDD frequency bands are agreed to be specified for NB-IoT (NB1 and NB2) the satellite access according to the approved WID [1]:

- Band 255

- Band 256

According to the agreements in RF group in the approved WF [2], both band 255 and band 256 for NB-IoT have the same REFSENS as defined for the legacy NB-IoT FDD bands i.e. -108.2dBm.

For the applicability of the RRM requirements the band groups for NB-IoT (NB1 and NB2) are defined in Table 3.5.1-2 in TS 36.133. All the FDD bands for NB-IoT have the same REFSENS. Therefore, they are all defined in the same band goup, G, in Table 3.5.1-2 in TS 36.133.

Therefore, bands 255 and 256 should be added in the same band group in TS 36.133.

The satellite access bands (SAB) will be introduced in TS 36.133 in separate section dedicated to the satellite access. Therefore, new SAB specific band groups are needed also for NB-IoT.

Consequently bands 255 and 256 for NB-IoT for satellite access can be assigned to new band group G, which can be termed as: NFDD_SAB_G. However, place holders for the same number of band groups (i.e. A to N) as in legacy can be defined NB-IoT bands for satellite access in the new table in TS 36.133.

# Analysis of satellite access band grouping for Cat-M1

The following two FDD frequency bands are agreed to be specified for FDD and HD-FDD capable UE category M1 the satellite access according to the approved WID [1]:

- Band 255

- Band 256

According to the agreements in RF group in the approved WF [2]:

- Band 255 for Cat-M1 has the same REFSENS as defined for the legacy Cat-M1 FDD band 24 i.e. -108.2dBm.

- REFSENS for band 256 for Cat-M1 is still under discussion i.e. FFS.

For the applicability of the RRM requirements the band groups for Cat-M1 are defined in Table 3.5.1-2 in TS 36.133. The same band groups are are applicable for both FDD and HD-FDD capable Cat-M1 UE.

Currently band 24 for legacy Cat-M1 belongs the band goup, A, in Table 3.5.1-2 in TS 36.133.

The SAB specific band groups are needed also for Cat-M1 in TS 36.133. Band 255 should be added in the same band group (A) in the new table for SAB specific band groups. However, place holders for the same number of band groups (i.e. A to N) as in legacy can be defined Cat-M1 bands for satellite access in the new table in TS 36.133.

The band group for Cat-M1 band 256 can be defined after RF group has agreed on the REFSENS for this band.

# Summary

The following are the observations and proposals related to the grouping for satellite access band for applicability of RRM requirements for NB-IoT and Cat-M1 in TS 36.133:

Satellite access band grouping for NB-IoT:

- Observation 1: UE REFSENS is the same for satellite access bands 255 and 256 for NB-IoT (NB1 and NB2).

- Observation 2: UE REFSENS of 255 and 256 is the same as legacy FDD bands defined for terrestrial NB-IoT network.

- Proposal #1: The satellite access bands 255 and 256 are assigned to same band group for applicability of RRM requirements in TS 36.133.

- Proposal #2: The band group for NB-IoT bands 255 and 266 is termed as: “NFDD_SAB_G”

  - where SAB stands for satellite access band to distinguish from the terrestrial band group naming.

Satellite access band grouping for Cat-M1:

- Observation 1: UE REFSENS of 255 is the same as legacy band 24 defined for terrestrial Cat-M1 network.

- Observation 2: UE REFSENS for satellite access band 256 is FFS.

- Proposal #1: The band group for Cat-M1 band 255 is termed as: “FDD-M1_SAB_A”

  - where SAB stands for satellite access band to distinguish from the terrestrial band group naming.

- Proposal #2: The band group for Cat-M1 band 255 will be assigned after REFSENS agreement in RF group.

Rel-18 Draft CRs to introduce the grouping of SABs for NB-IoT and Cat-M1 in TS 36.133 are provided in [3-4].

- References

- RP-221556, “NB-IoT/eMTC core & performance requirements for NTN”, Approved WID.

- R4-2214467, “Way forward on NB-IoT/eMTC NTN agenda item 12.5.1 & 12.5.4”, MediaTek.

- R4-2216858, “Draft CR on band grouping for NB-IoT for satellite access in 36.133”, Ericsson.

- R4-2216859, “Draft CR on band grouping for Cat-M1 for satellite access in 36.133”, Ericsson.