3GPP TSG-RAN WG4 Meeting # 104-bis-e												R4-2216886

Electronic Meeting, 10– 19 October 2022

Agenda item:			4.1.5.1, 4.1.5.2

Source:	Moderator (Huawei)

Title:	Email discussion summary for [104-bis-e][302] NR_Repeater_RFConformance_Part1

Document for:	Information

# Introduction

The scope of this email discussion is the agenda item 4.1.5.1 and 4.1.5.2 on the NR repeater conducted conformance testing, including TPs to TS 38.115-1.

The following contributions were submitted before the submission deadline to agenda items 4.1.5.1 and 4.1.5.2:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2216192 | NR Repeater stimulus signal | Nokia, Nokia Shanghai Bell |
| R4-2216500 | Repeater stimulus signal spectral purity annex | Keysight Technologies UK Ltd |
| R4-2216611 | TP to 38.115-1: EVM requirement | NEC |
| R4-2216838 | TP to TS 38.115-1: Repeater output power (6.1, 6.2) | Huawei, HiSilicon |
| R4-2216608 | TP to 38.115-1: ACRR requirement | NEC |
| R4-2216614 | TP to 38.115-1: ACLR requirement | NEC |
| R4-2215387 | TP for TS 38.115-1: scope and reference | CATT |
| R4-2216837 | TP to TS 38.115-1: Measurement uncertainties and test requirements (4.1) | Huawei, HiSilicon |
| R4-2216839 | TP to TS 38.115-1: Annex B: Environmental requirements for the repeater | Huawei, HiSilicon |
| R4-2216840 | TP to TS 38.115-1: Annex C: Test tolerances and derivation of test requirements | Huawei, HiSilicon |
| R4-2216841 | TP to TS 38.115-1: Annex E: Characteristics of interfering signals | Huawei, HiSilicon |

The following topics were identified for discussion:

- Improvements to the stimulus signal requirements

- TPs to TS 38.115-1

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round:

  - Collect feedback on the potential stimulus signal requirements adjustments, in order to implement them into a new TP during the second round.

  - Collect comments on the TPs to the TS 38.115-1, as input for further adjustments during the 2nd round, if any.

- 2nd round: TBA

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| ZTE | Fei Xue | Xue.fei25@zte.com.cn |
| Ericsson | Tom Chapman | Thomas.chapman@ericsson.com |
| Keysight | Takao Miyake | takao_miyake@keysight.com |
| QCOM | Phil Coan | pcoan@qti.qualcomm.com |
| Huawei | Michal Szydelko | Michal.szydelko@huawei.com |
| Nokia | Bartlomiej Golebiowski | bartlomiej.golebiowski@nokia.com |
| NEC | Tetsu Ikeda | tetsu.ikeda@nec.com |

# Topic #1: Improvements to the stimulus signal requirements

## Companies’ contributions summary

The following contributions are treating on the stimulus signal:

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2216192 | Nokia, Nokia Shanghai Bell | Proposal 1: For NR Repeater stimulus signal, spectral purity requirements should be design similar way, and experience there could be reused. Moderator: the above proposal is to follow TS 36.143 approach for E-UTRA repeater, including NR specific adjustments. |
| R4-2216500 | Keysight Technologies UK Ltd | Proposal-1 - Although it was agreed to re-use E-UTRA repeater specification for FR1 stimulus signal spectral purity requiments, revert this ageement and revise stimulus signal spectral purity requirement for FR1. This is because of findings of original intention and motivation of E-UTRA specification and difference of stimulus signal and testing between E-UTRA and 5G NR. Proposal-2 - For both FR1 and FR2 5G NR repeater specification, state following as signal spectral purity requimenets, because it is obvious that stimulus signal should clear test requirement by itself. - Option 1. State as “Stimulus signal spectral purity should clear test requirement by stimulus signal itself.” And remove current table for FR1. - Option 2. Remove Stimulus signal spectral purity requirement Annex from 5G NR repeater test specification. |

## Open issues summary

WF extract from R4-2214367:

2.1 Sub-topic 2-1 Input signal spectral purity As per Moderator proposal discussed during the GTW, the following WF is agreed: - No need to check repeater stimulus signal spectral purity requirements with ETSI ERM/MSG TFES, - For FR1, reuse the repeater stimulus signal spectral purity requirements from E-UTRA repeater specification, as baseline. - For FR2: the repeater stimulus signal spectral purity requirements are FFS.

### Sub-topic 1-1: Improvements to the stimulus signal requirements, FR1 and FR2

- Proposals (non-exclusive)

  - Proposal 1: For NR Repeater stimulus signal, spectral purity requirements should be design similar way, and experience there could be reused (Nokia, R4-2216192)

  - Proposal 2: Although it was agreed to re-use E-UTRA repeater specification for FR1 stimulus signal spectral purity requirements, revert this agreement and revise stimulus signal spectral purity requirement for FR1. This is because of findings of original intention and motivation of E-UTRA specification and difference of stimulus signal and testing between E-UTRA and 5G NR (Keysight, R4-2216500)

  - Proposal 3: For both FR1 and FR2 5G NR repeater specification, state following as signal spectral purity requirements, because it is obvious that stimulus signal should clear test requirement by itself (Keysight, R4-2216500).

    - Option 1. State as “Stimulus signal spectral purity should clear test requirement by stimulus signal itself.” And remove current table for FR1.

    - Option 2. Remove Stimulus signal spectral purity requirement Annex from 5G NR repeater test specification.

- Recommended WF

  - It is suggested to avoid reverting the previous agreements, i.e. Proposal 2. Instead, it is suggested to correct the stimulus signal requirements to reflect NR specifics.

  - In order to progress the work, it would be most beneficial to work on the TS text itself. Therefore for the first round it is proposed to seek for TP implementation proposals. For the second round, a volunteer is need to draft the related TP text, if any.

- GTW Oct 12th	Agreement: Spectral purity requirements shall be specified for NR Repeater stimulus signal, further discuss the details with NR requirement as starting point

  - Option 1: Absolute power level

  - Option 2: Relative power e.g., ACP type

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| ZTE | This was discussed in the previous meeting. Indeed the current stimulus signal are coming from `ETSI instead of 3GPP internally. We are open for further discussions, maybe to update the freq point for each range of stimulus signal could be one way. E.g. update from 1.4MHz to 5MHz, the its ACLR requirement should be still kept as 1.4MHz. |
| Ericsson | In principle the stimulus signal requirement should be sufficient that TE emissions do not impact measurements of SEM, ACLR, etc. The previously agreed E-UTRA mask is OK for this, but we are also OK to adapt it to NR. Certainly there is no need to consider 1.4MHz bandwidth. Another way is to use the OBUE mask, reduced by a sufficient amount that after amplification in the repeater the TE noise will still be small enough to not impact the measurement. (Also it should be checked that ACLR would not be impacted) |
| CATT | I may slightly to support Keysight’s proposal if it’s not easy to derive the requirements. Using option 1 to add some clarification may be a good candidate. |
| Keysight | We are fine with moderator’s suggestion, which is to revisit FR1 definition. Important point for FR1 is current definition is for 1.4MHz signal and E-UTRA requirement which is different from 5G NR and existing definition doesn’t guarantee any for 5G NR requirement. From TE vender point of view, difficulty to define this is because expected signal level (carrier power) is not clear, and device dependent but trying to come up absolute level for noise adjacent to carrier (channel edge). If it’s everyone’s preference to define something, then our proposal is to define signal purity for stimulus by the way of ACP like (adjacent noise level relative to carrier power) approach. Also to comment that, for FR2, same topic in thread [303] and our proposal is similar (will add comment to [303] discussion document) Regarding with Option 3, we received off line comment that proposed text is not clear, need refinement if people can agree with this approach. |
| Huawei | Ok with proposal 1 for FR1. For proposals 2 and 3: Proposes no spectral purity requirement or that it meets the requirement. This is probably not sufficient as if it only just meets the requirement then there is no room for the repeater to degrade it. The trouble is how much better than the requirement it needs to be depends on the repeater (and how much margin is needed when the 2 are added). It might be ok for it to be at least as good as the requirement is passed, but it needs to be clear that the input needs to be sufficiently pure that the repeater performance can be fairly measured. These easiest way is to define purity in the spec. |
| Nokia | Our proposal is proposal 1 that could be further discuss and however we are also open to further work on Keysight proposal. |

### CRs/TPs comments collection

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Not to revert the previous agreement and to keep the stimulus signal spectral purity requirements in the NR repeater specifications. Candidate options: Based on the GTW agreement, two options were initially identified for further discussion: - Option 1: Absolute power level - Option 2: Relative power e.g., ACP type Solution for FR1 and FR2 may vary. Recommendations for 2nd round: Continue the discussion on the FR1 and FR2 stimulus signal spectral purity requirements during the second round, considering the GTW agreement and two options (relative and absolute requirement). Related WF planned to capture agreements, if any. |

### CRs/TPs

## Discussion on 2nd round (if applicable)

# Topic #2: TPs to TS 38.115-1

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2216611 | NEC | TP to 38.115-1: EVM requirement |
| R4-2216838 | Huawei, HiSilicon | TP to TS 38.115-1: Repeater output power (6.1, 6.2) |
| R4-2216608 | NEC | TP to 38.115-1: ACRR requirement |
| R4-2216614 | NEC | TP to 38.115-1: ACLR requirement |
| R4-2215387 | CATT | TP for TS 38.115-1: scope and reference |
| R4-2216837 | Huawei, HiSilicon | TP to TS 38.115-1: Measurement uncertainties and test requirements (4.1) |
| R4-2216839 | Huawei, HiSilicon | TP to TS 38.115-1: Annex B: Environmental requirements for the repeater |
| R4-2216840 | Huawei, HiSilicon | TP to TS 38.115-1: Annex C: Test tolerances and derivation of test requirements |
| R4-2216841 | Huawei, HiSilicon | TP to TS 38.115-1: Annex E: Characteristics of interfering signals |

## Open issues summary

## Companies views’ collection for 1st round

### Open issues

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2216611 | ZTE: cannot open it. |
|  | Ericsson: For the core requirement, we think it is better to keep to the description of the requirement being on the difference between the symbols provided at the input to the repeater and the output. Otherwise, the core requirement implies that even if (during real operation) some noisy signal is provided to the repeater input then the repeater is still expected to achieve the EVM compared to ideal symbols at the output (i.e. clean up the signal). For conformance testing, we appreciate that comparing to the ideal signals may be more practical (provided that the TE has significantly better EVM than the requirement). For the conformance testing, the spec could be updated as follows: “The Error Vector Magnitude (EVM) is a measure of the difference between the symbols provided at the input of the repeater and the measured signal symbols at the output of the repeater after the equalization by the measurement equipment. During conformance testing, the symbols provided at the input to the repeater are assumed to be ideal symbols (with no distortion)” |
|  | CATT: I also can’t open it. |
|  | Keysight, this is good catch, EVM is to compare between ideal and actual. Thank NEC for finding this. (Note to ZTE, CATT, NEC has sent files through RAN4 reflector on last wed) |
|  | QCOM: The TP looks right, describing the reference as the ideal symbols. |
|  | Huawei: needs to be synchronized with the related modifications to be introduced in [301] to the core spec in R4-2216610. Disagree with this change, cannot use ideal symbols as repeater does not generate modulation. |
|  | Nokia: There is also similar update proposed for core spec in thread 301, where “ideal” term was discussed, and should not be used, thus some other wording should be used. Also, there should be alignment between core and perf specs text. |
|  | NEC: Difference between the ideal and the output signals shall be the measure for EVM. Evaluating the difference between the input and the output signals are not feasible. |
| R4-2216838 | Ericsson: The test procedure only describes testing with the lowest input power, but the test should be repeater with lowest input power +10dB. |
| R4-2216608 | ZTE: cannot open it. |
|  | CATT: I also can’t open it. |
|  | Keysight, this is also good catch, removal of “RRC” should be done. |
|  | QCOM: We don’t understand the purpose of the text “both are assumed to have a nominal channel bandwidth” . Why is this included? |
|  | Huawei: needs to be synchronized with the related modifications to be introduced in [301] to the core spec in R4-2216607, i.e. nominal ch BW needs adding to definitions. |
|  | Nokia: Similar discuss in core spec. |
|  | NEC: How to evaluate ACRR shall be clearly defined. Otherwise, the requirements could be interrupted differently. We propose to adopt the same approach with ACLR, therefore, it is proposed to assume the channels with nominal channel bandwidth. |
| R4-2216614 | ZTE: cannot open it. |
|  | CATT: I also can’t open it. |
|  | Huawei: needs to be synchronized with the related modifications to be introduced in [301] to the core spec in R4-2216613. Note 4 and 5 are perhaps not the best way to do this as we end up with the requirement in both the note and the table, maybe a different column (the ways its done in part 2 is better)? Bwconfig needs adding to symbols and defining there. Nominal ch BW needs defining. |
|  | Nokia: Similar discuss in core spec. |
|  | NEC: We are ok to add Nominal channel bandwidth definition. |
| R4-2215387 | ZTE: In general, we are fine with it, however not sure whether we need to add the reference for 38 .211, 331 for it. |
|  | CATT: 38.211 and 38.331 are referred in the latest version. |
|  | QCOM: We are OK with it but an editorial comment to make the grammar consistent: A repeater type 1-C only has conducted requirements so it requires compliance to this specification only. A repeater type 2-O has only radiated requirements so it requires compliance to TS 38.115-2 [3] only. |
| R4-2216837 | CATT: The reference part should be aligned with the final agreements, it can also be updated by the spec editor. The wording for “10 dB below Base Station according to TS 38.141-1” should be revised. There can be two approaches, first is to refer to NR repeater specification TS 38.106, second is to just remove it. |
|  | Huawei: for the 10dB offset: we will further check and propose resolution for the second round in the revision. Agree that referring to the BS spec is not the best approach… |
| R4-2216839 | ZTE: okay for us. |
|  | CATT: The reference can be handled by spec editor. |
|  | Huawei: agree with CATT. |
| R4-2216840 | ZTE: okay for us. |
| R4-2216841 | ZTE: interfering signal for ACRR might be still needed. |
|  | CATT: output intermodulation needs this? |
|  | Huawei: we are fine to keep it if there is good motivation. The original motivation was if there is just one or two interferers to be listed, it’s easier to capture such information directly in the requirements, and avoid unnecessary annex. Anyway, both ways are doable. |

## Summary for 1st round

### Open issues

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2216611 | Withdrawn / Not pursued; new tdoc number to be allocated instead. |
| R4-2216838 | Revised |
| R4-2216608 | Withdrawn / Not pursued; new tdoc number to be allocated instead. |
| R4-2216614 | Withdrawn / Not pursued; new tdoc number to be allocated instead. |
| R4-2215387 | Revised |
| R4-2216837 | Revised |
| R4-2216839 | Agreeable; The references can be handled by spec editor. |
| R4-2216840 | Agreeable |
| R4-2216841 | Agreeable; Interferer single information to be captured directly in the requirements sections. |

## Discussion on 2nd round (if applicable)

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on stimulus signal spectral purity requirements for , FR1 and FR2 | Nokia | Consider GTW agreement and two options identified. |
|  | TS 38.115-1, v 0.2.0 | CATT | E-mail approval |
|  | TP to 38.115-1: ACRR requirement | NEC | To replace the corrupted TP in R4-2216608; capture comments received during the first round. |
|  | TP to 38.115-1: ACLR requirement | NEC | To replace the corrupted TP in R4-2216614; capture comments received during the first round. |
|  | TP to 38.115-1: EVM requirement | NEC | To replace the corrupted TP in R4-2216611; capture comments received during the first round. |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2216192 |  | NR Repeater stimulus signal | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2216500 |  | Repeater stimulus signal spectral purity annex | Keysight Technologies UK Ltd | Noted |  |
| R4-2216611 |  | TP to 38.115-1: EVM requirement | NEC | Withdrawn / Not pursued |  |
| R4-2216838 |  | TP to TS 38.115-1: Repeater output power (6.1, 6.2) | Huawei, HiSilicon | Revised |  |
| R4-2216608 |  | TP to 38.115-1: ACRR requirement | NEC | Withdrawn / Not pursued |  |
| R4-2216614 |  | TP to 38.115-1: ACLR requirement | NEC | Withdrawn / Not pursued |  |
| R4-2215387 |  | TP for TS 38.115-1: scope and reference | CATT | Revised |  |
| R4-2216837 |  | TP to TS 38.115-1: Measurement uncertainties and test requirements (4.1) | Huawei, HiSilicon | Revised |  |
| R4-2216839 |  | TP to TS 38.115-1: Annex B: Environmental requirements for the repeater | Huawei, HiSilicon | Agreeable | The references can be handled by spec editor. |
| R4-2216840 |  | TP to TS 38.115-1: Annex C: Test tolerances and derivation of test requirements | Huawei, HiSilicon | Agreeable |  |
| R4-2216841 |  | TP to TS 38.115-1: Annex E: Characteristics of interfering signals | Huawei, HiSilicon | Agreeable | Interferer single information to be captured directly in the requirements sections. |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents