3GPP TSG-RAN WG4 Meeting # 104-bis-e												R4-2216914

Electronic Meeting, October 10 – 19, 2022

Agenda item:		     4.3.5

Source:	Moderator (Huawei)

Title:	Email discussion summary for [104-bis-e][203] NR_ext_to_71GHz_RRM_1

Document for:	Information

Introduction

This email discussion document focuses on maintenance of RRM requirements for NR operation extension to 71GHz including general requirements, timing requirements and impact of LBT on RRM requirements.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round:

  - General requirements

  - Timing requirements

  - LBT impact on RRM requirements

  - CR/Draft CR

- 2nd round: TBA

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Huawei, HiSilicon | Zhongyi Shen | shenzhongyi3@huawei.com |
| Apple | Steven Chen | steven.x.chen AT apple.com |
| LGE | Jin-Yup Hwang | jinyup.hwang@lge.com |
| Nokia | Erika Almeida | erika.almeida@nokia.com |
| CATT | Gao Lingyu | gaolingyu@catt.cn |
| Qualcomm | Prashant Sharma | prasshar@qti.qualcomm.com |
| vivo | Qian Yang | qian9.yang@vivo.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: General requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4- 2215416 | CATT | Proposal 1: Introducing UE capability on relaxed requirements is a more compromise method to make it easier for UEs of different levels to flexibly support various mobility cases without causing long cell detection delays for all UEs. |
| R4-2215799 | LG Electronics | - Proposal 1: Wait for the RAN1 reply LS in case there are no serving cells in FR2-2, and remove the “if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier” in the description of the case no TCI information. - Proposal 2: Clarify which serving cell carrier’s the latest received PDSCH and the latest monitored CORESET in the active BWP should be referred to when there are multiple serving cells on the FR2-2 band if no TCI state is provided in the RMTC configuration. - Proposal 3: Text proposal for inter-frequency RSSI measurement for an open issue is - If no TCI state is provided in the RMTC configuration UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the carrier in FR2-2 having the [lowest or highest] cell index among the serving cells for which corresponding TCI state ID is provided in another RMTC configuration. |
| R4-2216261 | Huawei, HiSilicon | Proposal 1: Update the SCell activation requirements for FR2-2 with beam sweeping scaling factor of 12. Proposal 2: Introduce UE capability on relaxed requirements for cell detection. |

Open issues summary

Sub-topic 1-1 QCL-ed assumption for RSSI

Background:

In RAN4#104e, following agreements were achieved with following FFS point, and an LS (R4-2214477) was sent to RAN1/2

Issue 1-2-2: Inter-frequency QCL-ed assumption Agreement in GTW: - The QCL reference serving cell is determined by serving cell ID indicated in TCI-StateInfo in RMTC-Config, when the TCI state is provided in the RMTC configuration. - For performing inter-frequency RSSI measurement in FR2-2, - if the TCI state is provided in RMTC configuration of FR2-2, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. - FFS on the following bullet - if no TCI state is provided in the RMTC configuration [or if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier] UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of [a or the] carrier in FR2-2. on which the RMTC configuration is provided - RAN4 to clarify the case with RAN1 when there is no serving cell in FR2-2. If a UE has no serving cell in FR2-2, it is not clear if that explicit TCI state should be configured to the UE for FR2-2 RSSI measurement. If explicit TCI state should be configured, how does the UE use such explicit TCI?

Issue 1-1-1: Inter-frequency QCL-ed assumption for RSSI

- Proposals

  - Option 1:  (LGE)

  - Wait for the RAN1 reply LS in case there are no serving cells in FR2-2, and remove the “if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier” in the description of the case no TCI information.

  - Clarify which serving cell carrier’s the latest received PDSCH and the latest monitored CORESET in the active BWP should be referred to when there are multiple serving cells on the FR2-2 band if no TCI state is provided in the RMTC configuration.

  - Test proposal:

    - If no TCI state is provided in the RMTC configuration UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the carrier in FR2-2 having the [lowest or highest] cell index among the serving cells for which corresponding TCI state ID is provided in another RMTC configuration.

- Recommended WF

  - Discuss above option with CR R4-2215800.

Comments collection

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1-1: |
| Huawei | Issue 1-1-1: We support the first bullet in section 1. RAN4 can wait for the LS reply from RAN1 for the case when there is no serving cell in FR2-2. This is based on the reasonable implementation that gNB will configure TCI to a FR2-2 if any. Thus, the case that TCI is provided in a FR1 serving is an unrealistic implementation. We don't agree with the second bullet. The motivation is that UE can utilize the beam of the latest cell when the beam switching may be not needed. But with the further restriction on cell index, it means the selected cell may not be the cell where the latest PDCCH/PDSCH is received. Thus, we propose following TP or we can leave it FFS until RAN1’s LS reply is received: if no TCI state is provided in the RMTC configuration] UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of a carrier in FR2-2 |
| Apple | We are OK with “• Wait for the RAN1 reply LS in case there are no serving cells in FR2-2, and remove the “if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier” in the description of the case no TCI information.” We don’t quite understand why use “the [lowest or highest] cell index”. |
| LGE | For the second bullet, we think the clarification is needed for which serving cell UE should assume the QCL TypeD to PDSCH or CORESET when there are multiple serving cells on FR2-2 carrier. So, we provided the text proposal of one of the possible clarifications, and lowest or highest cell index being an example from which serving cell could be selected. We are open to discuss how to define clear QCLed assumption when there are multiple serving cells. |
| Nokia | Issue 1-1-1: We are fine with waiting for RAN1 reply. |
| CATT | Agree with the first bullet and suggest to futher discuss the second bullet. |
| MTK | We can agree with the wording suggested by Huawei, i.e. - if no TCI state is provided in the RMTC configuration, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of a carrier in FR2-2. Waiting for RAN1 reply is also fine. |
| vivo | We agree with the first bullet. For the second bullet, we are fine to FFS. |

Sub-topic 1-2 Cell detection relaxation

Issue 1-2-1: Cell detection relaxation

- Proposals

  - Option 1: Introduce UE capability on relaxed requirements for cell detection. (CATT, Huawei)

- Recommended WF

  - Discuss above options.

Comments collection

| Company | Comments |
| --- | --- |
| XXX | Issue 1-2-1: |
| Huawei | Issue 1-2-1: We support option 1 as proponent. The issue was also discussed in last meeting. Companies against having the capability argued that RAN4 only define minimum requirements. From our views, it does not mean the requirements can be as relaxed as possible. As analysed in our paper, even at the worst case, the demand on UE processing and memory is increase by around 30%, but now the requirements is relaxed by 2 or 3 times. Our first preference is not relax the requirements and the to have a capability is a compromised solution. |
| Ericsson | Issue 1-2-1: Agree on Option 1. |
| Nokia | Issue 1-2-1: We are fine with Option 1. |
| CATT | Support option 1, which is a more compromise method to make it easier for UEs of different levels to flexibly support various mobility cases without causing long cell detection delays for all UEs. Some companies considered that we only need to define the minimum RAN4 requirements in the last meeting, we have the same view with Huawei and think it does not mean the requirements can be as relaxed as possible, the final implementation is also basically based on the defined requirements, so it is not a reason for objection. |
| Qualcomm | We don’t support option 1. The relaxation factors have already been agreed and they make sense from UE implementation point of view. There is no need to add additional complexity to the UE by introducing UE capabilities that are unlikely to be used in the field. If such a need ever arises, it could be discussed in future releases. At this moment, we think adding such a capability is unnecessary. |
| Huawei2 | The essential problem is that the delay requirements is over extended too much, which may have impacts on NW estimation/configuration on timetotrigger. Thus, we propose to have a capability which is a fair approach. It won’t put any further complexity to UE is one think the extension by 2/3 times is needed. They just simply indicate the extension is needed or (the relaxation is not needed). |
| vivo | It is not clear to us how NW will take use of the UE capability. Clarification is needed. |

Sub-topic 1-3 Imapct of beam sweeping factor

Issue 1-3-1:

- Proposals

  - Option 1: Update the SCell activation requirements for FR2-2 with beam sweeping scaling factor of 12.( Huawei)

- Recommended WF

  - Discuss above option with CR R4-2216263

Comments collection

| Company | Comments |
| --- | --- |
| XXX | Issue 1-3-1: |
| Huawei | Issue 1-3-1: We support option 1 and it is reflected in our CR 6263. |
| Ericsson | Issue 1-3-1: Agree on Option 1. |
| Apple | Option 1 is reasonable. |
| LGE | Support the proposal |
| Nokia | Issue 1-3-1: In general we support updating the Scell activation requirements. But since this requirement is being discussed in different Work Items, we would prefer not to modify the expression of the delay for the FR2-1 Ues. Therefore, we propose to create a separate formula for the FR2-2 requirements. |
| MTK | Agree on Option 1. |
| Qualcomm | Fine with the proposal |
| Huawei2 | We are fine with Nokia’s comments to have separate expression of the delay. |
| vivo | Fine with the proposal. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2215800 (LGE) CR on QCL-ed assumption for inter-frequency RSSI measurement in FR2-2 | Huawei: Depends on conclusion in issue 1-1-1 |
|  | Company B |
| R4-2216262 (Huawei, HiSilicon) CR on RLM requirements for FR2-2 |  |
| R4-2216263 (Huawei, HiSilicon) CR on SCell activation requirements of FR2-2 | Nokia: In general we support updating the Scell activation requirements. But since this requirement is being discussed in different Work Items, we would prefer not to modify the expression of the delay for the FR2-1 UEs. Therefore, we propose to create a new formula for the FR2-2 requirements. |
| R4-2216882 (Qualcomm Incorporated) Draft CR on Measurement Procedures | Huawei: Could QC clarify why 9.3.9.2 is removed? For table 9.3.9.1-4, it seems index detection is added for FR2 which seems not aligned with agreement of inter-frequency without gap. For samples number for detection, it depends on issue 1-2-1. Nokia. Not agreeable, we have the same comment as Huawei: why clause 9.3.9.2 is removed? Qualcomm: We don’t intend to remove clause 9.3.9.2. We submitted changes only for 9.3.9.1, but we erroneously deleted clause 9.3.9.2, which we can fix during the second round. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 | 7 companies commented in the 1st round. All companies agreed with waiting LS from RAN1 when there is no serving cell in FR2-2. And companies have different views on the text proposals. Tentative agreements: - Wait for the RAN1 reply LS in case there are no serving cells in FR2-2, and remove the “if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier” in the description of the case no TCI information. Candidate options: Text proposal: - Option 1: If no TCI state is provided in the RMTC configuration UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the carrier in FR2-2 having the [lowest or highest] cell index among the serving cells for which corresponding TCI state ID is provided in another RMTC configuration. - Option 2 (new): if no TCI state is provided in the RMTC configuration, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of a carrier in FR2-2. Recommendations for 2nd round: Discuss above two options in the CR. |
| Sub-topic#1-2 | 6 companies commented in the 1st round. 4 companies support to introduce the capability, one company disagree and one company want to FFS. Tentative agreements: NA Candidate options: Option 1: Introduce UE capability on relaxed requirements for cell detection. Recommendations for 2nd round: Keep discussion in 2nd round. |
| Sub-topic#1-3 | 8 companies commented in the 1st round, and all agree to update the beam sweeping factor for SCell activation. Tentative agreements: Update the SCell activation requirements for FR2-2 with beam sweeping scaling factor of 12 Candidate options: NA Recommendations for 2nd round: Discuss the CR directly. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2215800 (LGE) CR on QCL-ed assumption for inter-frequency RSSI measurement in FR2-2 | to be revised |
| R4-2216262 (Huawei, HiSilicon) CR on RLM requirements for FR2-2 | agreeable |
| R4-2216263 (Huawei, HiSilicon) CR on SCell activation requirements of FR2-2 | to be revised |
| R4-2216882 (Qualcomm Incorporated) Draft CR on Measurement Procedures | to be revised |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #2: LBT impact on RRM requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4- 2215416 | CATT | Observation 1: The SMTC occasion group or DRX group duration depends on the SMTC and DRX configurations of each UE and the RX beam sweeping factor, while the SMTC and DRX configurations depend on the mobility scenarios. Observation 2: The RX beam sweeping factor in FR2-2 has also changed, SMTC occasion group or DRX group duration is also different from that before. Proposal 4: In addition to the fixed maximum separation time, option 2 also takes DRX occasion group duration into account when configuring DRX is more reasonable. |
| R4-2215618 | Apple | Proposal 1: More discussion on Option 1 is needed for better understanding and comparison with Option 2. Proposal 2: RAN4 to adopt Option 2 on how to specify the maximum separation between two consecutive measurements. Proposal 3: LBT assumptions on neighbour cell measurement are known to the UE based on the RAN2 LS, and the issue is closed. |
| R4-2216257 | Nokia, Nokia Shanghai Bell | The requirements with CCA apply if the UE is signaled that the neighbor cells are configured with CCA, and requirements without CCA apply otherwise. Send a reply LS to RAN2 informing that the UE shall receive indication about the channel access mode in use by neighbor cells. The requirement apply provided any two measurement shall not be separated in time by more than the maximum time requirement for the cell to remain known. |
| R4-2216264 | Huawei, HiSilicon | Observation 1: RAN2 introduce explicit indications of neighbour cell’s channel access mode for IDLE mode and CONNCECTED mode neighbour cell measurement. Proposal 1: UE determines the CCA mode of a neighbour cell according to the indication of channelAccessMode2-r17, and requirements for CCA apply when channelAccessMode2-r17 is enabled. Observation 2: RSSI/CO measurement is performed on per OFDM symbols basis. Proposal 2: For RSSI/CO measurement with SCS of 480 kHz and 960 kHz, the RSSI/CO measurement results shall be derived based on symbols configured by RMTC except for the first symbol and the last symbol in RMTC. Observation 3: The specification about QCL assumptions when there is no FR2-2 serving cell can be updated with RAN1’s feedback and clarifications. |
| R4-2216606 | Nokia, Nokia Shanghai Bell | Reply LS on signalling of CCA configurations of neighbour cells in FR2-2 |

Open issues summary

Sub-topic 2-1 maximum separation between two consecutive measurements

Issue 2-1-1:

- Proposals

  - Option 1: The requirement apply provided any two measurement shall not be separated in time by more than the maximum time requirement for the cell to remain known. (Nokia)

  - Option 2: (Apple, CATT)

The requirement only applies when

- Within the set of measurements any two measurements shall not be separated in time by more than 2 seconds when no DRX is configured, and

- Within the set of measurements any two measurements shall not be separated in time by more than max(1 DRX occasion group duration, 2 seconds) when DRX is configured.

- Recommended WF

  - Discuss above options.

Comments collection

| Company | Comments |
| --- | --- |
| XXX | Issue 2-1-1: |
| Huawei | Fine with option 1 and 2 |
| Ericsson | Issue 2-1-1: We support Option1, and a exact number can be listed. |
| Apple | We support Option 2. For Option 1, as raised in our paper, we don’t quite understand what “the maximum time requirement for the cell to remain known” in FR2-2. Also, as discussed in Nokia’s paper, Option 1 leads to long time between two measurements, which is undesirable as it is more likely that a FR2-2 UE may move a bit or rotate, thereby making the two measurements hard to combine. |
| Nokia | We prefer option 2, but are fine with option 1 and option 2. We proposed option 1 as a compromise to the discussions in the last meeting, since it was supported by the majority. In many cases both proposals will lead to similar requirements. |
| CATT | We support Option 2. The SMTC occasion group or DRX group duration depends on the SMTC and DRX configurations of each UE and the RX beam sweeping factor, and the SMTC and DRX configurations depend on the mobility scenarios. In FR2-2, various channel conditions will also change more significantly, we need to consider different mobility scenarios as one of the factors. |
| vivo | We are in general fine with option 1 to have a fixed value, but would like to clarify the “the maximum time requirement for the cell to remain known”. Which requirements are referring to exactly and what is the value? |

Sub-topic 2-2 LBT assumptions on neighbour cell measurement

Issue 2-2-1:

- Proposals

  - Option 1: The requirements with CCA apply if the UE is signalled that the neighbour cells are configured with CCA, and requirements without CCA apply otherwise. (Apple, Nokia, Huawei)

- Recommended WF

  - Agree on option 1 and discuss the CR R4-2216256/R4-2216265.

  - Companies please comment on LS reply (R4-2216256)

Comments collection

| Company | Comments |
| --- | --- |
| XXX | Issue 2-2-1: |
| Huawei | Support option 1. We don't think the LS is needed. The signal is already reflected in RAN2 spec. |
| Ericsson | Issue 2-2-1: We suppose Option 1 is correct interpretation to feedback by RAN2, and OK with CRs. |
| Apple | We agree with Option 1. We wonder if the LS is needed, as RAN2 is not requesting any info from RAN4. |
| Nokia | Support option 1. Support sending the LS since RAN2 only endorsed the draft CRs and not implemented them yet. In RAN4 LS last meeting we only asked if it was feasible to indicate the channel access mode to the UE and RAN2 concluded that it is feasible. |
| CATT | Agree with option 1. We also think it is not necessary to send the LS to RAN2. |
| MTK | We agree with Option 1. Fine with sending LS as it can clarify its impact on requirements. |
| Qualcomm | Fine with option 1 and don’t really see a need to send an LS. |
| Huawei2 | After internally check, the changes in endorsed CR now, so we are fine to send the LS. |
| vivo | Agree with option 1. Not clear why reply LS is needed. There would be no RAN2 impact. |

Sub-topic 2-3 Imapct of 480/960 kHz on RSSI/CO measurement

Issue 2-3-1:

- Proposals

  - Option 1: For RSSI/CO measurement with SCS of 480 kHz and 960 kHz, the RSSI/CO measurement results shall be derived based on symbols configured by RMTC except for the first symbol and the last symbol in RMTC. (Huawei)

- Recommended WF:

  - Discuss above option.

Comments collection

| Company | Comments |
| --- | --- |
| XXX | Issue 2-3-1: |
| Huawei | Issue 2-3-1: We support option 1. The beam switching may impact the per-symbol RSSI/CO measurement results for 480/960 SCS. |
| Ericsson | Issue 2-3-1: We’re fine with Option1 |
| Apple | We agree with the reasoning for Option 1. At the same time, we wonder usually how many symbols RSSI/CO measurement would consider. If there is quite a few, it may not matter much by considering or not considering the first and last symbol, because RSSI/CO are simply power measurements. |
| LGE | Similar to Apple’s comment, RSSI is total received power in configured symbols, so we think that the beam switching time would not significant affect RSSI measurement. |
| CATT | The same concern with Apple. We should check usually how many symbols RSSI/CO measurement would consider. |
| MTK | Fine with Option 1. |
| Qualcomm | Fine with option 1 |
| Huawei2 | To Apple, LGE and CATT. The problem is the CO measurement based on RSSI. The CO results is counted by number of OFDM symbols whose measured RSSI is above threshold. It is possible that the power estimated in the first and last symbols is always underestimated, and they are considered as below the threshold, then the percentage of CO results reported are also less than the real CO. |
| vivo | If beam sweeping has impact on RSSI measurements, we wonder if it has impact on other L1 measurements. This needs further discussion. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2216265 ((Huawei, HiSilicon) CR on LBT assumption for FR2-2 | Nokia: we are generally fine with this CR. The idea is the same as in R4-22166256 (we prefer 6256 since we consider it to be future proof). We believe that if we adopt the approach in this CR; we need to carefully check the specification. For example, in our view handover clause would also need to be included here. |
|  | Company B |
| R4-2216266 (Huawei, HiSilicon) CR on RSSI measurement for FR2-2 | Company A |
|  | Company B |
| R4-2216256 (Nokia, Nokia Shanghai Bell) CR on applicability of RRM requirements with CCA in FR2-2 | Huawei: Generally fine. We think it is similar with our CR 6265. We slightly prefer CR 6265 where the applicability is stated in each requirements as R16 NR-U which is clearer. |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | 6 companies commented in the 1st round. There is no clear majority views. Tentative agreements:NA Candidate options: - Option 1: The requirement apply provided any two measurement shall not be separated in time by more than the maximum time requirement for the cell to remain known. - Option 2: The requirement only applies when - Within the set of measurements any two measurements shall not be separated in time by more than 2 seconds when no DRX is configured, and - Within the set of measurements any two measurements shall not be separated in time by more than max(1 DRX occasion group duration, 2 seconds) when DRX is configured. Recommendations for 2nd round: Keep discussion in 2nd round. It is encouraged to consider compromised solution to close the issue. |
| Sub-topic#2-2 | 8 companies commented in the 1st round, and all agree with option 1. But companies have different views on whether to send the LS to RAN2. Tentative agreements: The requirements with CCA apply if the UE is signalled that the neighbour cells are configured with CCA, and requirements without CCA apply otherwise Candidate options:NA Recommendations for 2nd round: Discuss how to capture the agreement in the CR. Discuss whether to send the LS in the dedicated thread for the LS. |
| Sub-topic#2-3 | Tentative agreements: 8 companies commented in the 1st round, 4 companies support option 1 and 4 companies want to FFS. Candidate options: - Option 1: For RSSI/CO measurement with SCS of 480 kHz and 960 kHz, the RSSI/CO measurement results shall be derived based on symbols configured by RMTC except for the first symbol and the last symbol in RMTC. Recommendations for 2nd round: Keep discussion in 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2216265 ((Huawei, HiSilicon) CR on LBT assumption for FR2-2 | to be revised |
| R4-2216266 (Huawei, HiSilicon) CR on RSSI measurement for FR2-2 | to be revised |
| R4-2216256 (Nokia, Nokia Shanghai Bell) CR on applicability of RRM requirements with CCA in FR2-2 | Merged |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on core requirements maintenance for FR2-2 | Huawei, HiSilicon | To capture agreements in this meeting |
|  | LS on capability of relaxed cell detection requirements for FR2-2 | Huawei, HiSilicon | To: RAN2 Reserved which depends on the conclusion of issue 1-2-1. |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2215800 |  | CR on QCL-ed assumption for inter-frequency RSSI measurement in FR2-2 | LGE | Revised |  |
| R4-2216262 |  | CR on RLM requirements for FR2-2 | Huawei, HiSilicon | agreeable |  |
| R4-2216263 |  | CR on SCell activation requirements of FR2-2 | Huawei, HiSilicon | Revised |  |
| R4-2216882 |  | Draft CR on Measurement Procedures | Qualcomm Incorporated | Revised |  |
| R4-2216265 |  | CR on LBT assumption for FR2-2 | Huawei, HiSilicon | Revised |  |
| R4-2216266 |  | CR on RSSI measurement for FR2-2 | Huawei, HiSilicon | Revised |  |
| R4-2216256 |  | CR on applicability of RRM requirements with CCA in FR2-2 | Nokia, Nokia Shanghai Bell | Merged |  |
| R4-2216606 |  | Reply LS on signalling of CCA configurations of neighbour cells in FR2-2 | Nokia, Nokia Shanghai Bell | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents