3GPP TSG-RAN WG4 Meeting # 104-bis-e												     R4-2216951

Electronic Meeting, 10– 19 October 2022

Agenda item:			5.18,5.21

Source:	Moderator (Huawei, HiSilicon)

Title:	Email discussion summary for [104-bis-e][114] HPUE_Basket_Intra-CA_TDD

Document for:	Information

Introduction

Thread [114] includes following topics:

1.	Topic #1: Issues for HPUE_NR_FR1_TDD_intra_CA_R18 (Agenda 5.18)

2.	Topic #2: Issues for HPUE_NR_FR1_TDD_R18 (Agenda 5.21)

It is appreciated that the delegates for this topic put their contact information in the table below.

# Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Huawei (JW) | Jin Wang | jinwang@huawei.com |
| Huawei, HiSilicon | Lingyu Kong | konglingyu4@hisilicon.com |
| Skyworks Solutions Inc. | Dominique Brunel | domnique.brunel@skyworksinc.com |
| CMCC | Chunxia Guo | guochunxia@chinamobile.com |
| Apple | James Wang | fucheng_wang@apple.com |
| AT&T | Ron Borsato | ronald.borsato@att.com |
| vivo | Ziqi Liu | liuziqi@vivo.com |
| T-Mobile USA | Bill Shvodian | bill.shvodian@t-mobile.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: HPUE_NR_FR1_TDD_intra_CA_R18 (5.18)

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2216080 | Huawei, HiSilicon | Proposal 1: only PC3 CA_n77C with a single uplink component carrier configured in n77 has been specified in the current spec, PC2 seems to need to be introduced as well, which needs to be further checked by the corresponding operators. Proposal 2: Considering that NS_55 and NS_57 with N/A A-MPR only represents the signalling procedure for PC3 specified in current spec, whether NS_55 and NS_57 with N/A A-MPR are applicable for PC1.5 need to be further checked by companies Proposal 3: PC1.5 CA_n77C with a single uplink component carrier configured in n77 needs to be added in Table 5.5A.1-1. Proposal 4: UE maximum output power for PC1.5 CA_n77C with a single uplink component carrier configured in n77 can be considered as 29dBm with +2/-3dB tolerance. Observation 1: A-MPR does not needs to be considered for PC1.5 CA_n78(2A) with a single uplink component carrier configured in n78. Proposal 5: PC1.5 CA_n78(2A) with a single uplink component carrier configured in n78 need to be added in Table 5.5A.2-1. Proposal 6: UE maximum output power for PC1.5 CA_n78(2A) with a single uplink component carrier configured in n78 can be considered as 29dBm with +2/-3dB tolerance. |
| R4-2216081 | Huawei, HiSilicon | Observation 1: A-MPR does not needs to be considered for PC2 CA_n77(2A). Proposal 1: UE maximum output power for PC2 CA_n77(2A) can be considered as 26dBm with +2/-3dB tolerance Observation 2: A-MPR does not needs to be considered for PC2 CA_n78(2A) with a single uplink component carrier configured in n78. Proposal 2: PC2 CA_n78(2A) with a single uplink component carrier configured in n78 should be added in Table 5.5A.2-1. Proposal 3: UE maximum output power for PC2 CA_n78(2A) with a single uplink component carrier configured in n78 can be considered as 26dBm with +2/-3dB tolerance. |
| R4-2216082 | Huawei, HiSilicon | According to MCC guidance, some wording in this revised WID is revised, which need to be further checked in RAN4, especially the objective. Views and comments from companies are collected to facilitate the discussion in RAN. |
| R4- 2216083 | Huawei, HiSilicon | This draft CR adds the requested intra-band CA combinations with PC2 and PC1.5. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: CA_n77(2A) with PC2 for UL CA_n77(2A)

Sub-topic description: handle the band-combination specific requirements

| NR CA configuration | Uplink CA configuration | Power class |
| --- | --- | --- |
| CA_n77(2A) | CA_n77(2A) | PC2 |

Open issues and candidate options before e-meeting:

Issue 1-1-1: A-MPR

- Options:

  - Option 1: No need to be considered.

  - Option 2: Specify other option if any.

- Recommended WF

  - TBA

Issue 1-1-2: UE maximum output power with tolerance

- Options:

  - Option 1: 26dBm with +2/-3dB.

  - Option 2: Specify other option if any.

- Recommended WF

  - TBA

Comments collection

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Issue 1-1-1: Option 1 Issue 1-1-2: Option 1 |
| Skyworks | Issue 1-1-1: A-MPR: in our understanding the request is for n77 UL configuration not n77(2A). For n77 no A-MPR applies: option 1 Issue 1-1-2: UE maximum output power with tolerance. 26dBm with +2/-3dB option 1 |
| Apple | Issue 1-1-1: A-MPR Option 1 Issue 1-1-2: UE maximum output power with tolerance Option 1 |
| Huawei, HiSilicon | Issue 1-1-1: Clarification to Skyworks: The request is UL CA_n77(2A) rather than single uplink carrier according to the Table 1 of WID RP- 222647. |
| AT&T | Issue 1-1-1: A-MPR Option 1. Issue 1-1-2: UE maximum output power with tolerance Option 1. |

Sub-topic 1-2: CA_n78(2A) with PC2 for single uplink carrier in n78

Sub-topic description: handle the band-combination specific requirements

| NR CA configuration | Uplink CA configuration | Power class |
| --- | --- | --- |
| CA_n78(2A) | n78A | PC2 |

Open issues and candidate options before e-meeting:

Issue 1-2-1: A-MPR

- Options:

  - Option 1: No need to be considered.

  - Option 2: Specify other option if any.

- Recommended WF

  - TBA

Issue 1-2-2: UE maximum output power with tolerance

- Options:

  - Option 1: 26dBm with +2/-3dB.

  - Option 2: Specify other option if any.

- Recommended WF

  - TBA

Issue 1-2-3: Introduce this band combination

- Proposal: CA_n78(2A) with PC2 for single uplink carrier in n78 need to be added in Table 5.5A.2-1 of TS38.101-1.

- Recommended WF

  - TBA

Comments collection

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Issue 1-2-1: Option 1 Issue 1-2-2: Option 1 Issue 1-2-3: Support. |
| Skyworks | Issue 1-2-1: A-MPR: in our understanding the request is for n78 UL configuration not n78(2A). For n78 no A-MPR applies: option 1 Issue 1-2-2: UE maximum output power with tolerance. 26dBm with +2/-3dB option 1 Issue 1-2-3: ok to introduce in in Table 5.5A.2-1 of TS38.101-1. |
| Apple | Issue 1-2-1: A-MPR Option 1 Issue 1-2-2: UE maximum output power with tolerance Option 1 Issue 1-2-3: Introduce this band combination We are okay with the proposal. |

Sub-topic 1-3: CA_n77C with PC1.5 for single uplink carrier in n77

Sub-topic description: handle the band-combination specific requirements

| NR CA configuration | Uplink CA configuration | Power class |
| --- | --- | --- |
| CA_n77C | n77A | PC1.5 |

Open issues and candidate options before e-meeting:

Issue 1-3-1: A-MPR

- Proposals: Whether NS_55 and NS_57 with N/A A-MPR are applicable for PC1.5.

  - Option 1: Yes.

  - Option 2: No. Please specify the issues

- Recommended WF

  - TBA

Issue 1-3-2: UE maximum output power with tolerance

- Options:

  - Option 1: 29dBm with +2/-3dB.

  - Option 2: Specify other option if any.

- Recommended WF

  - TBA

Issue 1-3-3: Introduce this band combination

- Proposal: CA_n77C with PC1.5 for a single uplink component carrier in n77 needs to be added in Table 5.5A.1-1 of TS38.101-1.

- Recommended WF

  - TBA

Issue 1-3-4: Introduce this band combination with PC2

- Proposal: only PC3 CA_n77C with a single uplink component carrier n n77 has been specified in the current spec, PC2 seems to need to be introduced as well, which needs to be further checked by the corresponding operators.

- Recommended WF

  - TBA

Comments collection

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Issue 1-3-1: Option 1 Issue 1-3-2: Option 1 Issue 1-3-3: Support. Issue 1-3-4: It depends on the operator request. But from the spec perspective, PC2 is always introduced before PC1.5. |
| Skyworks | Issue 1-3-1: A-MPR: NS_55 and 57 only correspond to spectrum restriction not A-MPR Issue 1-3-2: UE maximum output power with tolerance. 29dBm with +2/-3dB option 1 Issue 1-3-3: ok to introduce PC1.5 Issue 1-3-4: ok to introduce PC2 too |
| Apple | Issue 1-3-1: A-MPR Option 1: Yes Issue 1-3-2: UE maximum output power with tolerance Option 1 Issue 1-3-3: Introduce this band combination We are okay with the proposal. Issue 1-3-4: Introduce this band combination with PC2 We are okay with the proposal. It seems to be reasonable to introduce PC2 before PC1.5. |
| ### ZTE | Issue 1-3-4: Introduce this band combination with PC2 Although we think it is reasonable to introduce PC2, PC1.5 could not be included into the spec without PC2 is introduced first or at the same time. We should solve PC2 requests first. There are some discussions on the fallback rule in thread #126. |
| ### AT&T | Issue 1-3-1: A-MPR Option 1. In fact, NS_55 and NS_57 are used for barring purposes only. As such, they do not indicate any additional spurious emission and maximum output power reduction requirements. Issue 1-3-4: Introduce this band combination with PC2 We should follow RAN4 rules on introduction of band combinations with higher power classes. We would need to specify PC2 prior to PC1.5. Although, it seems that many higher order combinations containing n77C with a single uplink component carrier in n77 supporting both PC2 and PC1.5 have already been added in the Rel-17 specification which seems to violate the RAN4 rules. Perhaps, the PC1.5 note was added in error on those higher order combinations and should be removed. A Rel-17 maintenance CR is required to fill the gap to introduce CA_n77C with a single uplink component carrier in n77 with PC2 as a workaround and to also remove the PC1.5 note on the higher order combinations. |
| ### T-Mobile USA | CA_n77C with uplink n77A PC2 and PC1.5 has already been added to 38.101 following the RAN4 process. It was requested and added to the Release 17 HPUE WID. Issue 1-3-1: A-MPR Option 1. We agree with AT&T. NS_55 and NS_57 are used for barring purposes only. As such, they do not indicate any additional spurious emission and maximum output power reduction requirements. Issue 1-3-2: UE maximum output power with tolerance There is no need for this. The single band n77 maximum output tolerance is already specified. Issue 1-3-3: Introduce this band combination CA_n77C with UL n77 PC2 is not yet in the Table 5.5A.1-1. Was it in an endorsed CR from August? If not, PC2 needs to be added. Issue 1-3-4: Introduce this band combination with PC2 We agree with AT&T that PC2 needs to be added first. We also agree with AT&T that the higher order combinations with DL n77C and UL n77 PC2 and PC1.5 did not follow the proper process and should probably be removed. |

Sub-topic 1-4: CA_n78(2A) with PC1.5 for single uplink carrier in n78

Sub-topic description: handle the band-combination specific requirements

| NR CA configuration | Uplink CA configuration | Power class |
| --- | --- | --- |
| CA_n78(2A) | n78A | PC1.5 |

Open issues and candidate options before e-meeting:

Issue 1-4-1: A-MPR

- Options:

  - Option 1: No need to be considered.

  - Option 2: Specify other option if any.

- Recommended WF

  - TBA

Issue 1-4-2: UE maximum output power with tolerance

- Options:

  - Option 1: 29dBm with +2/-3dB.

  - Option 2: Specify other option if any.

- Recommended WF

  - TBA

Issue 1-4-3: Introduce this band combination

- Proposal: CA_n78(2A) with PC1.5 for single uplink carrier in n78 need to be added in Table 5.5A.2-1 of TS38.101-1.

- Recommended WF

  - TBA

Comments collection

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Issue 1-4-1: Option 1 Issue 1-4-2: Option 1 Issue 1-4-3: Support. |
| Skyworks | Issue 1-4-1: A-MPR: no A-MPR for n78 Issue 1-4-2: UE maximum output power with tolerance. 29dBm with +2/-3dB option 1 Issue 1-4-3: ok to introduce PC1.5 |
| Apple | Issue 1-4-1: A-MPR Option 1 Issue 1-4-2: UE maximum output power with tolerance Option 1 Issue 1-4-3: Introduce this band combination We are okay with the proposal. PC2 may also need to be introduced at the same time. |

Sub-topic 1-5: Revised WID

Sub-topic description: According to MCC guidance, some wording in this revised WID is revised, which need to be further checked in RAN4, especially the objective. Views and comments from companies are collected to facilitate the discussion in RAN.

Issue 1-5-1: Revised WID

- Recommended WF

  - Views and comments on R4-2216082 are collected to facilitate the discussion in RAN.

- Comments collection

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Fine with the revised wording. |
| Skyworks | OK as long as ULCA in PC1.5 is not requested wo the proper MPR work needed |
| AT&T | OK with the proposed WID revision. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4- 2216083: Draft CR on TS38.101-1 Addition of intra-band CA Combinations with PC2 and PC1.5 | Huawei, HiSilicon: It depends on the 1st round discussion outcomes. |
|  | Skyworks: CR OK in principle |
|  | Apple: Okay with the draft CR. PC2 may also need to be introduced for n77 in DL CA_n77C. |
|  | ZTE: As we commented in issue 1-3-4, PC1.5 could not be included into the spec without PC2 is introduced first or at the same time. We should solve PC2 requests first. Also, using one draft CR capture a full picture for a band combination is recommended. |
|  | AT&T: OK with the draft CR. However, we also think that there is a necessary Rel-17 maintenance CR in November to introduce CA_n77C with a single uplink component carrier in n77 with PC2. Otherwise, the higher-order combinations containing CA_n77C with a single uplink component carrier in n77 with PC2 should be removed from the Rel-17 spec since the fallback is not supported. Based on our comment above, we think that the higher-order combinations containing CA_n77C with a single uplink component carrier in n77 with PC1.5 should be removed from the Rel-17 spec since this case is covered in the Rel-18 basket WI. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 1-1: CA_n77(2A) with PC2 for UL CA_n77(2A) | Issue 1-1-1: A-MPR Summary of round 1 discussion: 3 companies are OK with Option1. 1 company thinks the request is for n77 UL configuration not n77(2A). Tentative agreement: no A-MPR for UL CA_n77(2A) Recommendations for 2nd round: confirm the tentative agreement and capture the agreement in WF. Issue 1-1-2: UE maximum output power with tolerance Summary of round 1 discussion: 3 companies are OK with Option1. Tentative agreement: 26dBm with +2/-3dB Recommendations for 2nd round: confirm the tentative agreement and capture the agreement in WF. |
| Sub-topic 1-2: CA_n78(2A) with PC2 for single uplink carrier in n78 | Issue 1-2-1: A-MPR Summary of round 1 discussion: 3 companies are OK with Option1. Tentative agreement: no A-MPR for CA_n78(2A) with PC2 for single uplink carrier in n78 Recommendations for 2nd round: confirm the tentative agreement and capture the agreement in WF. Issue 1-2-2: UE maximum output power with tolerance Summary of round 1 discussion: 3 companies are OK with Option1. Tentative agreement: 26dBm with +2/-3dB Recommendations for 2nd round: confirm the tentative agreement and capture the agreement in WF. Issue 1-2-3: Introduce this band combination Summary of round 1 discussion: 3 companies are OK with the proposal. Tentative agreement: CA_n78(2A) with PC2 for single uplink carrier in n78 need to be added in Table 5.5A.2-1 of TS38.101-1. Recommendations for 2nd round: confirm the tentative agreement and capture the agreement in WF. |
| Sub-topic 1-3: CA_n77C with PC1.5 for single uplink carrier in n77 | Issue 1-3-1: A-MPR Summary of round 1 discussion: 4 companies are OK with Option1. Tentative agreement: no A-MPR for CA_n77C with PC1.5 for single uplink carrier in n77 Recommendations for 2nd round: confirm the tentative agreement and capture the agreement in WF. Issue 1-3-2: UE maximum output power with tolerance Summary of round 1 discussion: 3 companies are OK with Option1. Tentative agreement: 29dBm with +2/-3dB Recommendations for 2nd round: confirm the tentative agreement and capture the agreement in WF. Issue 1-3-3: Introduce this band combination Summary of round 1 discussion: 3 companies are OK with the proposal. 3 companies think that PC1.5 could not be included into the spec without PC2 is introduced first or at the same time. Moderator suggests following the RAN4 rule. Tentative agreement: Focus on PC2 and no consider PC1.5 in RAN4#104bis e-meeting. Recommendations for 2nd round: confirm the tentative agreement and capture the agreement in WF. Issue 1-3-4: Introduce this band combination with PC2 Summary of round 1 discussion: 3 companies are OK with the proposal. 2 companies reminder that many higher order combinations containing n77C with a single uplink component carrier in n77 supporting both PC2 and PC1.5 have already been added in the Rel-17 specification which seems to violate the RAN4 rules. Moderator suggests further discussing the feasibility of the AT&T comment. Candidate options: - Option 1: a Rel-17 maintenance CR is required to fill the gap to introduce CA_n77C with a single uplink component carrier in n77 with PC2 as a workaround and to also remove the PC1.5 note on the higher order combinations. - Option 2: Specify other option if any. Recommendations for 2nd round: continue to discuss candidate options and capture the agreement in WF. |
| Sub-topic 1-4: CA_n78(2A) with PC1.5 for single uplink carrier in n78 | Issue 1-4-1: A-MPR Summary of round 1 discussion: 3 companies are OK with Option1. Tentative agreement: no A-MPR for CA_n78(2A) with PC1.5 for single uplink carrier in n78 Recommendations for 2nd round: confirm the tentative agreement and capture the agreement in WF. Issue 1-4-2: UE maximum output power with tolerance Summary of round 1 discussion: 3 companies are OK with Option1. Tentative agreement: 29dBm with +2/-3dB Recommendations for 2nd round: confirm the tentative agreement and capture the agreement in WF. Issue 1-4-3: Introduce this band combination Summary of round 1 discussion: 3 companies are OK with the proposal. As comments in Issue 1-3-3, Moderator suggests following the RAN4 rule. Tentative agreement: Focus on PC2 and no consider PC1.5 in RAN4#104bis e-meeting. Recommendations for 2nd round: confirm the tentative agreement and capture the agreement in WF. |
| Sub-topic 1-5: Revised WID | Issue 1-5-1: Revised WID Summary of round 1 discussion: 3 companies are OK with the revision, which will be decided in RAN-P. Recommendations for 2nd round: No further discussion |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4- 2216083: Draft CR on TS38.101-1 Addition of intra-band CA Combinations with PC2 and PC1.5 | to be revised |

Discussion on 2nd round

Sub-topic 1-3: CA_n77C with PC1.5 for single uplink carrier in n77

Sub-topic description: handle the band-combination specific requirements

| NR CA configuration | Uplink CA configuration | Power class |
| --- | --- | --- |
| CA_n77C | n77A | PC1.5 |

Issue 1-3-3: Introduce this band combination

Summary of round 1 discussion: 3 companies are OK with the proposal. 3 companies think that PC1.5 could not be included into the spec without PC2 is introduced first or at the same time. Moderator suggests following the RAN4 rule.

Recommendations for 2nd round: confirm the tentative agreement.

- Focus on PC2 and no consider PC1.5 in RAN4#104bis e-meeting.

Comments collection

| Company | Comments |
| --- | --- |

Issue 1-3-4: Introduce this band combination with PC2

Summary of round 1 discussion: 3 companies are OK with the proposal. 2 companies reminder that many higher order combinations containing n77C with a single uplink component carrier in n77 supporting both PC2 and PC1.5 have already been added in the Rel-17 specification which seems to violate the RAN4 rules. Moderator suggests further discussing the feasibility of the AT&T comment.

Recommendations for 2nd round: continue to discuss candidate options.

- Option 1: a Rel-17 maintenance CR is required to fill the gap to introduce CA_n77C with a single uplink component carrier in n77 with PC2 as a workaround and to also remove the PC1.5 note on the higher order combinations.

- Option 2: Specify other option if any.

Comments collection

| Company | Comments |
| --- | --- |

Sub-topic 1-4: CA_n78(2A) with PC1.5 for single uplink carrier in n78

Issue 1-4-3: Introduce this band combination

Summary of round 1 discussion: 3 companies are OK with the proposal. As comments in Issue 1-3-3, Moderator suggests following the RAN4 rule.

Recommendations for 2nd round: confirm the tentative agreement.

- Focus on PC2 and no consider PC1.5 in RAN4#104bis e-meeting.

Comments collection

| Company | Comments |
| --- | --- |

CRs/TPs comments collection

Considering the commemts on the RAN4 rule in the 1st round discussion, the PC1.5 part is removed and the tentative agreements are captured in the revised draft CR. Regarding the comment ” using one draft CR capture a full picture for a band combination is recommended” from ZTE, it is a good suggestion. Considering no AMPR issues, moderator suggest combining the two PC2 band combinations together for simplicity.

Comments collection

| CR/TP number | Comments collection |
| --- | --- |
| Rev. of R4- 2216083: Draft CR on TS38.101-1 Addition of intra-band CA Combinations with PC2 and PC1.5 |  |

Other

If you have any comments on the tentative agreements in 1.3 Summary for 1st round, except the issues mentioned in 1.4.1, 1.4.2 and 1.4.3, pleases write down below.

Comments collection

| Company | Comments |
| --- | --- |

Topic #2: HPUE_NR_FR1_TDD_R18 (5.21)

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215330 | Skyworks Solutions, Inc. | Proposal: - No additional A-MPR work for PC1.5 n34 and n40, PC1.5 can be added directly to those bands. - PC1.5 n39 NS_50 A-MPR is needed for smartphone UE, 10dB antenna isolation is used like for the related PC1.5 MPR. Proposal for PC1.5 NS_50 A-MPR: - For PC1.5, the worst case PC2 regions A1 and A2 need 1.5dB higher back-off to account for 3dB higher emissions per antenna and the additional contribution from RIMD. - A-MPR for 5MHz should be investigated. - New or different A-MPR regions may need to be defined. |
| R4-2215509 | CMCC | This draft CR adds power class 1.5 requirement of UE power class for the corresponding NR TDD bands in the new WI. |
| R4-2215853 | CMCC | This paper is TR skeleton for Rel-18 basket Work Item on “High power for FR1 TDD single bands with power class 1.5 UE” |
| R4-2216123 | vivo | Proposal 1: The maximum out power and tolerance of PC1.5 UE in n34, n39 and n40 is proposed as the following table: NR band; Class 1 (dBm); Tolerance (dB); Class 1.5 (dBm); Tolerance (dB). n34; 295; +2/-3. n39; 295; +2/-3. n40; 295; +2/-33. Proposal 2: To simplify the specification and align the current RAN2 spec, propose to restrict maxUplinkDutyCycle-PC2-FR1 to PC2 UE only, maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 for PC1.5 UE only, and not to differentiate UE type “- else if the UE does not support a power class with higher maximum output power than PC2; or - if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or - - if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or” Proposal 3: Alternative solution: add maxUplinkDutyCycle-PC1dot5 for PC1.5 handheld UE and restrict maxUplinkDutyCycle-PC2-FR1 to PC2 UE only. |
| R4- 2216775 | Huawei, HiSilicon | Proposal 1: Evaluate the PC1.5 A-MPR requirements for NS_50 for band n39, using the Rel-17 RF assumptions for mobile devices. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1: n34

Sub-topic description: handle the band specific requirements

Open issues and candidate options before e-meeting:

Issue 2-1-1: A-MPR

- Proposal: No additional A-MPR work for PC1.5 n34, PC1.5 can be added directly to those bands.

- Recommended WF

  - TBA

Issue 2-1-2: UE maximum output power with tolerance

- Options:

  - Option 1: 29dBm with +2/-3dB.

  - Option 2: Specify other option if any.

- Recommended WF

  - TBA

Comments collection

| Company | Comments |
| --- | --- |
| Huawei (JW) | Issue 2-1-1: Fine with the proposal. Issue 2-1-2: Fine with option 1. |
| Skyworks | Issue 2-1-1: A-MPR: no A-MPR for n34 Issue 2-1-2: UE maximum output power with tolerance. 29dBm with +2/-3dB option 1 |
| CMCC | Issue 2-1-1: OK for the proposal. Issue 2-1-2: option 1 is OK. |
| Apple | Issue 2-1-1: A-MPR We are okay with the proposal. Issue 2-1-2: UE maximum output power with tolerance Option 1 |
| vivo | Issue 2-1-1: OK with the proposal. Issue 2-1-2: Option 1. |

Sub-topic 2-2: n39

Sub-topic description: handle the band specific requirements

Open issues and candidate options before e-meeting:

Issue 2-2-1: A-MPR

- Proposals

  - Proposal 1: PC1.5 n39 NS_50 A-MPR is needed for smartphone UE, 10dB antenna isolation is used like for the related PC1.5 MPR.

  - Proposal 2: For PC1.5, the worst case PC2 regions A1 and A2 need 1.5dB higher back-off to account for 3dB higher emissions per antenna and the additional contribution from RIMD.

  - Proposal 3: A-MPR for 5MHz should be investigated.

  - Proposal 4: New or different A-MPR regions may need to be defined.

  - Proposal 5: Evaluate the PC1.5 A-MPR requirements for NS_50 for band n39, using the Rel-17 RF assumptions for mobile devices.

- Recommended WF

  - TBA

Issue 2-2-2: UE maximum output power with tolerance

- Options:

  - Option 1: 29dBm with +2/-3dB.

  - Option 2: Specify other option if any.

- Recommended WF

  - TBA

Comments collection

| Company | Comments |
| --- | --- |
| Huawei (JW) | Issue 2-2-1: proposal 5 is preferred. Regarding proposal 1, we’re fine with the assumption of 10 dB antenna isolation, which is the same as proposal 5. However, P1 seems to apply new A-MPR for smartphone UEs only and exclude that for FWA devices. We’re not convinced at this stage. P2 is based on theoretical analysis without verification by measurements/simulations, which is not agreeable at this stage. P3 and P4 are fine. Issue 2-2-2: Fine with option 1. |
| Skyworks | Issue 2-2-1: A-MPR: A-MPR is needed for n39, we will provide input at next meeting. Specific attention is needed for edge allocations. 5MHz and exact A-MPR regions need to be re-evaluated. Issue 2-2-2: UE maximum output power with tolerance. 29dBm with +2/-3dB option 1 |
| CMCC | Issue 2-2-1: Proposal 1 is OK for smart phone. The WID doesn’t exclude FWA form factor, so maybe we also need the A-MPR for FWA. Proposal 2 and 3 are based on analysis and seems reasonable. But it’s better to derive A-MPR based on simulation or measurement. Proposal 4 and 5 are OK for us. Issue 2-2-2: option 1 is OK for us. |
| Apple | Issue 2-2-1: A-MPR A-MPR should be investigated. Whether existing regions could be reused or not is up to the evaluation. Agree with proposals to investigate A-MPR for mobile devices using typical RF assumptions and re-use the 10dB antenna isolation assumption. Issue 2-2-2: UE maximum output power with tolerance Option 1 |
| vivo | Issue 2-2-2: Option 1. |

Sub-topic 2-3: n40

Sub-topic description: handle the band specific requirements

Open issues and candidate options before e-meeting:

Issue 2-3-1: A-MPR

- Proposal: No additional A-MPR work for PC1.5 n40, PC1.5 can be added directly to those bands.

- Recommended WF

  - TBA

Issue 2-3-2: UE maximum output power with tolerance

- Options:

  - Option 1: 29dBm with +2/-3dB.

  - Option 2: Specify other option if any.

- Recommended WF

  - TBA

Comments collection

| Company | Comments |
| --- | --- |
| Huawei (JW) | Issue 2-3-1: Fine with the proposal. Issue 2-3-2: Fine with option 1. |
| Skyworks | Issue 2-3-1: A-MPR: no A-MPR is needed for n40 Issue 2-3-2: UE maximum output power with tolerance. 29dBm with +2/-3dB option 1 |
| CMCC | Issue 2-1-1: OK for the proposal. Issue 2-1-2: option 1 is OK. |
| Apple | Issue 2-3-1: A-MPR We are okay with the proposal. Issue 2-3-2: UE maximum output power with tolerance Option 1 |
| ZTE | Issue 2-3-2: UE maximum output power with tolerance Although we agree with 29dBm with +2/-3dB in option 1, we see the proposals from company’s contribution to say ‘+2/-33’, we don’t know why the note 3 is applied since the note 3 is not applied to PC2/3 band n40? |
| vivo | Issue 2-3-1: OK with the proposal Issue 2-3-2: Option 1. |

Sub-topic 2-4: SAR

Sub-topic description: revisit SAR mechanism

Open issues and candidate options before e-meeting:

Issue 2-4-1: SAR

- Proposal: Simplify the specification and align the current RAN2 spec

  - Option1：Restrict maxUplinkDutyCycle-PC2-FR1 to PC2 UE only, maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 for PC1.5 UE only, and not to differentiate UE type.

  - Option2：Restrict maxUplinkDutyCycle-PC2-FR1 to PC2 UE only, and add maxUplinkDutyCycle-PC1dot5 for PC1.5 handheld UE.

- Recommended WF

  - TBA

Comments collection

| Company | Comments |
| --- | --- |
| Huawei (JW) | Issue 2-4-1: We’re concerned about the potential NBC issue, since band n41 supports PC1.5 since R16. A third option would be to change the description of maxUplinkDutyCycle-PC2-FR1 in RAN2 spec and align it with RAN4 usage. We’re open to further discuss the issue. |
| Skyworks | In our understanding the same Max UL duty cycle is signaled for PC2 and PC1.5 but for PC1.5, the actual value is half the declared value. Ie default declared is 50% correspond to 25% PC1.5 duty cycle. |
| CMCC | To be honest in RAN4 spec, there is no explicit explanation of whether maxUplinkDutyCycle-PC2-FR1 is supported by both PC2, PC1.5 UE or only by PC2 UE. RAN2 spec define it only for PC2 UE. so from my understanding, the reader will refer to 38.306 and this maxUplikDutyCycle is only for PC2. That’s my understanding. So the proposal is OK for us and option 2 is preferred. |
| ZTE | We have similar understanding with CMCC that RAN2 and RAN4 spec are not strictly aligned, we are open to discuss whether correct RAN2 spec or RAN4 spec. As mentioned by huawei, NBC issue should be taken into account. |
| CHTTL | In our understanding, the maxUplinkDutyCycle-PC2-FR1 applies to PC2 only in RAN2 and RAN4 spec, the default value 50% for PC2 and 25% for PC1.5 is set when no related capability is reported, we also think expanding the usage in the later release will cause NBC issue, and whether to maxUplinkDutyCycle-PC1dot5 needs further discuss in our view. |
| vivo | Both options are OK, slightly prefer option 1. |

Sub-topic 2-5: TR skeleton

Sub-topic description: TR skeleton

Open issues and candidate options before e-meeting:

Issue 2-5-1: TR skeleton

- Proposal:

  - Option1：To adopt the TR skeleton in R4-2215853 for HPUE_NR_FR1_TDD_R18

  - Option2：TBA

- Recommended WF

  - TBA

Comments collection

| Company | Comments |
| --- | --- |
| Huawei (JW) | Issue 2-5-1: Fine with option 1. |
| ZTE | Question for clarification, does this TP aim to include both single band (Topic#2) and intra-band ULCA (Topic#1) or only single band? |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2215509: Draft CR for updating high power class for FR1 TDD single bands | Huawei (JW): Band n39 has a pending A-MPR issue. Maybe worth waiting for its resolution. |
|  | Skyworks: CR is incomplete without A-MPR for n39 |
|  | ZTE: We are curious about why not TP first, then draft CR, considering there are A-MPR issue for n39. Draft CR is used to include full picture to include all of the requirement. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 2-1: n34 | Issue 2-1-1: A-MPR Summary of round 1 discussion: 5 companies are OK with the proposal. Tentative agreement: No additional A-MPR work for PC1.5 n34, PC1.5 can be added directly to those bands. Recommendations for 2nd round: confirm the tentative agreement and capture the agreement in WF. Issue 2-1-2: UE maximum output power with tolerance Summary of round 1 discussion: 5 companies are OK with option 1. Tentative agreement: UE maximum output power with tolerance is 29dBm with +2/-3dB Recommendations for 2nd round: confirm the tentative agreement and capture the agreement in WF. |
| Sub-topic 2-2: n39 | Issue 2-2-1: A-MPR Summary of round 1 discussion: proposals 3/4/5 are merged and captured in the tentative agreement according to the 1st round discussion. 3 companies agree with proposal 5, where 10dB antenna isolation in proposal 1 is covered. For the scope about smartphone or FWA, the rapporteur feedbacks FWA are not excluded in the WID. Tentative agreement: - A-MPR including 5MHz and region need to be re-evaluated. - Evaluate the PC1.5 A-MPR requirements for NS_50 for band n39, using the Rel-17 RF assumptions for mobile devices. - FWA are not excluded in the WID. Recommendations for 2nd round: confirm the tentative agreement and capture the agreement in WF. Issue 2-2-2: UE maximum output power with tolerance Summary of round 1 discussion: 5 companies are OK with option 1. Tentative agreement: UE maximum output power with tolerance is 29dBm with +2/-3dB Recommendations for 2nd round: confirm the tentative agreement and capture the agreement in WF. |
| Sub-topic 2-3: n40 | Issue 2-3-1: A-MPR Summary of round 1 discussion: 5 companies are OK with the proposal. Tentative agreement: No additional A-MPR work for PC1.5 n40, PC1.5 can be added directly to those bands. Recommendations for 2nd round: confirm the tentative agreement. Issue 2-3-2: UE maximum output power with tolerance Summary of round 1 discussion: 6 companies are OK with option 1. Tentative agreement: UE maximum output power with tolerance is 29dBm with +2/-3dB Recommendations for 2nd round: confirm the tentative agreement and capture the agreement in WF. |
| Sub-topic 2-4: SAR | Issue 2-4-1: SAR Summary of round 1 discussion: companies have different perspectives. According to the Huawei comment, the option 3 need to be added in the candidate options. Candidate options: - Option1：Restrict maxUplinkDutyCycle-PC2-FR1 to PC2 UE only, maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 for PC1.5 UE only, and not to differentiate UE type. - Option2：Restrict maxUplinkDutyCycle-PC2-FR1 to PC2 UE only, and add maxUplinkDutyCycle-PC1dot5 for PC1.5 handheld UE. - Option3：Send LS to RAN2 to change the description of maxUplinkDutyCycle-PC2-FR1 in RAN2 spec and align it with RAN4 usage. Recommendations for 2nd round: continue to discuss candidate options and capture the agreement in WF. |
| Sub-topic 2-5: TR skeleton | Issue 2-5-1: TR skeleton Summary of round 1 discussion: 1 company support the TR skeleton, and clarification to ZTE: only single band. Tentative agreement: To adopt the TR skeleton in R4-2215853 for HPUE_NR_FR1_TDD_R18 Recommendations for 2nd round: confirm the tentative agreement and capture the agreement in WF. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2215509: Draft CR for updating high power class for FR1 TDD single bands | to be revised |

Discussion on 2nd round

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Sub-topic 2-2: n39

Issue 2-2-1: A-MPR

Summary of round 1 discussion: proposals 3/4/5 are merged and captured in the tentative agreement according to the 1st round discussion. 3 companies agree with proposal 5, where 10dB antenna isolation in proposal 1 is covered. For the scope about smartphone or FWA, the rapporteur feedbacks FWA are not excluded in the WID.

Recommendations for 2nd round: confirm the tentative agreement and further list the RF assumptions for simulation or measurement.

Tentative agreement:

- A-MPR including 5MHz and region need to be re-evaluated.

- Evaluate the PC1.5 A-MPR requirements for NS_50 for band n39, using the Rel-17 RF assumptions for mobile devices.

- FWA are not excluded in the WID.

Comments collection

| Company | Comments |
| --- | --- |

Sub-topic 2-4: SAR

Issue 2-4-1: SAR

Summary of round 1 discussion: companies have different perspectives. According to the Huawei comment, the option 3 need to be added in the candidate options.

Recommendations for 2nd round: continue to discuss candidate options.

Candidate options:

- Option1：Restrict maxUplinkDutyCycle-PC2-FR1 to PC2 UE only, maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 for PC1.5 UE only, and not to differentiate UE type.

- Option2：Restrict maxUplinkDutyCycle-PC2-FR1 to PC2 UE only, and add maxUplinkDutyCycle-PC1dot5 for PC1.5 handheld UE.

- Option3：Send LS to RAN2 to change the description of maxUplinkDutyCycle-PC2-FR1 in RAN2 spec and align it with RAN4 usage.

Comments collection

| Company | Comments |
| --- | --- |

CRs/TPs comments collection

Considering the commemts on n39 in the 1st round discussion, moderator suggest removing n39.

| CR/TP number | Comments collection |
| --- | --- |
| Rev. of R4-2215509: Draft CR for updating high power class for FR1 TDD single bands |  |

Other

If you have any comments on the tentative agreements in 2.3 Summary for 1st round, except the issues mentioned in 2.4.1, 2.4.2 and 2.4.3, pleases write down below.

Comments collection

| Company | Comments |
| --- | --- |

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on HPUE_Basket_Intra-CA_TDD | Huawei, HiSilicon |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2215509 |  | Draft CR for updating high power class for FR1 TDD single bands | CMCC | Revised |  |
| R4-2215853 |  | High power for FR1 TDD single bands with power class 1.5 | CMCC | Approved |  |
| R4- 2216083 |  | Draft CR on TS38.101-1 Addition of intra-band CA Combinations with PC2 and PC1.5 | Huawei, HiSilicon | Revised |  |
| R4-2216080 |  | Discussion on UE RF requirements with PC1.5 | Huawei, HiSilicon | Noted |  |
| R4-2216081 |  | Discussion on UE RF requirements with PC2 | Huawei, HiSilicon | Noted |  |
| R4-2216082 |  | WID on HPUE_NR_FR1_TDD_intra_CA_R18 | Huawei, HiSilicon | Noted |  |
| R4-2215330 |  | A-MPR for PC1.5 TDD bands | Skyworks Solutions, Inc. | Noted |  |
| R4-2216123 |  | Discussion on the PC1.5 UE RF requirements | vivo | Noted |  |
| R4- 2216775 |  | Initial consideration on PC1.5 TDD bands | Huawei, HiSilicon | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents