3GPP TSG-RAN WG4 Meeting #104-bis-e                                                    	R4-2217077

Online, 10th – 19th October 2022

Source:	Vodafone

Title:	TP for TR 37.718-21-11: DC_20-32_n7

Agenda item:			5.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.718-21-11 to include DC_20-32_n7.

The proposed TIB,c, RIB and MSD values were derived from the requirements already present for CA_7-20-32 in 36.101, and DC_20_n7 in 38.101-3.

# 2. Reference

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	DC_20-32_n7

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_20A-32A_n7A | DC_20A_n7A |

### 5.x.2	Co-existence studies

Table 5.x.2-1 lists the Band 20A + Band n7A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2-1: Band 20 and Band n7 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 2500 | 2570 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 5000 – 5140 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2586 |  |  | 7500 – 7710 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1638 – 1738 |  |  | 3332 – 3432 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 776 – 906 |  |  | 4138 – 4308 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4164 – 4294 |  |  | 5832 – 6002 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 782 – 912 |  |  | 2480 – 2590 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 – 86 |  |  | 6638 – 6878 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 3276 – 3476 |  |  | 6664 – 6864 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4996 – 5156 |  |  | 8332 – 8572 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9138 – 9448 |  |  | 758 – 948 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5776 – 6046 |  |  | 2414 – 2644 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10832 – 11142 |  |  | 5828 – 6018 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9164 – 9434 |  |  | 7496 – 7726 |  |  |

Based on Table 5.x.2-1,

-	3rd order harmonics may fall into Rx frequencies of bands 38, 41, 69 and 90.

-	2nd order IMD may fall into Rx frequencies of bands 42, 52, 77 and 78.

-	3rd order IMD may fall into Rx frequencies of bands 5, 6, 18, 19, 20, 26, 27, 28, 44, 46, 47, 68 and 77.

-	4th order IMD may fall into Rx frequencies of bands 42, 46, 52, 77, 78 and 79.

-	5th order IMD may fall into Rx frequencies of bands 5, 6, 7, 8, 14, 18, 19, 20, 26, 27, 28, 38, 41, 44, 46, 47, 53, 68, 69 and 90.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.x.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.x.2-2: 2UL Band 20 + Band n7 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4, IMD5 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | IMD4, IMD5 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_20_n7.

### 5.x.3	∆TIB and ∆RIB values

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-32_n7 | 20 | 0.3 |
|  | n7 | 0.7 |

### 5.x.4	Reference sensitivity exceptions

No addition exceptions required compared to fallbacks.

<End of Text Proposal>