3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2217091

# Electronic Meeting, October 10-19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0502 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-2 to clarify P-MPR behavior when DMRS bundling is configured |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc., MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Core |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During the RAN4 #103 meeting, RAN4 identified an issue related to UE UL Tx power adaptation, which is documented in an LS to RAN1 as follows [R4-2211225]: RAN4 acknowledged that the modification of Tx power may mean that phase continuity and power consistency is not able to be maintained by the UE, as seen in an earlier LS to RAN1 in R4-2103393. In RAN4 #103e meeting, RAN4 has also discussed whether text relating to UL Tx transmission power during transmission of a DMRS bundle in TS 38.101 needs further clarification. It has been raised during RAN4 discussions that in TS 38.214 v17.1.0 section 6.1.7 it states that “The UE shall maintain power consistency and phase continuity within an actual TDW…”. RAN4 would therefore appreciate feedback from RAN1 on whether the text in TS 38.214 section 6.1.7 prevents the UE from modifying its Tx power when necessary, i.e. prevents the UE from following TS 38.213 clause 7.1.1 and 7.2.1, or prevents the UE from adapting P-MPR, during transmission of a DMRS bundle. We ask this because further alignment of understanding between RAN1 and RAN4 may be needed. RAN1 has sent a response from the RAN1 #110 meeting [R1-2208212] as follows: For the question about UE UL Tx power adaption asked in RAN4 LS, RAN1 had the discussion and made the following conclusion: RAN1 spec requires power consistency and phase continuity to be maintained during an actual TDW, as defined in Clause 6.1.7 in 38.214. RAN1 has not taken into account any power change due to P-MPR for satisfying the regulatory requirements. It is up to RAN4 to define any needed requirements for power consistency and phase continuity while also taking P-MPR into account. Considering that defining P-MPR behavior for satisfying the regulatory requirements is up to RAN4, a clarification of the related behavior is necessary. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify UE behavior with the following additional text to Clause 6.4.2.6: - Add the qualifier “when the UE is configured with DMRS bundling” to the section. - Add the sentence “Phase continuity requirements do not apply when the UE applies a change of P-MPRc value during a DMRS bundled transmission.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If this CR is not approved, then it is ambiguous as to whether P-MPR is allowed to be applied, which could conflict with the need for the UE apply P-MPR when necessary to adhere to relevant emission regulation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< start of change >>

#### 6.4.2.6	Phase continuity requirements for DMRS bundling

For bands that UE indicates the support of DMRS bundling, when the UE is configured with DMRS bundling, the maximum allowable difference between the measured phase value in any slot p-1 and slot p shall satisfy the requirements as listed in Table 6.4.2.6-1 for the measurement conditions defined in Table 6.4.2.6-2, within a measurement time window limited by the UE capability of maximum duration for DMRS bundling [maxDurationDMRS-Bundling-r17], and defined for each frequency band separately. The phase value for each slot is measured as shown in Annex F.8. These requirements apply to PUCCH and PUSCH transmissions with DFT-s-OFDM and CP-OFDM waveforms.  Phase continuity requirements do not apply when the UE applies a change of P-MPRc value during a DMRS bundled transmission.

Table 6.4.2.6-1: Maximum allowable phase difference for DMRS bundling

| UL channel | Modulation order | Phase difference between any slot p-1 and slot p (NOTE 2) |
| --- | --- | --- |
| PUSCH | Pi/2 BPSK, QPSK | [25] degrees |
| PUCCH | Pi/2 BPSK, BPSK, QPSK |  |
| NOTE 1: The UE capability of the length of maximum duration refers to the maximum time duration during which UE is able to meet the phase continuity requirements, assuming no phase consistency violating events defined in TS 38.214 in between. NOTE 2: This requirement applies for TDD bands, for supported DMRS bundling configurations ≤ 8 slots. |  |  |

The above requirements are applicable when all the following conditions are met within the measurement time window.

-	RB allocation in terms of length and frequency position does not change, and intra-slot and inter-slot frequency hopping is not activated.

-	Modulation order does not change.

-	No network commanded TA takes effect.

-	The TPMI precoder does not change.

-	There is no change in UE EIRP level, and no change in the level of P-MPR applied by the UE.

-	UE is not scheduled with uplink transmission of other physical channel/signal in-between the PUSCH or PUCCH transmissions.

-	For TDD, no downlink slot(s) or downlink symbol(s) or flexible symbol(s) with/without DL monitoring occasion configured in-between the PUSCH or PUCCH transmissions.

-	No uplink beam switching occurs.

Table 6.4.2.6-2: Measurement conditions for the maximum allowable phase difference

| Parameter | Unit | Level |
| --- | --- | --- |
| UE EIRP | dBm | PUMAX,f,c in clause 6.2.4, P-MPR = 0 |
| UE downlink received power |  | Not change |
| Operating conditions |  | Normal conditions |
| Transmission bandwidth |  | Confined within FUL_low + [4] MHz and FUL_high – [4] MHz |
| DL signal frequency |  | Not change before and during the measurement window |
| DL signal timing |  | Maintained constant before and during the measurement window |
| UL slots for testing |  | Tested on consecutive UL slots |
| PUSCH waveform for testing |  | DFT-s-OFDM |

NOTE: Phase continuity requirements for DMRS bundling is defined only within FR2-1 in this release of the specification.

<< end of change >>